3GPP TSG-SA5 Meeting #146 	S5-227025

Toulouse, France, 14 -18 November 2022

# Source:	Huawei, Deutsche Telekom

# Title:	pCR 28.824 Concepts of filtering and combination in exposure governance

# Document for:	Approval

Agenda Item:	6.9.6.3

# 1	Decision/action requested

Approval

# 2	References

[1]	3GPP TR 28.824 V0.9.0 Study on network slice management capability exposure

[2]	3GPP TS 28.533 Management and orchestration; Architecture framework

# 3	Rationale

As described in TS 28.533 [2] clause 4.4, the exposure governance of management services achieves the exposure control for the basic components (e.g. management services component type A, B, C) of one management service.

The figure 4.4.1 below comes from TS 28.533 clause 4.4. As described in Figure 4.4.1 left hand part, when there is a Management Service A exposure without exposure governance, Management Service A Consumer (e.g. 3rd party) can access all management capability offered by Management Service A Producer. As described in Figure 4.4.1 right hand part, when Management Service A is exposed with applied exposure governance it becomes Management Service A'. Exposure governance plays an important role during the process of management capability exposure. For instance, if the capability offered by management service A’ is a subset of that offered by management service A, without the exposure governance, the management service A’ consumer may have the access to consume all capability provided by management service A which will cause the illegal/improper management capabilities consumption.

Figure 4.4.1: Management capability exposure governance applied on exposed Management Service A

According to TS 28.533 [2] clause A.2, Exposure governance management function (EGMF) shown in Figure A.2.1 is management function in network function model with the role of management service exposure governance (i.e. abstraction, simplification, filtering, etc.) as below.

Figure A.2.1: Example NF management function layer structure

However, details of EGMF management capability exposure governance is FFS in TS 28.533 [2], i.e., what exactly does the exposure governance mean hasn’t been specified in [2]. Based on meeting discussion in SA5#145e, this contribution is proposed to give definitions of filtering and combination.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	TM Forum TMF622 Product Order API REST Specification

[3]		TM Forum TMF641 Service Ordering API

[4]	TM Forum TMF652 Resource Order Management API

[5]		3GPP TS 28.531: "Management and orchestration; Concepts, use cases and requirements"

[6]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2"

[7]	3GPP TR23.700-99 “Study on Network Slice Capability Exposure for Application Layer Enablement (NSCALE)”

[8]	3GPP TS23.434 “Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows.”

[9]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[10]	3GPP TS 28.537: "Management and orchestration; Management capabilities"

[11]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[12]	TM Forum TMF633 Service Catalogue Management API

[13]	TM Forum TMF620 Product Catalogue Management API

[14]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2"

[15]	3GPP TS 28.532: "Management and orchestration; Generic Management Service"

[16]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions"

[17]	3GPP TS 28.622: " Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP) Information Service (IS)"

[18]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2"

[19]	3GPP TS 29.222: “Common API Framework for 3GPP Northbound APIs”.

[20]	3GPP TS 31.222: “Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs”.

[21]	CAMARA: https://github.com/camaraproject

[X]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)"

Next change

#### 4.1.1.x	Exposure Governance

To avoid unauthorised management capability consumption, management services should be applied with exposure governance before being exposed to different types of MnS consumers. As introduced in TS 28.533 [11], management capability exposure governance provides exposure governance (i.e. filtering) on basic elements of management function service based interface:

1)	Management service component type A

2)	Management service component type B

3)	Management service component type C

Definitions and typical cases of filtering are given in the following subclauses.

##### 4.1.1.x.1	Filtering

Definition: The exposure access control on MnS component type A, type B and/or type C.

Typical case 1: Assuming the management service A is provisioning MnS with operations createMOI, getMOIAttributes, modifyMOIAttributes and deleteMOI as MnS component type A, the filtering on management service A could be that only a subset of original MnS is exposed to MnS consumer, e.g. only operation getMOIAttributes, is exposed to management service consumer.

Typical case 2: Assuming the management service A is provisioning MnS with NRM MOIs as the component type B, the filtering on management service A could be that only a subset of original MnS is exposed to MnS consumer, e.g. only part of the attributes of the NRM MOI, is exposed to management service consumer.

To implement the filtering on MnS, attribute mnsScope of IOC MnsInfo, see further details in TS 28.622 [17], may be used to provide information about the management scope of a Management Service. The management scope is defined as the set of managed object instances that can be accessed (as filtering) using the Management Service.

End of changes