3GPP TSG-SA5 Meeting #146Bis-e 	S5-231074

Electronic meeting, 16 - 19 January 2023

- Source:	Huawei Technologies

- Title:	Rel-18 pCR 28.828 Evaluation and Conclusion for SNPN Topic 3

- Document for:	Approval

Agenda Item:	7.5.4

# 1	Discussion

This is a pCR to TR 28.828 about evaluation and conclusion for SNPN Topic 3.

# 2	References

[1]	3GPP TR 28.828 " Study on charging aspects for enhanced support of non-public networks".

# 3	Rationale

This pCR proposes to introduce the evaluation and conclusion for Topic 3 in TR 28.828 "Study on charging aspects for enhanced support of non-public networks".

# 4	Detailed proposal

Propose to incorporate the following change into the TR 28.828 [1].

First changes

### 5.3.5	Evaluation

The solution #1 solves the key issue 3a.

The solution #1

-	solves the data connectivity in PLMN Charging per UE (access the PLMN via SNPN) for End user charging, with the RAT type specifics to SNPN with the SNPN ID if available: (PLMN ID and NID identifying SNPN).

-	For end user charging, the SNPN ID is not necessary. Without the SNPN ID, there is no impaction on the solution #1.

Both solution #2 and solution #3 solves the key issue 3b.

The solution #2

-	solves the data connectivity in PLMN Charging per SNPN (access the PLMN via SNPN) for inter-provider charging, which aggregates usage for UEs belonging to the same SNPN.

-	For inter-provider charging, the SNPN ID is required which requires the underlay network PLMN can provide the SNPN identifier, which depend on the 5GS core system. Without the SNPN ID, solution #2 does not work, may impact on the conclusion.

The solution #3

-	solves the data connectivity in PLMN Charging per SNPN (access the SNPN via PLMN) for inter-provider charging, with the volume usage per SNPN based on the accumulation of usage reporting per UE, with the same N3IWF address to indicate the SNPN.

-	requires the underlay network PLMN can provide the N3IWF address of the SNPN, which depend on the 5GS core system, may impact on the conclusion.

### 5.3.6	Conclusion

Solution #1, solution #2 and solution #3 are selected to the normative work.

Editor’s Note: whether the solution is dependent on 5GC support which may impact the conclusion is ffs.

End of changes