3GPP TSG-SA5 Meeting #146Bis-e 	S5-231136

Electronic meeting, 16 - 19 January 2023

# Source:	Ericsson

# Title:	Adding comparison between double or single CHF

# Document for:	Endorsement

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

There are basically three different potential architectures, where one CTF (AMF, SMF and SMSF) have for one chargeable event/session:

A.	two charging events/sessions, one to each of V-CHF and H-CHF.

B.	two charging events/sessions to V-CHF, the V-CHF forwards one of these to H-CHF

C.	one charging events/sessions to V-CHF, the V-CHF creates one new charging event/session to H-CHF

There is a fourth variant (D) which is a mix between B and C, where the CTF would only have one chargeable/event session towards the V-CHF but would sperate the information sent to V-CHF and the information to be forwarded to the H-CHF.

For potential architecture A the service used would be the Nchf_ConvergedCharging for both charging events/sessions, while for potential architectures B and C this doesn’t have to be the case. The comparison assumes that the Nchf_ConvergedCharging is used for all potential architectures i.e., between CTF and CHFs as well as between CHFs.

All potential architectures add latency, how much will to a large extent depend on the interaction with the HPLMN. Using the non-blocking mode could mitigate it to some extent but have other cons.

# 4	Detailed proposal

First change

### 7.2.5	Evaluation

Solutions #2.2 and #2.1 both solves key issues #2a and #2b and covers requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02. Solution #2.1 solves more key issues and covers more requirements, and #2.2 is already supported in the scope of 5G connectivity charging.

Solutions #2.4, #2.1, #2.16 all solves key issues #2f and #2g and covers requirements REQ-CH_CVTOH-03, and REQ-CH_CVTOH-04. Solution #2.1 solves more key issues and covers more requirements, and #2.16 is already supported in the scope of SMS charging as is.

Solutions #2.5 and #2.1 both solves key issues #2h and #2i and covers requirements REQ-CH_CVTOH-05, and REQ-CH_CVTOH-06. Solution #2.1 solves more key issues and covers more requirements and #2.5 is already supported in the scope of 5G connection and mobility charging as is.

Solutions #2.8 and 2.9 both solves key issues #2c, both require solution #2.1 or #2.3, and can be supported at the same time depending on the need and capability of the V-CHF.

Solutions #2.6 and #2.7 both solves key issues #2d.

Solutions #2.10, #2.11 and #2.12 all solves key issues #2e.

Editor’s note:	Further evaluations are FFS.

##### 7.2.5.x	Comparison of potential architectures

Comparison of the four different potential architectures, where one CTF (AMF, SMF and SMSF) have for one chargeable event/session:

A.	two charging events/sessions, one to each of V-CHF and H-CHF, solutions #2.2, #2.4, and #2.5.

B.	two charging events/sessions to V-CHF, the V-CHF forwards one of these to H-CHF, solution #2.x.

C.	one charging events/sessions to V-CHF with single charging information, the V-CHF creates one new charging event/session to H-CHF, solution #2.1

D.	one chargeable/event session towards the V-CHF with dual charging information, the V-CHF creates one new charging event/session to H-CHF, solution #2.13 and #2.14

Table 4-1: Comparison potential architectures

| Proposal | Pros | Cons |
| --- | --- | --- |
| A | - Already specified for some NFs - From H-CHF view like home routed roaming - From V-CHF view like home routed roaming | - Additional CTF handling - NF and vendor specific failure handling - No support of more than two CHFs, if required - Synchronization of rating groups |
| B | - Could reuse what is specified for A - From H-CHF view like home routed roaming - Allows the VPLMN to monitor what the HPLMN is receiving and requesting | - Additional CTF handling - NF and vendor specific failure handling - Synchronization of rating groups - Using V-CHF as a proxy/routing agent |
| C | - From H-CHF view like home routed roaming - From CTF view like home routed roaming (simplification) - V-CHF could be used to translate rating groups - Allows the VPLMN to monitor what the HPLMN is receiving and requesting - Allows the VPLMN to influence what the HPLMN is receiving and requesting - Supports more than two CHFs | - Additional speciation - Additional handling in V-CHF i.e., the CHF needs to be able to act as a Nchf consumer |
| D | See B, except for the possibility to reuse what is specified for A |  |