| 3GPP TR 28.841 V0.4.0 (2023-3) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Management Aspects of IoT NTN Enhancements; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Concepts and background	10

4.1	Reference management architecture for integrated satellite components	10

4.1.1	Management architecture #1 for integrated satellite NBIoT/LTE-M RAT	10

4.1.2	Management architecture #2 for integrated satellite NBIoT/LTE-M  -RAT	10

4.1.3	Management architecture #3 for integrated non-3GPP satellite RAN	11

4.2	Architecture Scenarios for integrated satellite components	12

4.2.1	scenario #1: A 3GPP network with access networks and terrestrial access networks	12

4.2.2	scenario #2: Satellite enabled 3GPP network as a roaming network for terrestrial network operators	13

5	Use cases and key issues	13

5.1	Use cases	13

5.1.1	Use case #1: Management of satellite components	13

5.1.1.1              Use case for NGSO regenerative satellite components	13

5.1.1.1.1 	Pre-conditions	13

5.1.1.1.2 	Description	13

5.1.1.1.3 	Post-description	14

5.1.1.2              Use case for NGSO transparent satellite components	14

5.1.1.2.1 	Pre-conditions	14

5.1.1.2.2 	Description	14

5.1.1.2.3 	Post-description	14

5.1.1.3              Use case for NTN Mobility Management	14

5.1.1.3.1 	Pre-conditions	14

5.1.1.3.2 	Description	14

5.1.1.3.3 	Post-description	14

5.1.2	Use case #2: Monitoring of satellite components	15

5.1.2.1              Use case for monitoring of average delay on DL-Air Interface for MEO and GEO satellite components	15

5.1.2.1.1 	Goal	15

5.1.2.1.2 	Pre-conditions	15

5.1.2.1.3 	Description	15

5.1.2.1.4 	Post-description	15

5.2	Potential requirements	15

5.2.1	Use case #1 Management of satellite components	15

5.2.1.1              Management of NGSO regenerative satellite components	15

5.2.1.2              Management of NGSO transparent satellite components	15

5.2.1.3              Mobility Management enhancement	15

5.2.2	Use case #2 Monitoring of satellite components	16

5.2.2.1              Monitoring of average delay on DL-Air Interface with MEO or GEO satellite components	16

5.3	Potential solutions	16

5.3.1	Use case #1 potential solutions for the management of NGSO regenerative and transparent satellite components	16

5.3.1.1             Solution #1: Prevent PCI conflicts, PCI confusion and continuous NCR-reconfigurations by using SON functions	16

5.3.1.2              Solution #2: Potential solution for NTN Mobility Management enhancement with discontinuous satellite coverage	17

5.3.2	Use case #2 potential solutions for the monitoring of satellite components	18

5.3.2.1              Solution to monitor delay on the DL-Air Interface with MEO or GEO satellite components	18

6	Conclusions and recommendations	19

Annex A	20

A.1	General	20

A.2	Class of orbit	20

Annex B	21

B.1	CIoT EPS with transparent satellite enabled RAN	21

B.2	CIoT EPS with regenerative satellite enabled RAN and on-board eNB	21

Annex <X> (informative): Change history	22

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

In Release 17, a work item of RAN is carried out to enable NB-IoT and eMTC to support Non-Terrestrial Networks (NTN). The objective of RAN is to specify enhanced IoT radio interfaces and NG-RAN/ E-UTRAN. With the gradual deepening of related RAN work, the present document studies the key issues associated with service and network management of IoT NTN enhancements (whether as NG-RAN or E-UTRAN) and potential solutions.

1	Scope

The present document  studies on management aspects of IoT NTN enhancements. It investigates specific IoT NTN related parameters which should be considered by O&M, investigates NRM enhancement, performance measurement and related new KPIs of IoT NTN. It studies the key issues associated with service and network management of IoT NTN enhancements (whether as NG-RAN or E-UTRAN) and potential solutions, and provides recommendations for the further normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 36.300: " Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3] 	3GPP TS 38.300: " NR; NR and NG-RAN Overall description; Stage-2 ".

[4] 	3GPP TR 28.808: "Study on management and orchestration aspects of integrated satellite components in a 5G network".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[7]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode"

[8]	3GPP TR 38.821: " Solutions for NR to support non-terrestrial networks (NTN) ".

[9]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[10]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].AN	Access Network

BL                       Bandwidth reduced Low complexity

5GC	5G Core Network

CN	Core Network

eMTC                  enhanced Machine-Type Communication

GEO 	Geostationary satellites

gNB	generic Node B

gNB-CU	gNB-Central Unit

gNB-DU	gNB-Distributed Unit

HARQ                 Hybrid Automatic Repeat Request

HEO	Highly-Eccentric Orbiting satellites

LEO 	Low-Earth Orbiting satellites

IoT                       Internet of Things

MEO 	Medium-Earth Orbiting satellites

ms	milli-second, a thousandth of a second.

NGSO                  Non-geostationary Orbiting satellites

NOP	Network Operator

NS	Network Slice

NSI	Network Slice Instance

NSSI	Network Slice Subnet Instance

SON                     Self-organized network

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RU	Radio Unit

4	Concepts and background

4.1	Reference management architecture for integrated satellite components

4.1.1	Management architecture #1 for integrated satellite NBIoT/LTE-M RAT

The reference architecture (according to TS 36.300[1] and TS 38.300[2]) depicted in figure 4.1.1 below considers the case of a 3GPP RAN integrating a satellite NBIoT/LTE-M RAT, possibly together with a Terrestrial RAT. The NOP operates the 5G network interfacing through API's with Communication Service Customers or Verticals on the one hand and delivering services to UEs on the other hand. The 3GPP Management system manages the 3GPP RAN aspect.

Figure 4.1.1: Reference architecture #1 for the management of a satellite NBIoT/LTE-M-RAT

4.1.2	Management architecture #2 for integrated satellite NBIoT/LTE-M  -RAT

The reference architecture (according to TS 36.300[1] and TS 38.300[2]) depicted in figure 4.1.2 considers another case of a 3GPP RAN integrating a satellite NBIoT/LTE-M -RAT, possibly together with a Terrestrial RAT and a satellite NR-RAT. The NOP operates the 5G network interfacing through API's with Communication Service Customers or Verticals on the one hand and delivering services to UEs on the other hand. The 3GPP Management system manages the 3GPP RAN aspect.

Figure 4.1.2: Reference architecture #2 for the management of a satellite NBIoT/LTE-M -RAT

4.1.3	Management architecture #3 for integrated non-3GPP satellite RAN

The reference architecture depicted in figure 4.1.3 considers the case of a non-3GPP satellite RAN integrated in a EPS/5GS. The NOP operates the 5G network interfacing through API's with Communication Service Customers or Verticals on the one hand and delivering services to UEs on the other hand. The 3GPP management system manages the 3GPP RAN and the non-3GPP RAN.

Figure 4.1.3: Reference architecture for the management of a non-3GPP satellite RAN

4.2	Architecture Scenarios for integrated satellite components

4.2.1	scenario #1: A 3GPP network with access networks and terrestrial access networks

In this scenario, a 3GPP network is composed out of a 3GPP core network, a satellite IoT 3GPP access network, a satellite NR 3GPP access network and a terrestrial 3GPP access network, where the satellite component is also integrated as a 3GPP access network. The satellite IoT network, satellite NR network and the terrestrial network share the same PLMN and access networks are managed by the same 3GPP management system. This architecture scenario follows the Satellite access class scenario in clause 4.2.2 from 3GPP TR 28.808[3] .

Figure 5.1.1: Architecture of 3GPP network with a satellite IoT 3GPP access network, a satellite NR 3GPP access network and a satellite NR 3GPP access network

Note: 	Both access networks are in the same management domains.

4.2.2	scenario #2: Satellite enabled 3GPP network as a roaming network for terrestrial network operators

In this scenario, consider two separate 3GPP networks, one 3GPP network with a satellite IoT access network and satellite NR access network, another 3GPP network with a terrestrial access network. Both networks have their own PLMN ID and have a roaming agreement in place with each other. Roaming is used by the terrestrial operator to use the 3GPP network with a satellite access. Both the satellite and terrestrial network have their own separate 3GPP management domain. This architecture scenario follows the Satellite access class scenario in clause 4.2.1 from 3GPP TR 28.808[3].

Figure 5.1.2: Architecture of satellite 3GPP network (green) 
as roaming network to a terrestrial 3GPP network (blue)

NOTE: 	The green and blue networks are separate management domains.

5	Use cases and key issues

5.1	Use cases

5.1.1	Use case #1: Management of satellite components

5.1.1.1              Use case for NGSO regenerative satellite components

5.1.1.1.1 	Pre-conditions

A NOP operates a non-terrestrial network for BL UEs, UEs in enhanced coverage and NB-IoT UEs.

The NOP manages the NB-IoT/eMTC Non-Terrestrial Networks in accordance with the 5G specifications. The functionality is largely aligned with that of Rel-17 NR Non-Terrestrial Networks.

5.1.1.1.2 	Description

The NOP integrates a satellite component based on NGSO regenerative satellites. NGSO regenerative satellites embark eNBs.

The network management is implemented in such a way that moving eNBs can be managed.

5.1.1.1.3 	Post-description

The NOP operates a non-terrestrial network for BL UEs, UEs in enhanced coverage and NB-IoT UEs that manages the satellite component.

5.1.1.2              Use case for NGSO transparent satellite components

5.1.1.2.1 	Pre-conditions

A NOP operates a non-terrestrial network for BL UEs, UEs in enhanced coverage and NB-IoT UEs.

The NOP manages the NB-IoT/eMTC Non-Terrestrial Networks in accordance with the 5G specifications and wants to integrate one or multiple NGSO transparent satellites into a NB-IoT/eMTC Non-Terrestrial Networks.

5.1.1.2.2 	Description

The NOP integrates a satellite component based on NGSO transparent satellites. A SaT/BL UE/UE in enhanced coverage/NB-IoT UE will connect to an EPC/5GC via a NGSO transparent satellite component. A eNB may use a special remote radio unit to connect via one or multiple transparent satellite(s) to the NB-IoT/eMTC Non-Terrestrial Networks, as illustrated in clause B.

The network management is implemented in such a way that moving transparent satellite components could be managed.

5.1.1.2.3 	Post-description

The NOP operates a non-terrestrial network for BL UEs, UEs in enhanced coverage and NB-IoT UEs that manages a 5GS with transparent satellite component(s).

5.1.1.3              Use case for NTN Mobility Management

5.1.1.3.1 	Pre-conditions

Support for IoT NTN discontinuous coverage was introduced in TS 23.401 [5]. The UE is assumed to know how the NTN coverage varies with time based on information defined in TS 36.331 [6] and TS 36.304 [7] (e.g. from the ephemeris data of a satellite access system that the UE is using).

A NOP operates a non-terrestrial network for BL UEs, UEs in enhanced coverage and NB-IoT/ eMTC UEs.

The NOP manages the NB-IoT/eMTC Non-Terrestrial Networks providing discontinuous coverage for NTN MEO/LEO satellite or satellite constellation, which is considered as the satellite access in 5GS/EPS, wants to support modeling of mobility management enhancement related to discontinuous coverage for the power saving enhancement.

5.1.1.3.2 	Description

The NOP integrates a satellite component based on NGSO transparent satellites. For a UE using a Non-Terrestrial Network that provides discontinuous coverage (e.g. for satellite access with discontinuous coverage), the UE may be out of network coverage at a certain time. The UE may then deactivate its Access Stratum functions in order to optimize power consumption until coverage returns.

3GPP management system can help to provide the coverage information and UE unreachability period to both the UE and AMF/MME.

The network management is implemented in such a way that mobility management enhancement and/or power saving could be supported.

5.1.1.3.3 	Post-description

The NOP operates a non-terrestrial network for BL UEs, UEs in enhanced coverage and NB-IoT UEs, modeling of mobility management enhancement related to discontinuous coverage and the power saving could be supported.

5.1.2	Use case #2: Monitoring of satellite components

5.1.2.1              Use case for monitoring of average delay on DL-Air Interface for MEO and GEO satellite components

5.1.2.1.1 	Goal

The goal is to allow the monitoring of the delay on the DL-Air Interface when a MEO or GEO satellite component for which the HARQ process has been modified with respect to a terrestrial RAN.

5.1.2.1.2 	Pre-conditions

A NOP operates a non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs.

The 5G network as operated by the NOP satellite RAN supporting NB-IoT/eMTC.

5.1.2.1.3 	Description

To cope with the effects of satellite delay and associated RTT, the HARQ operation is disabled or modified when compared to terrestrial RAN.

The management system of the NB-IoT/eMTC Non-Terrestrial Network is designed to provide a measured of the average delay on DL-Air Interface for MEO and GEO satellite components without an HARQ process

5.1.2.1.4 	Post-description

The management system monitors the average delay on the DL-Air Interface for MEO and GEO satellite components.

5.2	Potential requirements

5.2.1	Use case #1 Management of satellite components

5.2.1.1              Management of NGSO regenerative satellite components

[REQ-FS_IoT_NTN_mgmt_NGSO_reg] In a NB-IoT/eMTC Non-Terrestrial Network integrating an NGSO regenerative satellite RAT, the 5G network shall have the capability of managing moving eNBs.

5.2.1.2              Management of NGSO transparent satellite components

[REQ-FS_IoT_NTN_NGSO_tra] In a NB-IoT/eMTC Non-Terrestrial Network integrating an NGSO transparent satellite RAT, the 5G network shall have the capability to manage one or multiple moving regenerative satellite component(s).

5.2.1.3              Mobility Management enhancement

[REQ-FS_IoT_NTN_MM_enh] In an NB-IoT/eMTC Non-Terrestrial Network integrating an NGSO transparent satellite RAT, the 3GPP management system shall have the capability to support mobility management enhancement and/or power-saving for discontinuous coverage.

5.2.2	Use case #2 Monitoring of satellite components

5.2.2.1              Monitoring of average delay on DL-Air Interface with MEO or GEO satellite components

[REQ-FS_IoT_NTN-mon] In a NB-IoT/eMTC Non-Terrestrial Network integrating a MEO or GEO satellite components, it shall be possible to monitor the average delay on the DL-Air Interface.

5.3	Potential solutions

5.3.1	Use case #1 potential solutions for the management of NGSO regenerative and transparent satellite components

5.3.1.1             Solution #1: Prevent PCI conflicts, PCI confusion and continuous NCR-reconfigurations by using SON functions

Satellite RAN can be comprised out of either LEO, MEO, GEO satellites or a combination of these three. LEO and MEO satellites do not have a fixed position relative to the earth and travel at high speed in trajectories around the earth. The possible satellite beam coverage area may therefore change in a continuous fashion, as can also be seen in Figure 5.3.1.1. This is also described in more detail in clause A.2.

A LEO, MEO or GEO satellite can either be working in transparent mode or in regenerative mode. When a satellite is configured in transparent mode, no data processing will be done locally in the satellite. The incoming signal will be received, amplified, and transmitted back to earth. A regenerative satellite may actually demodulate/decode and process the incoming signal before sending data back to earth. A regenerative satellite may therefore embark a eNB/gNB/gNB-DU/gNB-CU and use inter satellite links to communicate with other satellite  eNB/gNBs. A transparent satellite may be coupled to a eNB/gNB which is located on earth, this eNB/gNB will use a satellite gateway to transmit to the satellite which reflects the signal back to earth.

In case of MEO and LEO satellites with moving beams the cell coverage may not be fixed to a specific location on earth, and since the possible coverage area of a satellite may even span multiple countries, it would be possible that the neighbouring (terrestrial) cells of the satellite eNB/gNB will change continuously. This may result in continuous NCR reconfigurations and PCI conflicts and/or PCI confusion since the NTN cell coverage may overlap with terrestrial RAN. This effect is also described in clause 8.5.3 from TR 38.821 [8].

Figure 5.3.1.1: Example of a MEO or LEO satellite flying over multiple geographical areas. Here it is assumed that the TAC will be fixed on earth. Picture taken from TR 38.821 [x]

To prevent these issues, it would be possible to use/extent the SON ANR and SON PCI re-configuration functions from TS 28.313 [9] and adapt these to support continuously moving cells. This way, the PCI re-configuration procedure may deconflict PCI's when an PCI conflict happens between an NTN cell and a terrestrial cell. The ANR could be used to automatically configure new neighbouring cells when the NTN cell coverage moves over the earth atmosphere. The SON ANR and SON PCI functions should be fast enough to keep up with the moving satellites which may have new neighbouring cells every 5 - 30 seconds.

In case of GEO satellites or MEO and LEO satellites with earth fixed beams, there is no need to adapt the SON ANR and SON PCI re-configuration functions since the coverage area of the satellite RAN will be fixed to a geographical area on earth's surface.

5.3.1.2              Solution #2: Potential solution for NTN Mobility Management enhancement with discontinuous satellite coverage

NTN MEO/LEO satellite or satellite constellation that provides discontinuous coverage is considered as the satellite access in 5GS and EPS. Satellite assistance information (e.g. ephemeris information), can be used for the handling of coverage holes or discontinuous satellite coverage in a power efficient way. For a UE, it should be possible to predict discontinuous coverage based on the satellite assistance information.

The 3GPP management system should provide the following IoT-NTN satellite ephemeris parameters to the eNB providing non-terrestrial access as outlined in TS 38.300 [3], which the NR NTN agreements can be used as the baseline.

This solution uses the instances of following IOCs or attributes representing the Ephemeris information, for example, named as ephemerisInformation, and it contains the following attributes:

-    Ephemeris information describing the orbital trajectory information or coordinates for the NTN vehicles. This information is provided on a regular basis or upon demand to the eNB/gNB;

-    Two different sets of ephemeris formats shall be supported:

-    Set 1: Satellite position and velocity state vectors:

-    Position;

-    Velocity.

-    Set 2: At least the following parameters in orbital parameter ephemeris format:

-    Semi-major axis;

-    Eccentricity;

-    Argument of periapsis;

-    Longitude of ascending node;

-    Inclination;

-    Mean anomaly at epoch time to.

-    The explicit epoch time associated to ephemeris data;

-    The location of the NTN-Gateways;

NOTE: 	The name of the IOCs and attributes are to be decided in the normative phase.

Besides satellite ephemeris data, satellite coverage information refers to location and time-related coverage availability of satellite/satellite constellation that provides discontinuous coverage can be provided by AMF/MME.

To determine UE’s out-of-coverage period, 3GPP management system could provide the satellite coverage information (timing information when UE moves out/in of NTN coverage or/and location) to MME/AMF and UE.

This solution uses the instances of following IOCs or attributes representing the unreachability period information, for example named as coverageInformation, and it contains the following attributes:

-    Wait range based on network configuration (e.g. TA list);

-    UE out-of-coverage period, which indicates the timing information for when a UE is expected to be out of coverage, and when the UE is expected to re-gain coverage again.

NOTE: 	Whether the satellite coverage information provided by AF via the NEF/SCEF or UE or OAM is to be decided by SA2

5.3.2	Use case #2 potential solutions for the monitoring of satellite components

5.3.2.1              Solution to monitor delay on the DL-Air Interface with MEO or GEO satellite components

Solution #1: Adapt the Average delay DL air-interface measurement and Distribution of delay DL air-interface measurement to support cases where HARQ feedback is disabled

The round-trip delay between a NB-IoT/eMTC UE and a MEO or GEO satellite can be very large when compared to a traditional terrestrial case. This effect requires changes to the HARQ process, so that it is able to cope with these high latencies. In TR 38.821[8] it is proposed to optimise the HARQ process for MEO and GEO satellites by increasing the number of HARQ processes and to add an option to disable or enable HARQ feedback for the cases when the increased number of HARQ processes is still not sufficient. .

The Average delay DL air-interface measurement (clause 5.1.1.1.1 of TS 28.552[10]) and Distribution of delay DL air-interface measurement (clause 5.1.1.1.2 of TS 28.552[10]) use the HARQ feedback ACK message to verify that the transmitted and timed RLC SDU packet was received successfully by the NB-IoT/eMTC UE. This HARQ ACK may not be transmitted when the HARQ feedback is disabled for MEO and GEO satellite links.

6	Conclusions and recommendations

In conclusion, the following issues are identified in this study:

-	Reference management architectures and scenarios for integrated satellite components;

-	Use cases and solutions related to the general management of satellite RAN with regenerative/transparent satellite components;

-	Use cases and solutions related to the monitoring of satellite RAN.

It is recommended to consider the following aspects for normative work:

-	Specify/extend SON concepts to allow for moving non-terrestrial NBs.

-	Adapt the performance measurements which make use of the HARQ process, which may be unavailable when using satellite RAN.

-	Introduce the NRM based on solutions to support handling of coverage holes or discontinuous satellite coverage in a power efficient way.

Annex A

Characteristics of satellite systems

A.1	General

This annex describes the main characteristics of satellite systems when considering their integration with the 5G system.

A.2	Class of orbit

On the one hand, our planet attracts as a main body the much smaller satellite, which motion is dictated as a consequence by the laws of Kepler. On the other hand, the environment of Earth can be also constraining: the higher density of the atmosphere, debris from launchers and former satellites in the lower altitudes, as well as higher energy particles trapped in the Van Allen belts between 2,000 and 8,000 km's altitudes are to be avoided. These two constraints contribute to defining several classes of orbits that are used for communication satellites:

-	Geostationary (GEO) satellites, located precisely in the plane of the Equator at an altitude of 35,786 km, these satellites rotate at the same rate as the Earth's rotation: a GEO satellite stands still with respect to Earth. Thanks to this property, a single GEO satellite is sufficient to create a continuous coverage.

-	Non-Geostationary Orbiting (NGSO) satellites: NGSO satellites do not stand still with respect to Earth. Should service continuity be required over time, a number of satellites (a constellation) is required to meet this requirement; the lower the altitude the higher the number of satellites.

Different classes of NGSO satellites are listed below:

-	Low-Earth Orbiting (LEO) satellites, with altitude ranging from 500 km to 2,000 km, and with inclination angle of the orbital plane ranging from 0 to more than 90 degrees. These constellations are placed above the International Space Station and debris, and below the first Van Allen belt.

-	Medium-Earth Orbiting (MEO) satellites, with altitude ranging from 8,000 to 20,000 km. The inclination angle of the orbital plane ranges from 0 to more than 90 degrees. These constellations are placed above the Van Allen belts.

-	Highly-Eccentric Orbiting (HEO) satellites, with a range of operational altitudes (the orbit of such satellites being designed for the spacecraft to be exploited when the vehicle is closer to its apogee - the higher part of the orbit -) between 7,000 km and more than 45,000 km. The inclination angle is selected so as to compensate, completely or partially, the relative motion of Earth with respect to the orbital plane, allowing the satellite to cover successively different parts of Northern land masses (e.g. Western Europe, North America, and Northern Asia).

Figure A.2-1: Illustration of the classes of orbits of satellites

Annex B

Reference architecture with satellite enabled RAN

This annex depicts a reference architecture for a direct access with a satellite enabled RAN.

E-UTRAN supports radio access over non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs. Non-terrestrial networks encompasse platforms that provide radio access through satellites in Geosynchronous orbits (GSO) as well as Non-Geosynchronous Orbit (NGSO), which includes Low-Earth Orbit (LEO) and Medium Earth Orbit (MEO).

Knowing the possible nature of satellite payloads (transparent or re-generative), the instantiation of this architecture can have several forms:

B.1	CIoT EPS with transparent satellite enabled RAN

In this implementation, the satellite is transparent, the signals are generated from eNB's from a satellite enabled RAN that are located on ground. The satellite is equivalent to a Radio Frequency (RF) Remote Unit, and is full transparent to the IoT protocols, including the physical layer (see below). This implementation uses CIoT-Uu interface to transport signals between UEs and eNBs, and uses SGi between 5GC/EPC and CIoT services.

Figure B.1: CIoT EPS with transparent satellite enabled RAN

B.2	CIoT EPS with regenerative satellite enabled RAN and on-board eNB

In this implementation, the satellite is regenerative. The satellite payload implements a full eNB supporting a satellite enabled RAN. A Satellite Radio Interface (SRI) transports the S1 interface between the on-ground EPC/5GC and the on-board eNB (see below). This implementation uses SGi between 5GC/EPC and CIoT services.

Figure B.2: CIoT EPS with regenerative satellite enabled RAN and on-board eNB

One can note that with a regenerative implementation a CIoT EPS can have a global coverage, providing a single EPC/5GC with global or regional (continental or sub-continental) coverage as well. This is valid for GEO or LEO.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 07-2022 | SA5#144e | S5-224065 S5-224066 S5-224067 S5-224068 S5-224433 |  |  |  | Update to implement the agreed pCRs in SA5#144e: - S5-224065 Initial skeleton (v0.0.0) of TR 28.841 - S5-224066 pCR 28.841 Add TR structure - S5-224067 pCR TR 28.841 Add Scope - S5-224068 pCR TR 28.841 Add Introduction - S5-224433 pCR 28.841 Add concepts and background | 0.1.0 |
| 08-2022 | SA5#145e | S5-225385 S5-225405 |  |  |  | Update to implement the agreed pCRs in SA5#145e: - S5-225385 pCR 28.841 update structure - S5-225405 pCR 28.841 add scenarios for IOT networks with an integrated satellite component | 0.2.0 |
| 11-2022 | SA5#146 | S5-226392 S5-226393 S5-226394 S5-226395 S5-226397 |  |  |  | Update to implement the agreed pCRs in SA5#146: - S5-226392 pCR 28.841 add use cases for IoT network of satellite components - S5-226393 pCR 28.841 add potential requirements of satellite components - S5-226394 pCR 28.841 add potential solutions for IoT networks of satellite components - S5-226395 pCR 28.841 add Annex of characteristics of satellite systems - S5-226397 pCR 28.841 add Annex of Reference architecture with satellite enabled RAN | 0.3.0 |
| 03-2023 | SA5#147 | S5-232438 S5-232439 S5-232867 S5-232868 S5-232869 S5-232870 S5-232871 |  |  |  | Update to implement the agreed pCRs in SA5#147: - S5-232438 pCR 28.841 add references and abbreviations - S5-232439 pCR 28.841 Rapporteur clean up - S5-232867 pCR 28.841 Add use case for NTN mobility management enhancement - S5-232868 pCR 28.841 Add potential requirement for NTN mobility management enhancement - S5-232869 pCR 28.841 Add possible solution for NTN mobility management enhancement - S5-232870 pCR 28.841 add management architectures for satellite components - S5-232871 pCR 28.841 add conclusion and recommendations | 0.4.0 |