3GPP TSG-SA5 Meeting #147	S5-233053

# Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

# Source:	Samsung

# Title:	pCR 28.903 Logging Key Issues.docx

# Document for:	Approval

Agenda Item:	6.8.2.7

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1]	New Study on Basic SBMA enabler enhancements SP-220145

# 3	Rationale

This contribution provides logging related Key Issues and requirements.

# 4	Detailed proposal

First modification

## 4.5	Issue #x: Logging

### 4.5.1	Issue description

Logging is a crucial functionality of the OAM systems. It assist in several functionalities e.g troubleshooting. The functionality of logging is available in IRP framework in TS 32.332. However, the SBMA lacks the ability of Logging.

The SBMA (service based management architecture) principle call for a management system where the MnS (management services) are defined for all management tasks e.g provisioning, performance assurance.  The authorized consumer can access the MnSes as per their requirements. The current SBMA framework do not provide any mechanism to control and produce the logs for consumed management services.

The logging in IRP framework is restricted to only to logging of notifications. However, logging of Management Service require logging of all Component A, B and C i.e what operation and notifications were accessed, what were the related MIBs and what PM/KPI data was collected.

The MnS can be accessed from within the management system and from outside the management system. The logging may support both the scenarios.

### 4.5.2	Analysis

The logging mechanism for SBMA framework need to be defined.

### 4.5.3 Potential Requirements

- The 3GPP Management System shall have the capability to enable logging including recording of the operations invoked.

- The 3GPP Management System shall have the capability to enable logging including recording of the notifications produced.

- The 3GPP Management System shall have the capability to enable logging including recording of the MIBs accessed.

- The 3GPP Management System shall have the capability to enable logging including recording of the performance measurements reporting. This implies logging the measurement job contents, but excludes logging the individual measurement results.

- The 3GPP Management System shall have the capability to enable logging including recording of the KPIs produced. This implies logging the KPI generated, but excludes logging the individual KPI results.

- The 3GPP Management System shall provide a way for the consumer to retrieve the logs.

- The 3GPP Management System shall have the capability to manage log overflow situation.

- The 3GPP Management System shall include a timestamp for each log record.

- The 3GPP Management System shall include the user/consumer identity for each log record.

### 4.5.2 Potential Solutions

TBD

### 4.5.3	CR proposal

TBD