# 3GPP TSG-SA5 Meeting #147	S5-233002

# 27 Feb to 3 Mar 2023, E-meeting                                                                was S5-232696

# Source:	China Telecom

# Title:	Provide description of the requirements having been discussed in KI#5 TR28864

# Document for:	Approval

# Agenda Item:	6.7.6.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.864-050: "Study on Enhancement of the management aspects related to NWDAF".

3	Rationale

It's better to provide explicit requirements based on description of the KI#5 and the potential solutions discussed, so that the reader can follow the discussion easier.

4	Detailed proposal

Start of 1st Change

## 4.5	Key Issue #5: Performance Measurement on Data Collection Exposed by NWDAF

4.5.1      Description

Data collection play an important role for the NWDAF providing analytic services and model training services.

In the Rel-17, the enhanced data collection feature is introduced, with which the data collection may be performed via Nnwdaf DataManagement services provided by NWDAF.

The Nnwdaf_DataManagement services enable the consumer to subscribe/unsubscribe, be notified about data exposed by NWDAF, or fetch the subscribed data. The consumer of the Nnwdaf_DataManagement services can be the other NWDAF instances.

The operator may be interested in the how often the Nnwdaf_DataManagement services are requested or how many data are provided via Nnwdaf_DataManagement services.

On one hand, the service provider NWDAF will prepare the data before they are provided to consumer, and since the preparation includes both collection and processing of the data from the other data sources and historical data having been collected by NWDAF, this measurement will indicate how busy the NWDAF is on data collection related services which can be taken as a reflection of the overhead variation of NWDAF. This measurement will also lead to adjusting the network resource allocated to the service provider NWDAF instance to support data exchange with other consumers. And on some other cases, this measurement may indicate that deploying a dedicated DCCF is necessary.

On the other hand, as the enhanced data collection feature is introduced for Rel-17 NWDAF to increases data collection efficiency, this measurement will be a complimentary for the performance measurement of the NWDAFs which does not support DataManagement services. This measurement will also provide information on how many data collection is performed with the more effective manner, which can contribute the potential reduction of the CAPEX.

In this key issue, the potential solutions are provided on the performance measurement about the Nnwdaf_DataManagement services provided by NWDAF.

### 4.5.2      Potential Requirements

REQ-NWDAF-DM-SERV-COUNT-1：the 3GPP management system should have a capability to provide the statistic information of the Nnwdaf_DataManagement services provided by NWDAF about how frequently the services are requested and about the service successful rate.

### 4.5.3      Potential solutions

#### 4.5.3.1	Potential solution #1: number of Nnwdaf_DataManagement service subscriptions and/or requests

##### 4.5.3.1.1	Introduction

This solution is proposed on how to provide the performance measurement about the Nnwdaf_DataManagement service provided by NWDAF. In this solution, the proposed performance measurements are defined as the number of Nnwdaf_DataManagement service subscriptions and/or requests received by NWDAF.

The proposed solution can be a potential solution for the performance measurement about the Nnwdaf_DataManagement services provided by NWDAF.

##### 4.5.3.1.2	Description

The proposed performance measurements about the Nnwdaf_DataManagement service are defined as the number of Nnwdaf_DataManagement service subscriptions and/or requests received by NWDAF.

The number of Nnwdaf_DataManagement service subscriptions received by NWDAF can be measured as follows:

- When the NWDAF receives the Nnwdaf_DataManagement_Subscribe service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted subscription is added to the relevant counter.

- When the NWDAF receives the Nnwdaf_DataManagement_Subscribe service operation (See TS 23.288 [2]) from the NWDAF service consumer, each rejected subscription is added to the relevant counter.

The number of Nnwdaf_DataManagement service requests received by NWDAF can be measured as follows:

- When the NWDAF receives the Nnwdaf_DataManagement_Fetch service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted request is added to the relevant counter.

- When the NWDAF receives the Nnwdaf_DataManagement_Fetch service operation (See TS 23.288 [2]) from the NWDAF service consumer, each rejected request is added to the relevant counter.

#### 4.5.3.2	Potential solution #2:  number of Nnwdaf_DataManagement service notifications and/or responses

##### 4.5.3.2.1	Introduction

This solution is proposed on how to provide the performance measurement about the Nnwdaf_DataManagement service provided by NWDAF.  In this solution, the proposed performance measurements are defined as the number of Nnwdaf_DataManagement service notifications and/or responses generated by NWDAF.

The proposed solution can be a potential solution for the performance measurement about the Nnwdaf_DataManagement services provided by NWDAF.

##### 4.5.3.2.2	Description

The proposed performance measurements of the NWDAF are defined as the number of Nnwdaf_DataManagement service notifications and/or responses generated by NWDAF.

The number of Nnwdaf_DataManagement service notifications generated by NWDAF can be measured as follows:

- When the NWDAF generates the Nnwdaf_DataManagement_Notify service operation to NWDAF service consumer (See TS 23.288 [2]), each generated notification is added to the relevant counter.

- When the NWDAF generates the Nnwdaf_DataManagement_Notify service operation to NWDAF service consumer (See TS 23.288 [2]), each generated error notification is added to the relevant counter.

The number of Nnwdaf_DataManagement service responses generated by NWDAF can be measured as follows:

- When the NWDAF generates the Nnwdaf_DataManagement_Fetch Response service operation to NWDAF service consumer (See TS 23.288 [2]), each generated response is added to the relevant counter.

- When the NWDAF generates the Nnwdaf_DataManagement_Notify service operation to NWDAF service consumer (See TS 23.288 [2]), each generated error response is added to the relevant counter.

### 4.5.4	Evaluation

In order to measure how busy the NWDAF is on data collection related services, there are multiple potential solutions for the performance measurement on data collection exposed by NWDAF. The data collection may be performed via Nnwdaf DataManagement services provided by NWDAF.

In the potential solution #1, the performance measurements on data collection exposed by NWDAF are provided as the number of Nnwdaf_DataManagement service subscriptions and/or requests received by NWDAF. In the potential solution #2, the performance measurements on data collection exposed by NWDAF are provided as the number of Nnwdaf_DataManagement service notifications and/or responses generated by NWDAF.

These two proposed performance measurements can indicate how often the Nnwdaf_DataManagement services are requested and how busy the NWDAF is on data collection related services.

Consequently, these two potential solutions are feasible candidates as input to the normative phase on the performance measurement on data collection exposed by NWDAF.

End of 1st Changes