3GPP TSG-SA5 Meeting #149 	S5-233870

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | - | Current version: | 17.K.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 solution for ML abstract behavior |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of ML abstract behavior has been discussed and agreed (in TR 28.908). This CR is to normatively propose the solution for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | sage 2 solution has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the use case “ML abstract behavior” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.V(new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 7.K	ML Inference emulation Information Model

## 7.K.1	Class diagram

##### 7.K.1.1	Relationship

@startuml

' UML diagram for 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class "MLInferenceFunction"<<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

class abstractBehavior <<InformationObjectClass>>

ManagedEntity "1" *-- "*" "MLInferenceFunction":<<names>>

"MLInferenceFunction""1" --> "*" MLEntity:<<names>>

"MLInferenceFunction""1" *--> "*" abstractBehavior:<<names>>

MLEntity "1" *-r-> "*" abstractBehavior:<<names>>

note left of ManagedEntity #white

Represents the IOCs:

Subnetwork or

ManagedFunction or

ManagementFunction

end note

@enduml

@startuml

' UML diagram for 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class "MLInferenceFunction"<<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

class abstractBehavior <<InformationObjectClass>>

ManagedEntity "1" *-- "*" "MLInferenceFunction":<<names>>

"MLInferenceFunction" "1" --> "*" MLEntity:<<names>>

MLEntity "1" *-r-> "*" abstractBehavior:<<names>>

note left of ManagedEntity #white

Represents the IOCs:

Subnetwork or

ManagedFunction or

ManagementFunction

end note

@enduml

Figure 7.K.1.1-1: NRM fragment for ML Abstract behavior

##### 7.K.1.2	Inheritance

@startuml

' UML diagram for 3GPP TS 28.541 clause 6

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class Function <<InformationObjectClass>>

class "MLInferenceFunction"<<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

class abstractBehavior << InformationObjectClass >>

Function <|-- "MLInferenceFunction"

Top <|-- MLEntity

Top <|-- abstractBehavior

@enduml

Figure 7.K.1.2-1: ML inference emulation Inheritance Relations

## 7.K.2	Class definitions

### 7.K.2.1.	MLInferenceFunction <<IOC>>

#### 7.K.2.1.1.	Definition

This IOC represents the properties of an MLInferenceFunction.

Each MLInferenceFunction is a managed object instantiable from the MLInferenceFunction information object class and name-contained in either a Subnetwork, a ManagedFunction or a  ManagementFunction. The MLInferenceFunction is a type of managed Function, i.e., the MLInferenceFunction is a subclass of and inherits the capabilities of a managed Function

- Each MLInferenceFunction shall be associated with one or more MLEntities.

- Each MLInferenceFunction may be associated infact shall have a candidateAbstractBehavior

- Each MLInferenceFunction may be associated, in fact shall have, one or more instances of AbstractBehavior which is a pair of lists respectively containing the candidate and the selected state-action pairs.

- An instance of AbstractBehavior at the MLInferenceFunction may also be associated with a specific MLEntity

- The abstractBehavior is conditionally mandatory with the condition that it must be associated with the MLInferenceFunction if it is not associated with the mLEntity that itself is associated with the MLInferenceFunction

#### 7.K.2.1.2.	Attributes

The MLInferenceFunction IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLInferenceFunctionId | CM | T | F | F | F |

### 7.K.2.1.	MLEntity <<IOC>>

#### 7.K.2.1.1.	Definition

This IOC represents the properties of an MLEntity .

Each MLEntity is a managed object contained on or associated with an MLInferenceFunction.

- Each MLEntity  may be associated, in fact shall have, an instance of AbstractBehavior

which is a pair of lists respectively containing the candidate and the selected state-action pairs.

- The abstractBehavior is conditionally mandatory with the condition that it must be associated with the MLEntity if it is not associated with the MLInferenceFunction for which the mLEntity computes outcomes.

#### 7.K.2.1.2.	Attributes

The MLEntity  IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLEntityId | CM | T | F | F | F |

#### 7.K.2.3 	abstractBehavior <<IOC>>

#### 7.K.2.3.1.	Definition

This IOC represents the properties of abstractBehavior  .

- The abstractBehavior  is associated to either an MLEntity or an MLInferenceFunction. The abstract behavior contains characteristics of the abstract behavior of the MLEntity or ML function.

- The abstract behavior contains 2 lists, a list of candidate abstract states and their candidate actions and a list of the selected and configured abstract states and their respective selected actions.

#### 7.K.2.3.2.	Attributes

The MLKnowledgeRequest IOC includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| candidateAbstractStates | M | T | F | F | F |
| appliedAbstractStates | M | T | F | F | F |

## 7.K.3.	DataTypes Definitions

#### 7.K.3.1. 	candidateAbstractState <<datatype>>

#### 7.K.3.1.1.	Definition

This dataType represents the properties of abstractState. The candidateAbstractState is a list of abstract states and where each state has a list of candidate abstract actions for that abstract state

Each abstractState, may be identfed with an identifier. The abstractState may be characterized by a human readable description which enables the human MnS consumers to know what features are grouped within that abstract state.

- Each abstractState may have at least 2 possible actions that may be taken within that state. These are listed in the possibleActions attribute on the abstractState. The possibleActions are an enumeration of possible actions from which the MnS consumers can pick one one that should be applied.

#### 7.K.3.1.1	Attributes

The abstractState <<datatype>> includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| abstractStateId | M | T | F | F | F |
| Description | O | T | F | F | F |
| possibleActions | M | T | F | F | F |

#### 7.K.3.1. 	appliedAbstractState <<datatype>>

#### 7.K.3.1.1.	Definition

This dataType represents the properties of appliedAbstractState. The appliedAbstractStates is  a list of state-action tuples.

- Each appliedAbstractState has 1 action that has been selected either by the MnS producer or by an MnS consumer to be applied.

- Each state may be represented by an identifier for the respective state as listed in the candidateAbstractStates.

- Similarly, each action may be represented by an identifier for the respective action as listed in the candidateAbstractBehavior

#### 7.K.3.1.1	Attributes

The abstractState <<datatype>> includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| abstractStateId | M | T | F | F | F |
| action | M | T | F | F | F |

## 7.K.3	DataType definition

##### 7.K.3.1 	MLInferenceEmulationReport << dataType >>

###### 7.K.3.1.1	Definition

This dataType represents the properties of MLInferenceEmulationReport.

- MLInferenceEmulator may generate one or more MLInferenceEmulationReports,

- Each MLInferenceEmulationReport is associated to one or more MLEntitys.

- All MLInferenceEmulationReports must be associated with the MLInferenceEmulationJob instance, i.e., the MLInferenceEmulationJob provides/instantiates reports about MLEntitys or about the MLInferenceEmulationJob that is associated with the MLEntitys for which inference emulation is requested and/or executed. It may also report on specific MLInferenceEmulationRequests.

7.K.3.1.2		Attributes

The MLInferenceEmulationReport includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| MLInferenceEmulationReportID | M | T | F | F | F |
| MLEntitys | M | T | F | F | F |
| Attributes related to role |  |  |  |  |  |
| MLInferenceEmulationRequests | O | T | F | F | F |
| MLInferenceEmulationJobRef | M | T | F | F | F |

## 7.K.4	Attribute definitions

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLInferenceFunctionId | It indicates an identifier for the MLInferenceFunction | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEntityId | It indicates an identifier for a specific MLEntity on or associated with the MLInferenceFunction and whose abstract behavior shall be described. It may include the version identifiers of any such MLEntity or MLEntity | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| candidateAbstractStates | It indicates the list of candidate abstract states and their candidate actions for the MLEntity or the associated with the MLInferenceFunction | type: candidateAbstractState multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| appliedAbstractStates | It indicates the list of the selected and configured abstract states and their respective selected actions for the MLEntity or the associated with the MLInferenceFunction | type: appliedAbstractState multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| abstractStateId | It indicates an identifier for the respective state | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| Description | It indicates a human understandable description of of the abstractState. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| possibleActions | It indicates the possible actions that could be taken in a given abstractState . an abstractState may have at least 2 possible actions that may be taken within that state. | type: enum multiplicity: 2..* isOrdered: False isUnique: True defaultValue: None isNullable: True |
| action | It indicates an identifier for the respective action as listed in the candidateAbstractBehavior that can be selected by the MnS consumer | type: enum multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |

End of Changes