3GPP TSG-SA5 Meeting #149 	S5-234481

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson

# Title:	Rel-18 pCR 28.826 Adding use case and key issue on handling of event charging triggers

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: " Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Adding new use case and key issue on handling of event charging triggers.

# 4	Detailed proposal

First change

#### 5.2.2.x	Use Case #2x: Trigger handling for IEC and PEC

An operator has set up event based charging for 5G connection and mobility domain charging and wants to be able to control the applicable trigger conditions for each functionality (i.e., Registration, N2 connection, Location Reporting) during the UE registration lifetime in the AMF i.e., activate/deactivate charging for any AMF functionality.

Second change

### 5.2.3	Potential charging requirements

REQ-CH_REQ-01: Possibility to support other than immediate trigger for own defined management or network triggers.

REQ-CH_REQ-x: Possibility to support trigger setting for event based charging.

Second change

### 5.2.4	Key issues

Key Issue #2a: What trigger to use.

Key Issue #2b: What information to be included with the trigger, if any.

Key Issue #2x: Allow the CHF to set triggers for events.

End of changes