3GPP TSG-SA5 Meeting #149 	S5-234630

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson

# Title:	Rel-18 pCR 28.839 Adding solution using charging from trusted AF and PEC

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.839: "Study on charging aspects for Time Sensitive networking"

# 3	Rationale

Adding solution using charging from trusted AF and PEC.

# 4	Detailed proposal

First change

## 7.x		Solution #x: Exposure of Time Synchronization Service with trusted AF

### 7.x.1	General

The solution #x, addresses the Key Issue #1a and #1b, covering the REQ-CH_ETS-02.

The TS 23.501 [3] clause 4.4.8.3 states that if the AF is considered to be trusted by the operator, the AF could interact directly with TSCTSF. The TS 23.502 [4] clause 5.2.27, describes the TSCTSF services that either the AF or NEF can consume:

-	Ntsctsf_TimeSynchronization_: create update and delete time synchronization configuration and to activate and deactivate the time synchronization service, further described in TS 23.501 [3] clause 5.27.1.8.

-	Ntsctsf_ASTI_: control the 5G access stratum time distribution configuration, further described in TS 23.501 [3] clause 5.27.1.8.

-	Ntsctsf_QoSandTSCAssistance_: specific QoS and provide assistance for handling traffic characterized by TSC QoS parameters, further described in TS 23.503 [20] clause 6.1.3.22.

### 7.x.2	Architecture description

Figure 7.x.2-1: AF converged charging architecture

### 7.x.3	Flow description

The following figure 7.x.3-1 describes the AF session with required QoS update procedure, based on figure 4.15.6.6-1 TS 23.502 [4] description:

Figure 7.x.3-1: AF Request directly to TSCTSF using PEC

1 to 4b.	Start and determination that TSCTSF needs to be invoked, implying that steps 3a, 3b, 4a, 4b, 5, 6a, 7a, 7b, and 8 from figure 4.15.6.6-1 TS 23.502 [4] are applicable.

4ch-a.	The AF sends Charging Data Request [Event] to CHF for the received Ntsctsf_QoSandTSCAssistance_Create response.

4ch-b.	The CHF creates a CDR.

4ch-c.	The CHF acknowledges by sending Charging Data Response [Event] to the AF.

6a to 6b.	From figure 4.15.6.6a-1 TS 23.502 [4]

6ch-a.	The AF sends Charging Data Request [Event] to CHF for the received Ntsctsf_QoSandTSCAssistance_Notify.

6ch-b.	The CHF creates a CDR.

6ch-c.	The CHF acknowledges by sending Charging Data Response [Event] to the AF.

Editor’s note: the subscriber id is ffs.

End of changes