3GPP TSG-SA5 Meeting #149 	S5-234662

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0040 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the use cases for Intent containing an expectation for delivering 5GC subnetwork |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AsiaInfo,Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The objective of IDMS_MN_ph2 is that specifies requirements and solutions for new scenarios for Intent driven approach for 5GC management, specifically 5GC network delivering |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the use cases for Intent containing an expectation for delivering 5GC radio networknetwork |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intent based management of 5GC subnetwork is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.X	Intent containing an expectation for 5GC network

5.1.X.1	Introduction

This use case describes a scenario where a MnS consumer express intent containing an expectation related to 5GC network to a MnS producer. In this scenario, MnS consumer expresses its intent expectation which may include location information (e.g. geographic location, data center), type of the network (e.g. toB), included 5GC NF list (e.g. NF types information, range of NF instance ID), PLMN information, supported APN information, transport related parameters (e.g. list of related end point addresses information), and target network capacity information (e.g. number of PDU session of network, number of registered subscribers, UL/DL throughput)

Based on the intent containing an expectation related for 5GC network as received, the intent MnS producer decides whether to deploy a new 5GC network in the specified location or to reconfigure an existing 5GC network. If a new 5GC network is to be delivered, the intent MnS producer generates network configuration parameters (including 5GC network/NFs configuration parameters and transport network configuration parameters). If an existing 5GC network is to be reconfigured, the intent MnS producer identifies the potential 5GC performace issues (e.g. low performance because of high load ) for the existing 5GC network, modifies the 5GC NF configuration parameters if needed to satisfy the performance expectation targets (this may also trigger scaling procedure by interworking with ETSI NFV MANO). Multiple interactions between the Intent MnS consumer and the Intent MnS producer may be needed based on the intent management capabilities (e.g. intent translation and intent feasibility check) provided by intent MnS producer.

The Intent MnS producer continuously monitors the 5GC performance (e.g. mean number of registered UE, mean number of created PDU session), and decides whether 5GC related expectation is satisfied.If the 5GC related expectation is not satisfied, the intent MnS producer identifies the potential 5GC performace issues and modifies the 5GC NF configuration parameters if needed to satisfy the performance expectation targets.

On a regular basis, MnS producer notifies MnS consumer about the fulfilment information of the intent.

5.1.X.2	Requirements

REQ-Intent_5GC_Net-CON-1 The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for delivering a 5GC network for the specified area to MnS producer.

REQ-Intent_5GC_Net-CON-2 The intent driven MnS shall have capability enabling MnS consumer to obtain fulfilment information of the intent containing an expectation for delivering a 5GC network.

REQ-Intent_5GC_Net-CON-3 The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation on 5GC network performance to be assured to the Intent MnS producer.

REQ-Intent_5GC_Net-CON-4 The intent driven MnS shall have capability enabling MnS consumer to obtain fulfilment information of the intent containing an expectation on 5GC network performance to be assured.