3GPP TSG-SA5 Meeting #149 	S5-234716

# Berlin, Germany	May 22 – May 26, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | draftCR | rev | 3 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | InputToDraftCR Rel-18 28.533 on Access control for management service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Telefonica, Deutsche Telekom, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSAC |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Stage 2 contributions related to the introduction of IOCs related to the Stage 1 approved requirements with focus on authorization for static information object classes and attributes. The requirements elaborated are related to authorization for permissions related to static objects |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Stage 2 related IOCs for usecases and requirements related to static information object classes and attributes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The design and subsequent implementation for access control in management systems for role-based access control will not progress |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.x, Y.x, Annex Z.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a revision of S5-234381 S5-234381 is the input to latest approved DraftCR S5-233137 |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[xx]	IETF RFC 6749: " The OAuth 2.0 Authorization Framework".

[yy]	IETF RFC 8341: " Network Configuration Access Control Model".

First change

## X.2	Class diagrams

### X.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for the concept of the role based access control. This clause provides an overview of the relationships between relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

In order to realise the use cases for authentication and authorization, the need is to model the required IOCs and the relevant operations which utilise the IOCs. The operations supported on the IOCs are those supported by the generic management services.

As mentioned in the usecases the IOCs are provisioned during the integration time by a network operator. The data is sent to a authentication and authorization service producer.

Post this during integration time when a MnS consumer invokes an operation on the MnS producer, the authentication and authorization service producer validates the action on the resource to enable the decision for the MnS producer. The decision could be that the MnS producer allows or disallows the action on the resource.

Below are the IOCs that need to interact in order to store the data required for the authentication and authorization operations with respect to role based access control.

Figure X.2.1-1 NRM for role based access control name containment

The IOCs related to Identity, Role,  PermissionForMnSs are name contained within the Subnetwork or ManagedFunction or ManagedElement.

Editor’s note: access right, for particular consumer, pertaining to attributes of Component B will differ. How to model this functionality is FFS

Figure X.2.1-1 NRM for role based access control relationship

### X.2.2	Inheritance

This subclause depicts the inheritance relationships. The IOCs are derived from the TOP class

Figure X.2.2-1 Inheritance diagram for role based access control

## X.3	Class definitions

### X.3.1	Identity

#### X.3.1.1	Definition

This IOC represents an identity of a MnS consumer.

It is used for authentication and authorization. It can be name-contained by SubNetwork or Managed Function or a Managed Element.

The MnS consumer can be a human or a machine user. This IOC enables the creation and storage of an identity of a MnS consumer. The information in this IOC is the starting point for a MnS consumer to identity who it is. This is validated against an authentication service producer.

For the authentication operation to take place the identity related information has to be provisioned into the system by a network operator who could be an administrator. The administrator adds the identityType attribute and identityName attribute which characterizes a machine user or human user respectively. For example we could have an identity like a tenant mapped to the relevant list of roles.

Attribute credential is used to provide information for the credential used together with identity when requesting authentication. The examples of credential are password, certificate, biometric, etc.

The roleRefLis attribute defines the role names associated to a particular user

The IOC stores the details of the expected tasks to be performed by an identity. The tasks are what is to be done on the network management system. To ease the administration on the system, the tasks are organised as roles. The user can be associated to one or more roles.

#### X.3.1.2	Attribute

The Identity IOC includes the attributes inherited from Top IOC (as defined in Generic resource Model) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| identityType | M | T | F | F | T |
| identityName | M | T | F | F | F |
| credential | O | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| roleRefList | M | T | F | F | T |

#### X.3.1.3	Attribute constraints

None

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions

### X.3.2	Role

#### X.3.2.1	Definition

The Role IOC represents a task or collection of tasks in a network management system.

The Role can be name contained under the Subnetwork or Managed Function or a Managed Element.

The Role IOC enables the storage of information as to what resources and actions an identity can work upon. This IOC  maintains the static resources that are known to the management system during integration time. This contains all the granular level resources and the corresponding actions.

The permissionsForMnSs attribute contains a list of PermissionForMnSs to contain the list of granular permission sets

#### X.3.2.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| roleName | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| permissionsForMnSsList | M | T | F | F | T |

#### X.3.2.3	Attribute constraints

None

#### X.3.2.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions

### X.3.3	PermissionForMnSs

#### X.3.3.1	Definition

The PermissionForMnSs IOC represents the granular resource and actions in a network management system.

The PermissionForMnSs can be name contained under the Subnetwork or Managed Function or a Managed Element

This IOC enables the storage of the resource types in the system and the possible actions that are allowed on it. The permutations and combinations of these permissions are assigned to a role.

The mnsType attribute is the type of the management service as defined TS 28.622.

The permissionList attribute contains a list of permissions.

#### X.3.3.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnsType | M | T | T | F | F |
| permissionList | M | T | T | F | F |

#### X.3.3.3	Attribute constraints

None

#### X.3.3.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions

### X.3.4	Permission <<dataType>>

#### X.3.4.1	Definition

The Permission data type represents the most granular resource and corresponding actions in a network management system.

#### X.3.4.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| componentA | M | T | F | F | F |
| componentB | M | T | F | F | T |
| componentC | O | T | F | F | T |

#### X.3.4.3	Attribute constraints

None

#### X.3.4.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions

## X.4	Attribute definitions

### X.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| identityType | This defines a readable string to uniquely represent an identity AllowedValues: username, email address, phone number, IP address, machineuser | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| identityName | This indicates a type of identifier, AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| credential | The credential of an MnS consumer or producer used for authentication with authentication service producer. It could be password, certificate, key, pass phrase, etc., based on authentication protocol and factor. AllowedValues: NA | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No value isNullable: False |
| roleRefList | This defines the list of roles associated with an identity AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| roleName | This string defines the name of a role AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| mnsType | This attribute defines the type of management service AllowedValues: ProvMnS, FaultSupervisionMnS, StreamingDataReportMnS, FileDataReportingMnS | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| permissionsForMnSsList | This defines the list of permissions for MnSs which constitue the resources and corresponding actions AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| permissionList | This defines the list permissions for resources and corresponding actions for static objects AllowedValues: NA | type: Permission multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentA | This attribute is as indicated in 28.532 AllowedValues: values as defined in 28.532 | type: FFS multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| componentB | This attribute defines a MnS component type B represented by information models of managed entities. A MnS component type B is also called Network Resource Model (NRM). MnS component type B examples are: 1) Network resource models as defined in TS 28.622 2) Network resource models as defined in TS 28.541 AllowedValues: NA | type: FFS multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentC | This attribute defines a MnS component type C which is the performance information of the managed entity and fault information of the managed entity. The following are examples of Management service component type C: 1. Alarm information as defined in TS 28.532 and TS 28.545 2. Performance data as defined in TS 28.552 , TS 28.554 and TS 32.425. AllowedValues: NA | type: FFS multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

## X.5	Common Notifications

FFS

## Y.x	Access Control management service

### Y.x.1	Operations

#### Y.x.1.1	authentication operation (CM)

##### Y.x.1.1.1	Description

This operation is invoked by MnS consumer to request the authentication service producer to authenticate MnS consumer. MnS consumer and authentication service producer shall use one of the following methods for authentication:

-	Mutual authentication between MnS consumer and authentication service producer provided by the transport layer protection solution.

-	Client credentials assertion based authentication.

When client credentials assertion based authentication is used, the operation is invoked by a MnS consumer to request authentication by authentication service producer. The authentication service producer gets the identifier and credential of the MnS consumer for validation. Based on identifier in the request, authentication service producer gets identity information, e.g. credential of the identity, associated role(s) of the identity, etc., from local data store (e.g., a directory). The authentication service producer authenticates the MnS consumer by validating the identity information. associated to the role(s) the MnS consumer belongs to.

If authentication is successful, the authentication service producer sends successful response with assertion to the MnS consumer. MnS producer validates the assertion to authenticate a MnS consumer.

If fail for authentication, the authentication service producer sends failure response to the MnS consumer.

##### Y.x.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| identityName | M | String | This parameter specifies the identity of the MnS consumer that is to be authenticate. |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the identity to be authenticated. |

##### Y.x.1.1.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the authenticated MnS consumer and the actual value assigned to each. |
| Status | M | ENUM (OperationSucceeded, OperationFailed) |  |

#### Y.x.1.2	authorization operation (M)

##### Y.x.1.2.1	Definition

The operation is used to authorize a MnS consumer by the authorization service producer.

MnS consumer, MnS producer and authorization service producer shall use one of the following methods for authorization:

-

-    Token based authorization framework including various grant modes (e.g., as specified in RFC 6749 [xx])

-	Static authorization as specified (e.g.,. in RFC 8341 for NACM [yy])

When token based authorization framework is used, authorization service producer receives authorization request, gets the assertion of the MnS consumer related to the authentication and probably other context information (e.g. resource , action) from the request. If access token is supported by the MnS producer and consumer:

-  The MnS consumer gets access token from authorization service producer by providing the assertion from authentication session.

-  The MnS consumer starts normal operation with MnS producer with the access token. The MnS producer validates the token. If the token is valid, the MnS producer performs the action(operation) on the resource authorized and returns result to the MnS consumer.

If access token is not supported by the MnS producer and consumer:

-  When normal operation starts, the MnS producer validate the authentication assertion and check permission of the MnS consumer with authorization service producer.

-  The MnS producer performs the operation on the targeted resource which is part of the authorized scope and returns result to the consumer if the MnS request is allowed according to permissions.

If fail for authorization, the authorization service producer sends failure response to the consumer.

##### Y.x.1.2.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the identity to be authorized. |

##### Y.x.1.2.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the authorization for the MnS consumer and the actual value assigned to each. |
| Status | M | ENUM (OperationSucceeded, OperationFailed) |  |

End of changes

Start of next modification

# Annex Z (Informative):

# Z.1	Informative sequence diagram: Commissioning flow to be carried out by an administrator:

The sequence diagram below explains the IOCs to be used as a pre-requisite for enabling the role based access control. The notes above in the diagram also depict an example of each of the tasks.

The sequence diagram below explains the operations and the interactions with the authentication and authorization service which typically should suit any implementation be in oAUTH or a NACM implementation

The notes along the flows show the examples related to the IOCs provisioning operations.

Figure Z.1 - Sequence of commissioning for role based access control

## Commissioning usecases to be carried out by an administrator

- To carry out authentication and authorization based on role based access a set of tasks needs to be in place at the commissioning phase.

The network equipment vendor will typically take care of the below:

    - The granularity level at which the management objects need to be authorized

    - The actions that need to be permitted on these granular level objects

This is typically carried out by an administrator who has to setup the system for operational purposes which included authentication and authorization for every operation. Hence the administrator will need to have a good view of the permissions of the devices being integrated and managed. The administrator will need to know the possible scope of action for various users in the system on the instances that are created. The administrator will need to know the roles that need to be created in the system as well as the mapping of the same to the users. Below is the detail of every task.

The below elaborates the sequence flow to depict the IOCs involved for provisioning.

### a. Permissions with resources and actions

- Permissions are combinations of static resources and actions. The network equipment vendor can introduce the resources and actions at various levels of granularity as well as in the language that can be suitable for the vendor. When the commissioning phase happens it is important to translate these permissions to the authentication and authorization system so that it is aware of the static resources and corresponding actions on them for access control.

Hence the IOC for PermissionForMnSs is required to create the necessary data.

### b. Role creation with association to permissions and conditions

The meaningful task is the role which defines the responsibility realm for the user. Hence the role contains the overall distribution of resources and actions it can work on by resolving the condition and permissions. The static resources identified need to be combined with the dynamically created instances of the resource in order to be assigned to a meaningful task in the system.

The roles are typically created according to the convenience and hierarchy of the organisation.  Coarse grained or fine grained permissions can be created with the help of roles.

The management system should be setup for access control by a system administrator who will have the know how of creating the required users and roles. Roles will use the various possibilities to map resources and actions.

Hence the IOC for Role is required to create the necessary data related to various roles in the system.

### c. Identity to role assignment

The identity or user of the system needs to be assigned the responsibilities to carry out the OAM operations. Hence the set of responsibilities are defined in various roles. Assignment of the roles to an identity is required to make the administrative task more efficient and flexible. This also shields the administrator from getting into the details of the know how of granular resources and actions relevant to it.

Hence the IOC for Identity is required to create the necessary data.

# Z.2 Informative sequence diagram: Authentication and Authorization with role based access control:

Post the above tasks we have the possibility to have role-based access in operation with every service call being authenticated and authorized.

The below sequence diagram shows how the operations are invoked from a consumer towards an authentication and authorization service producer.

When an MnS is invoked, the MnS consumer authenticates towards an authentication service producer. This is then followed by checking the access rights with respect to the role-based access. This takes place in the authorization service producer which is invoked from the MnS producer to check if its resources can be accessed with respect to the related operation.

Hence then need of a authentication and authorization service producer which will validate a user who want to access the resources as well as validates what the user can access.

Figure Z.2 - Sequence diagram for operations of authentication and authorization

End of changes