3GPP TSG-SA5 Meeting #149 	S5-233901

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0176 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Procedure of Network Slice Subnet Instance Allocation to support asynchronous operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | S5-233155 was endorsed with the following: - Introduce a solution using NetworkSliceController IOC to monitor progress and result for network slice (subnet) allocation and deallocation as an enhancement to support async mode in 3GPP Rel-18. - The solution introduced above will support both network slice lifecycle management using allocate/deallocate operation and CRUD based operation. Network Slice and Network Slice Subnet LCM operations are long running and hence need to be asynchronous. The changes in this CR introduces support for asynchronous mode of operations for Procedure of Network Slice Subnet Instance Allocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Procedure of Network Slice Subnet Instance Allocation to support asynchronous or long running operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of shared objects as well as asynchronous or long running operations for Network Slice Subnet Instance Allocation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS28.541 CR 0800 TS28.623 CR 0214 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.3	Procedure of Network Slice Subnet Instance Allocation

The Figure 7.3-1 illustrates the procedure of creating a new NetworkSliceSubnet MOI or using an existing NetworkSliceSubnet MOI to satisfy the required network slice subnet related requirements.

Figure 7.3-1: Network Slice Subnet Instance Allocation Request procedure

1a-1)	If CRUD operations are not visible to the Network Slice Subnet Management Service Consumer (NSSMS_C), then Network Slice Subnet Management Service Provider (NSSMS_P) receives an AllocateNssi request (see AllocateNssi operation defined in clause 6.5.2) from NSSMS_C with network slice subnet related requirements (network slice subnet related requirements defined in SliceProfile see clause 6.3.4 in TS 28.541 [6]).

1a-2)	The NSSMS_P creates an MOI of NetworkSliceSubnetController.

1a-3) The NSSMS_P sends the response to NSSMS_C for the AllocateNssi request.

1b-1)	If CRUD operations are visible to the NSSMS_C, then NSSMS_P receives a createMOI request (see createMOI operation defined in TS 28.532 [8]) from NSSMS_C with network slice subnet controller (see NetworkSliceSubnetController IOC and its attribute defined in TS 28.541[6])) including the network slice subnet related requirements in attribute inputSliceProfile. The NSSMS_C may check the status and completion of the network slice instance allocation procedure any time by monitoring the values of NetworkSliceSubnetController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor by querying the values or by subscribing to notifications.

1b-2)	The NSSMS_P creates an MOI of NetworkSliceSubnetController.

1b-3)	The NSSMS_P sends the response to NSSMS_C for the createMOI request.

2)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for creation (see notifyMOICreation operation defined in TS 28.532 [8]) of NetworkSliceSubnetController MOI.

3) NSSMS_P may invoke corresponding procedure of feasibility check and reservation of network slice subnet as described in clause 7.14. If the network slice subnet related requirements can be satisfied, the following step 3) to step 5) are needed, else go to step 6).

4)	Based on the network slice subnet related requirements and the existing NetworkSliceSubnet MOI capabilities, NSSMS_P decides whether to use an existing NetworkSliceSubnet MOI or create a new NetworkSliceSubnet MOI. If the network slice subnet related requirements allow the requested NetworkSliceSubnet MOI to be shared and if an existing suitable NetworkSliceSubnet MOI can be reused, the NSSMS_P decides to use the existing NetworkSliceSubnet MOI.

5a-1) If using an existing NetworkSliceSubnet MOI, the NSSMS_ P identifies the NetworkSliceSubnet MOI to be re-used.

5a-2)	The NSSMS_P configures NetworkSliceSubnet MOI attribute networkSliceSubnetControllerRefList with the DN of NetworkSliceSubnetController MOI.

5a-3)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnet MOI.

5a-4)	NSSMS_P configures the NetworkSliceSubnetController MOI attribute networkSliceSubnetRef with DN of NetworkSliceSubnet MOI (identified in step 4a-1 or 4b-1).

5a-5)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnetController MOI.

5a-6)	If the existing NetworkSliceSubnet MOI needs to be modified to satisfy the network slice subnet related requirements, the NSSMS_P invokes the procedure of network slice subnet instance modification as described in clause 7.7.

5b-1)	If using a new NetworkSliceSubnet MOI, the NSSMS_P creates an MOI of NetworkSliceSubnet. The NSSMS_P configures NetworkSlice MOI attribute networkSliceSubnetControllerRefList with DN of NetworkSliceSubnetController MOI.

5b-2)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for creation (see notifyMOICreation operation defined in TS 28.532 [8]) of NetworkSliceSubnet MOI.

5b-3)	NSSMS_P configures the NetworkSliceSubnetController MOI attribute networkSliceSubnetRef with DN of NetworkSliceSubnet MOI (identified in step 4a-1 or 4b-1).

5b-4)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnetController MOI.

5b-5) NSSMS_P derives the corresponding network slice subnet constituent (i.e. NF, constituent NetworkSliceSubnet) related requirements and transport network related requirements (e.g. 3GPP endpoint information, latency requirements, bandwidth requirements and isolation requirements) from the received network slice subnet related requirements. Part of these requirements may be referenced by attribute epTransportRef  as defined in clause 6.3.2.2 in TS 28.541[6]. Before NSSMS_P derives the constituent network slice subnet related requirements, NSSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.2.

5b-6)	If the NetworkSliceSubnet MOIcontains virtualisation part (i.e. VNF or VL), NSSMS_P derives the NS instance instantiation information (the NS instance instantiation information is described in clause 7.3.2.2 and clause 7.3.3.2 [3]) based on network slice subnet related requirements. NSSMS_P determines new VNF instance(s) that need to be deployed and the existing VNF instance(s) that need to be reused according to the necessary network function(s) and then derives the profile of virtual link(s) according to the connection requirements between the network functions. NSSMS_P chooses a proper NSD deployment flavour and creates data concerning the SAPs of the NS instance. NSSMS_P invokes the NS instantiation procedures to create a NS instance.

Note:	NS instantiation procedure is described in TS 28.526 [7].

5b-7)	NSSMS_P configures NetworkSliceSubnet MOI attribute nsInfo with NS instance information.

5b-8)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnet MOI.

5b-9) For each required constituent network slice subnet (NetworkSliceSubnet MOIs), the following step 5b-9a) to 5b-9c) are needed:

5b-9a)	NSSMS_P invokes procedure of network slice subnet instance allocation.

5b-9b)	NSSMS_P configures NetworkSliceSubnet MOI attribute networkSliceSubnetRef with DN of NetworkSliceSubnet MOI.

5b-9c)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnet MOI.

5b-10)	For each required constituent NF instance (ManagedFunction MOI), the following step 5b-10a) to 5b-10c) are needed:

5b-10a)	NSSMS_P invokes procedure of NF instance creation described in clause 7.10 or procedure of NF instance modification described in clause 7.11.

5b-10b)	NSSMS_P configures NetworkSliceSubnet MOI attribute managedFunctionRef with DN of ManagedFunction MOI.

5b-10c)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnet MOI.

5b-11)	For each required transport network related requirements, NSSMS_P invokes corresponding procedure of coordination with relevant TN Manager to handle the TN part as described in clause 7.9.

6)	NSSMS_P configures NetworkSliceSubnetController MOI attributes operationalState, administrativeState, availabilityStatus and processMonitor to indicate completion of the procedure.

7)	If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnetController MOI attributes.

End of change