3GPP TSG-SA5 Meeting #149	S5-234552

# Berlin, Germany, 22-26 May 2023

# Source:	Huawei, CMCC

# Title:	pCR 28.830 Add description fault recovery related enhancement

# Document for:	Approval

Agenda Item:	6.7.4.2

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	TR 28.830 Fault supervision evolution

[3]	TS 28.104: "Management Data Analytics (MDA)"

[4]	TS 32.111-1: "Fault Management; Part 1: 3G fault management requirements"

[5]	TR 28.912: "Study on enhanced intent driven management services for mobile networks

3	Rationale

This tdoc discusses potential enhancements for fault recovery related enhancement in FSEV study [1, 2].

In TS 32.111-1 [4], the fault recovery related scenarios and actions are described.

In TR 28.912 [5] the intent driven management related studies, there are some descriptions about the  intent expectation for 5G Core network fault management.

Proposal 1: It is proposed to include the potential enhancements for fault recovery related functions.

4	Detailed proposal

1st Change

5.X	Key Issue #X: Potential enhancements for fault related analysis

5.X.1	Description

Editor’s note: This clause provides a description of the key issue.

If a potential fault/failure is predicted and reported to the consumer, the consumer would like to know the consequence. More analysis information on 3GPP system is helpful for the consumer to perform more proper actions, e.g., performance degradation analysis and predictions, KPI anomaly analysis and predictions, etc.

For example, the impacts on 3GPP system may not be perceived significantly in densely populated urban areas if there are overlapping coverage when a few sites encounter faults. However, service outage may occur due to faults in a site if there are few overlapping coverage in suburban areas. If this kind of information can be provided, the different handlings may be performed by the consumer.

Based on the topology relationship of the concerned managed objects, the impacted scope and degree are evaluated and provided. It may contain the following aspects:

-	scope and severity level of the impacted managed objects: the service types and coverage areas which are affected by the fault, e.g., the VoNR, URLLC service types, number of affected PDU sessions, and the coverage areas etc;

-	impacted managed objects: the managed objects which are affected by the fault, e.g. network slice, network slice subnet, network elements, network functions, number of affected gNBs and cells etc;

-	Rootcause types, name and descriptions of different types of rootcauses, etc.

-	Recommended actions for recovery;

5.X.2	Potential solutions

5.X.2.a	Potential solution #1: fault recovery related enhancement

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

In TS 32.111-1 [x], the fault recovery related scenarios and actions are described. In TR 28.912 [y] the intent driven management related studies, there are some descriptions about the intent expectation for 5G Core network fault management.

For the fault recovery, the MnS consumer of FM may provide some input information for the MnS producer of FM. The information may be the recovery time duration and the expectations of reduced service impacts, e.g. the maxmum acceptable number of impacted users within certain fault recovery time duration.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

5.X.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

End of change