# 3GPP TSG SA WG5 Meeting #150                 	           S5-235466

# Göteborg, Sweden, 21-25 August 2023

# Source:	Huawei

# Title:	Correction to solution #1.1

# Document for:	Approval

Agenda Item:	7.5.1

1	Decision/action requested

Include the proposed solution in TR 28.826.

2	References

[1] 		TR 28.826: "Study on Nchf charging services phase 2 improvements and optimizations".

3	Rationale

The solution description for the new immediate trigger variant is not precise. The change intends to make the following corrections:

-	The paragraph before Figure 5.1.5.1-3: propose to add a statement to describe the difference of the new immediate trigger variant from the previous two situations in Figure 5.1.5.1-1 and Figure 5.1.5.1-2.

-	The step 4 in Figure 5.1.5.1-3: the last statement is not correct, since if there are no no immediate triggers are met, the CTF won’t issue charging data request. Propose to remove the statement.

-	The step 6 in Figure 5.1.5.1-3: the last statement is not correct, since the process does not involve the two functions, but use the two functions. Propose to pephrase the statement based on the description in TS 32.290.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.826 [1].

First change

5.1.5.1	Solution #1.1: New immediate trigger variant

A possible solution for key issues #1a, #1b, and #1c covering requirements REQ-CH_INFO-01, charging information optimization.

This solution adds a variant of the immediate trigger category.

For the current immediate report chargeable events for which, when occurring, charging data generated by the NF consumer and sent to the CHF in a Charging Data Request message. Zero usage may be reported.

Figure 5.1.5.1-1: Message flow for current immediate triggers

1)	Service delivery ongoing: there is a connection setup for service delivery and a charging session is established between the NF (CTF) and the CHF.

2)	Trigger event: the NF (CTF) generates charging information related to the service delivered, due to that an immediate trigger for usage reporting or quota request is met.

3)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the CHF reporting the used units and if the service is under quota management it includes a quota request for the service to continue.

4)	Account, rating, and reservation control: The CHF performs the process related to the reported usage and if the service is under quota management it performs processes related to quota requested, it may involve the rating entity and user's account balance.

5)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

6)	Charging Data Response [Update, Quota granted]: The CHF responds and if the service is under quota management it includes a quota granted for the service to continue.

7)	Uplink/downlink transmission: first data or events transmission after the trigger event.

8)	Service delivery continuing service delivery is continuing and the charging session is continuing between the NF (CTF) and the CHF.

For the deferred report chargeable events for which, when occurring, charging data generated by the NF consumer and stored. Zero usage may be included. The charging data will be sent to the CHF in next a Charging Data Request message.

Figure 5.1.5.1-2: Message flow for deferred triggers

1)	Service delivery ongoing: there is a connection setup for service delivery and a charging session is established between the NF (CTF) and the CHF.

2)	Trigger event: the NF (CTF) generates charging information related to the service delivered, deferred trigger for usage reporting is met.

3)	Close counts and start new counts: the NF (CTF) close all counts related to the service and starts new, keeping the closed counts for reporting when an immediate trigger occurs.

4)	Uplink/downlink transmission: first data or events transmission after the trigger event.

5)	Service delivery continuing service delivery is continuing and the charging session is continuing between the NF (CTF) and the CHF.

For the new immediate report chargeable events for which, when occurring and there is usage, charging data generated by the NF consumer and sent to the CHF in a Charging Data Request message.

Figure 5.1.5.1-3: Message flow for new immediate triggers

1)	Service delivery ongoing: there is a connection setup for service delivery and a charging session is established between the NF (CTF) and the CHF.

2)	Trigger event: the NF (CTF) notices that a new immediate trigger with "zero data transmission without reporting" indicator is met. If there is zero usage, NF(CTF) will ignore the reporting for this trigger.

NOTE: which triggers are applicable for the immediate trigger "zero data transmission without reporting" should be specified in the normative work. If the immediate trigger without the "zero data transmission without reporting" indicator, the charging data message should be reported even zero usage for the immediate triggers.

3)	Uplink/downlink transmission: There are data or events transmission start after the immediate trigger "zero data transmission without reporting ", was met, and the NF (CTF) generates charging information related to the service delivered.

4) Trigger event: the NF (CTF) notices that new immediate triggers (e.g. Start of service data flow, in case no valid quota for this rating group) are met, the NF(CTF) generates charging data request message with the new immediate trigger and timestamp, the previous immediate trigger "zero data transmission without reporting" in step 2 is ignored.

5)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the CHF reporting the used units with the trigger and if the service is under quota management it includes a quota request for the service to continue.

6)	Account, rating, and reservation control: The CHF performs the process related to the reported usage and if the service is under quota management it performs processes related to quota requested, it may involve using the reported charging data to rate the usage and deduct the funds corresponding to the usage on the account balance..

7)	 Update CDR: based on policies, the CHF updates the CDR with charging data related to the service.

8)	Charging Data Response [Update, Quota granted]: The CHF responds and if the service is under quota management it includes a quota granted for the service to continue.

9)	Service delivery continuing service delivery is continuing and the charging session is continuing between the NF (CTF) and the CHF.

End of change