3GPP TSG-SA5 Meeting #150	S5-235452

# 21 - 25 August 2023, Goteborg, Sweden

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | - | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Input to Draft CR] Add MDA capability for resource utilization analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The use case and requirements for resource utilization analytics are captured in DraftCR S5-234846. This contribution is to add the analytics data definitions to support these use cases and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added analytics data definitions for resource utilization analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource utilization analysis cannot be supported by MDA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x (new), 8.5.x (new), 8.5.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

### 8.4.x	Resource related analytics

#### 8.4.x.1	NF resource utilization analysis

##### 8.4.x.1.1	Virtualized resource utilization analysis

###### 8.4.x.1.1.1	MDA type

The MDA type for virtualized resource utilization analysis is: ResourceAnalytics.virtualizedResourceUtilizationAnalysisNF.

###### 8.4.x.1.1.2	Enabling data

The enabling data for virtualized resource utilization analysis are provided in table 8.4.x.1.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.1.2-1: Enabling data for virtualized resource utilization analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | VR (including Virtual CPU, Virtual Memory, and Virtual Disk) usage of NF | VR usage of NF (clause 5.7.1 of TS 28.552 [4]) |
|  | Connection Point data volumes of NF | Connection data volumes of NF (clause 5.7.2 of TS 28.552 [4]) |
|  | N3 interface data volume | N3 interface related measurements (clause 5.4.1 of TS 28.552 [4]) |
|  | N4 interface session establishments | N4 session establishments (clause 5.4.3.1 of TS 28.552 [4]) |
|  | N6 interface link usage | N6 related measurements (clause 5.4.2 of TS 28.552 [4]) |
|  | N9 interface data volume | GTP Data Packets and volume on N9 interface (clause 5.4.4.2 of TS 28.552 [4]) |
|  | Number of PDU sessions | Number of PDU sessions (mean) (clause 5.3.1.1 of TS 28.552 [4]) |
|  | Number of QoS flows | Mean number of QoS flows (clause 5.3.2.1.7 of TS 28.552 [4]) |
|  | Radio resource utilization | Radio resource utilization (clause 5.1.1.2 of TS 28.552 [4]) |
|  | RRC connection number | RRC connection number (clause 5.1.1.4 of TS 28.552 [4]) |
|  | Mean number of PDU sessions in NR cell | Mean number of PDU sessions being allocated (clause 5.1.1.5.4 of TS 28.552 [4]) |
|  | Mean number of DRBs in NR cell | Mean number of DRBs being allocated ( clause 5.1.1.10.9 of TS 28.552 [4]) |
|  | QoS flow release in NR cell | QoS flow release (clause 5.1.1.13.1 of TS 28.552 [4]) |
|  | Number of Active UEs | Number of Active UEs (clause 5.1.1.23 of TS 28.552 [4]) |
|  | PDCP Data Volume | PDCP Data Volume (clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4]) |
| Configuration data | The NRMs of the analyzed NFs | The NRMs defined in TS 28.622 [19] and TS 28.541 [15]. |
| Network Data Analytics | Analysis data from the control plane produced by NWDAF including NF load, observed/predicted service experience, user plane performance, and slice load level analytics. | Analytics data from NWDAF in TS 23.288 [10] including e.g. NF load analytics (clause 6.5), observed/ predicted service experience related network data analytics (clause 6.4), analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate in clause 6.14), and Slice load level related network data analytics (clause 6.3). |

###### 8.4.x.1.1.3	Analytics output

The specific information elements of the analytics output for virtualized resource utilization analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.1.1.3-1.

Table 8.4.x.1.1.3-1: Analytics output for virtualized resource utilization analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| lowVRUsageNFs | The NFs with low virtualized resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| highVRUsageNFs | The NFs with high virtualized resource usage (see Note 1) during some time periods in the past. |  | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predictedVRUsageForNFs | The predicted virtualized resource usage for NFs during some time periods in the future. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs. The recommended action may be (but not limited to): - scale in a list of NFs; - scale out a list of NFs. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: it is up to the MDA MnS producer to decide the thresholds for low and high usage. |  |  |  |

##### 8.4.x.1.2	Physical resource utilization analysis

###### 8.4.x.1.2.1	MDA type

The MDA type for physcial resource utilization analysis is: ResourceAnalytics.PhyiscalResourceUtilizationAnalysisNF.

###### 8.4.x.1.2.2	Enabling data

The enabling data for physical resource utilization analysis are provided in table 8.4.x.1.2.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2.2-1: Enabling data for physical resource utilization analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Radio resource utilization | Radio resource utilization (clause 5.1.1.2 of TS 28.552 [4]) |
|  | RRC connection number | RRC connection number (clause 5.1.1.4 of TS 28.552 [4]) |
|  | Mean number of PDU sessions in NR cell | Mean number of PDU sessions being allocated (clause 5.1.1.5.4 of TS 28.552 [4]) |
|  | Mean number of DRBs in NR cell | Mean number of DRBs being allocated ( clause 5.1.1.10.9 of TS 28.552 [4]) |
|  | QoS flow release in NR cell | QoS flow release (clause 5.1.1.13.1 of TS 28.552 [4]) |
|  | Number of Active UEs | Number of Active UEs (clause 5.1.1.23 of TS 28.552 [4]) |
|  | PDCP Data Volume | PDCP Data Volume (clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4]) |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed RAN (NG-RAN and E-UTRAN). | The geographical information (longitude, latitude, altitude) information (see the peeParametersList attribute of the ManagedFunction IOC in TS 28.622 [19]). |
| Configuration data | The NRMs of the analyzed gNB-CUs, and gNB-DUs | The GNBCUCPFunction, GNBCUUPFunction and GNBDUFunction defined in TS 28.622 [19] and TS 28.541 [15]. |
| Network Data Analytics | Analysis data from the control plane produced by NWDAF including NF load, observed/predicted service experience, user plane performance, and slice load level analytics. | Analytics data from NWDAF in TS 23.288 [10] including e.g. NF load analytics (clause 6.5), observed/predicted service experience related network data analytics (clause 6.4), analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate in clause 6.14), and Slice load level related network data analytics (clause 6.3). |

###### 8.4.x.1.2.3	Analytics output

The specific information elements of the analytics output for physical resource utilization analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.1.2.3-1.

Table 8.4.x.1.2.3-1: Analytics output for physical resource utilization analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| lowPRUsageNFs | The NFs with low physical resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| highPRUsageNFs | The NFs with high physical resource usage (see Note 1) during some time periods in the past. |  | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predictedPRUsageForNFs | The predicted physical resource usage for NFs during some time periods in the future. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs. The recommended action may include (but not limited to) scaling out the gNB-CU, or increasing the capabity of gNB-DU. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: it is up to the MDA MnS producer to decide the thresholds for low and high usage. |  |  |  |

Next modified section

### 8.5.x	ResourceUsageNF <<dataType>>

#### 8.5.x.1	Definition

This data type specifies the type of resource usage for an NF.

#### 8.5.x.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| nFId | It provides the DN of the NF (which can be a 5GC or an NG-RAN NF). | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startTime | It indicates the start time of the time period. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| endTime | It indicates the end time of the time period. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| overallResourceUsage | It provides the statistical or predicted overall average usage (in percentage) of all of the resources. | M | type: Real multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| specificResourceUsage | It provides the statistical or predicted average usage of the specific type(s) of resources. | M | type: ResourceUsage multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

#### 8.5.x.3	Constraints

None.

Next modified section

### 8.5.y	ResourceUsage <<dataType>>

#### 8.5.y.1	Definition

This data type specifies the type of resource usage.

#### 8.5.y.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| resourceType | It indicates the type of resource. The allowed value is “VirtualCpu”, “VirtualMemory”, “VirtualDisk”, “DLPRBTotal”, “ULPRBTotal”, or a vendor-specific value. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| meanUsage | It provides the mean usage or predicted mean usage (in percentage) of the resource (indicated by the “resourceType” information element). | M | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

#### 8.5.y.3	Constraints

None.

End of modified sections