3GPP TSG-SA5 Meeting #151 	S5-237221

Xiamen, China, 9 - 13 October 2023															(rev of S5-236691)

# Source:	L.M. Ericsson Limited, Deutsche Telekom AG

# Title:	Discussion paper on slice agnostic service profile solutions

# Document for:	Agreement

Agenda Item:	6.5.2.2 (NSRULE)

# 1	Decision/action requested

The group is asked to agree the proposal in 4.

# 2	References

[1]	3GPP TS 28.541 "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[2]	3GPP TS 28.531 "Management and orchestration; Provisioning"

[3]	3GPP TS 28.312 "Management and orchestration; Intent driven management services for mobile networks"

# 3	Rationale

## 3.1	Introduction

A single instance of a ServiceProfile should describe the service requirements from a CSC to a 3GPP system. The CSP operating the 3GPP system decides how to fulfil the service requirements, the service requirements should therefore be network slice agnostic.

Currently some of the ServiceProfile attributes use network slice in the name and in the attribute description while the service requirements should be agnostic on how they will be fulfilled. A network slice agnostic service profile could also be used

The re-use of serviceProfile is limited to network slice. In the discussion on sharing of network slices there has to be a clear separation between service (described by service profile) and the network slice (described by the network slice), using a mixed terminology is confusing.

The following attributes in the ServiceProfile use slice in the name or hint that the solution provided to fulfil the requirements based on network slice;

- networkSliceSharingIndicator,

- sST,

- dLThptPerSlice,

- uLThptPerSlice,

- sliceSimultaneousUse,

- nssaaSupport.

## 3.2	Different approaches

The following approaches are discusssed:

Approach 1: attributes with slice in the name are properties of a network slice,

Approach 2a: attributes with slice in the name are service requirements and should be renamed accordingly,

Approach 2b: attributes with slice in the name are service requirements and added with new names,

Approach 3: each attribute is individually assessed as either being a network slice property or service requirement

Approach 1

Since the attributes are applicable to network slice, an attribute being a setting (configuration), a function or a characteristics requirement the information is on network slice level, therefore the attributes that have slice in the name are property of a NetworkSlice. The attributes are removed from the ServiceProfile, added to the NetworkSlice as shown in the tabel below.

| Current attribute name | Attribute name as NetworkSlice property |
| --- | --- |
| networkSliceSharingIndicator | networkSliceSharingIndicator |
| sST | sST |
| dLThptPerSlice | dLThptPerSlice |
| uLThptPerSlice | uLThptPerSlice |
| sliceSimultaneousUse | sliceSimultaneousUse |
| nssaaSupport | nssaaSupport |

Approach 2

Since the attributes are service profile attributes, the name should reflect this therefore they should be renamed to be slice agnostic. The proposed names are shown in the tabel below.

| Current attribute name | Slice agnostic attribute name |
| --- | --- |
| networkSliceSharingIndicator | sharingIndicator |
| sST | serviceType |
| dLThptPerSlice | dLThpt_requirement |
| uLThptPerSlice | uLThpt_requirement |
| sliceSimultaneousUse | simultaneousUse |
| nssaaSupport | saaSupport_requirement |

Approach 3

Since the attributes are service profile attributes, the name should reflect this therefore they should be renamed to be slice agnostic. In this approach the current attributes are not replaced, and the new attributes are added with slice agnostic name, see table below.

| Current attribute name | Proposed attribute names |
| --- | --- |
| networkSliceSharingIndicator | networkSliceSharingIndicator |
| sST | sST |
| dLThptPerSlice | dLThptPerSlice |
| uLThptPerSlice | uLThptPerSlice |
| sliceSimultaneousUse | sliceSimultaneousUse |
| nssaaSupport | nssaaSupport |
|  | sharingIndicator |
|  | serviceType |
|  | dLThpt_requirement |
|  | uLThpt_requirement |
|  | simultaneousUse |
|  | saaSupport_requirement |

Approach 4

Each attribute is individually assessed as either being a service requirement or network slice property. An exampel is shown in the tabel below.

| Current attribute name | Slice agnostic attribute name | Attribute name as NetworkSlice property |
| --- | --- | --- |
| networkSliceSharingIndicator | sharingIndicator |  |
| sST | serviceType |  |
| dLThptPerSlice | dLThpt_requirement |  |
| uLThptPerSlice | uLThpt_requirement |  |
| sliceSimultaneousUse | simultaneousUse |  |
| nssaaSupport |  | nssaaSupport |

## 3.3	Recommended approach

Approach 3 is recommended as this solution is backward compatible unlike the other approaches.

# 4	Detailed proposal

The group is asked to endorse the recommendation in section 3.3 in the rational.

# 5	Discussion (Xiamen, 10-12)

Agenda:  Discuss new action item “evaluate the potential change to make SP slice agnostic in R-18 (6691)”

- Group agreed to target to evaluate the potential change to make SP slice agnostic in R-18.

Comments from chairnotes (discussion during meeting earlier in the week):

S: whether SP is slice agnostic or not? S support no change.

SP be slice agnostic?

Support:

E, TEF

Not Support:

N: relation with isolation?

TEF: approach 3 is bkwd compatible, but the soltuion needs more discussion.

Breakout session Friday morning (Oct-12):

- Does SA5 agree there is a need to generalize a Service Profile?   If yes, is the current NS SP the start point?

  - Agree (at least no objections to) a generalized service mgmt. solution

  - Agree (at least no objections to) a generalized service mgmt. reqs going forward such that the same Reqs are applicable to multiple service mgt use cases

- If yes, timing R18 vs R19:

  - R18 is very near conclusion.  Is it realistic to make any changes now?

    - Keep R18 ambition level low

    - Is it worth modifying any attributes now (noting the R19 work on srv. Mgmt.)

      - N: prefer to define genralized solution for Svc. Mgmt. reqs/profile handling then address slice impacts (as migration to new solution)

  - R18 limits to consider:

    - No behaviour change

    - No modelling/mapping of the profile reqs to NRM

    - Could consider removing the slice specificity from the current attribute naems/defs (i.e. this DP focus)

    - No WI for this (not in current scope of NSRULE)

- Details for further consideration (R19, Gen. svc. Mgmt.):

  - Generalized solutions lose their 1:1 associations to the use cases and reqs in the sercices themselves (e.g. nssaa vs saa for auth.; how would generalized parms be associated?  Is this a drawback to a generalized solution vs. use case specific?)

    - Noting similar concerns have been raised for Generic ICM (intent model) where IOC/DT/attibutes are generic and optional.  The use cases must clearly define if/how these are used specifically.

    - There are other options available to handle this – new IOC which inherits from existing solution?

  - Relationship to ng116?

    - GSMA thinks in terms of network slice, and current solution caters/aligns to this.  If changed how to retain the handling/relationship?

    - There is no req (from gsma) that SA5 must align.  It was SA5 decision.

  - Is there an existing R19 WI that cover this?

    - Svc. Exposure (likely best fit) – take back to broader SA5 audience