3GPP TSG-SA5 Meeting #151 	S5-236402

# Xiamen, China, 9-13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0050 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on performance assurance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ECM |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Several errors found for performance assurance in clause 8.2, 1, PerfMetricJob IOC is not defined in TS 28.531, but in TS 28.622; 2, According to the performace assurance procedure described in clause 7.2, establishStreamingConnection, notifyFileReady notification and reportStreamData operation can also be used for create PerfMetricJob IOC scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct several errors for performance assurance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May lead to implementation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

8.2	Performance Assurance

8.2.1	EAS performance assurance

8.2.1.1	MnS component type A

Table 8.2.1.1-1: EAS performance assurance type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1, 11.5 and 11.6 of TS 28.532 [5]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - establishStreamingConnection operation - notifyFileReady notification - reportStreamData operation | It is supported by using Provisioning MnS to manage PerfMetricJob IOC, as defined in TS 28.622 [4]. |
| Operations defined in clause 11.5 and 11.6 in TS 28.532 [3] and clause 6.1 of TS 28.550 [8]: - createMeasurementJob - stopMeasurementJob - listMeasurementJobs - establishStreamingConnection - notifyFileReady notification - reportStreamData operation | It is supported by using Measurement job control services for EAS, as defined in TS 28.550 [8]. |

8.2.1.2	MnS Component Type C definition

Performance measurements related EAS are captured in Table 8.2.1.2.-1:

Table 8.2.1.2-1. EAS related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Mean virtual CPU usage | Includes the mean usage of the underlying virtualized CPUs for a virtualized 3GPP NF (see clause 5.7.1.1.1 in TS 28.552 [10]). |  |
| Mean virtual memory usage | Includes the mean usage of the underlying virtualized memories for a virtualized 3GPP NF (see clause 5.7.1.2.1 in TS 28.552 [10]). |  |
| Mean virtual disk usage | Includes the mean usage of the underlying virtualized disks for a virtualized 3GPP NF (see clause 5.7.1.3.1 in TS 28.552 [10]). |  |
| Data volume of incoming bytes to EAS | Includes the number of incoming bytes received by the EAS (see clause 5.7.2.1 in TS 28.552 [10]). |  |
| Data volume of outgoing bytes to EAS | Includes the number of outgoing bytes received by the EAS (see clause 5.7.2.2 in TS 28.552 [10]). |  |
| Data volume of incoming packets to EAS | Includes the number of incoming packets received by the EAS (see clause 5.7.2.3 in TS 28.552 [10]). |  |
| Data volume of outgoing packets to EAS | Includes the number of outgoing packets received by the EAS (see clause 5.7.2.4 in TS 28.552 [10]). |  |

8.2.2	ECS performance assurance

8.2.2.1	MnS component type A

Table 8.2.2.1-1: ECS performance assurance type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1, 11.5 and 11.6 of TS 28.532 [5]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - establishStreamingConnection operation - notifyFileReady notification - reportStreamData operation | It is supported by using Provisioning MnS to manage PerfMetricJob IOC, as defined in TS 28.622 [4]. |
| Operations defined in clause 11.5 and 11.6 in TS 28.532 [3] and clause 6.1 of TS 28.550 [8]: - createMeasurementJob - stopMeasurementJob - listMeasurementJobs - establishStreamingConnection - notifyFileReady notification - reportStreamData operation | It is supported by using Measurement job control services for ECS, as defined in TS 28.550 [8]. |

8.2.2.2	MnS Component Type C definition

Performance measurements related ECS are captured in Table 8.2.1.2.-1:

Table 8.2.2.2-1. ECS related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Mean virtual CPU usage | Includes the mean usage of the underlying virtualized CPUs for a virtualized 3GPP NF (see clause 5.7.1.1.1 in TS 28.552 [10]). |  |
| Mean virtual memory usage | Includes the mean usage of the underlying virtualized memories for a virtualized 3GPP NF (see clause 5.7.1.2.1 in TS 28.552 [10]). |  |
| Mean virtual disk usage | Includes the mean usage of the underlying virtualized disks for a virtualized 3GPP NF (see clause 5.7.1.3.1 in TS 28.552 [10]). |  |
| EES Registration | Includes the total, mean and successful number of EES Registration request processed by ECS (see clause 5.X.1 in TS 28.552 [10]). |  |
| Service Provisioning | Includes the total, mean and successful number of service provisioning request processed by ECS (see clause 5.X.1 in TS 28.552 [10]). |  |

8.2.3	EES performance assurance

8.2.3.1	MnS component type A

Table 8.2.3.1-1: EES performance assurance type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1, 11.5 and 11.6 of TS 28.532 [5]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - establishStreamingConnection operation - notifyFileReady notification - reportStreamData operation | It is supported by using Provisioning MnS to manage PerfMetricJob IOC, as defined in TS 28.622 [4]. |
| Operations defined in clause 11.5 and 11.6 in TS 28.532 [3] and clause 6.1 of TS 28.550 [8]: - createMeasurementJob - stopMeasurementJob - listMeasurementJobs - establishStreamingConnection - notifyFileReady notification - reportStreamData operation | It is supported by using Measurement job control services for EES, as defined in TS 28.550 [8]. |

8.2.3.2	MnS Component Type C definition

Performance measurements related EES are captured in Table 8.2.3.2.-1:

Table 8.2.3.2-1. EES related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Mean virtual CPU usage | Includes the mean usage of the underlying virtualized CPUs for a virtualized 3GPP NF (see clause 5.7.1.1.1 in TS 28.552 [10]). |  |
| Mean virtual memory usage | Includes the mean usage of the underlying virtualized memories for a virtualized 3GPP NF (see clause 5.7.1.2.1 in TS 28.552 [10]). |  |
| Mean virtual disk usage | Includes the mean usage of the underlying virtualized disks for a virtualized 3GPP NF (see clause 5.7.1.3.1 in TS 28.552 [10]). |  |
| EAS Registration | Includes the total, mean and successful number of EAS Registration request processed by ECS (see clause 5.X.1 in TS 28.552 [10]). |  |
| EAS Discovery | Includes the total, mean and successful number of EAS discovery request processed by ECS (see clause 5.X.1 in TS 28.552 [10]). |  |
| EEC Registration | Includes the total, mean and successful number of EEC Registration request processed by ECS (see clause 5.X.1 in TS 28.552 [10]). |  |

End of Change