3GPP TSG-SA5 Meeting #151	S5-236246

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1022 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.541 Rel18 NRM enhancements for class diagram |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_ph2 |  |  |  |  |  | Date: |  |  | 2023-09-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support the newly enhanced Core NF, the clause 5.2.1.1 shall be updated for example the name containment for MBSMFFunction, MBUPFFunction, and newly introduced EP_Nxx, inheritance diagram. Not like the English text, which can be merged word by word, the binary form of digram can not. Hence a separate CR is provided to provide a complete diagram for MCC to merge if approved. Additionally, old version of Figure 5.2.1.1-1 is not deleted |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the name containment for MBSMFFunction, MBUPFFunction and transport view - Add inheritance diagram for MBSMFFunction, MBUPFFunction and all newly introduced EP_Nxx. - Old version of The Figure 5.2.1.1-1 deleted Add the plantUML code for some complex diagrams As Informative Annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification leads to wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1, 5.1.1.2, Annex Q |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.541 CR1020, CR1021 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 5.2.1	Class diagram of 5GC NFs

#### 5.2.1.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of 5GC NFs definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The Figure 5.2.1.1-1 shows the 5GC NF NRM containment/naming relationship.

Figure 5.2.1.1-1: 5GC NRM containment/naming relationship

The Figure 5.2.1.1-2 shows the transport view of AMF NRM.

Figure 5.2.1.1-2: Transport view of AMF NRM

The Figure 5.2.1.1-3 shows the transport view of SMF NRM.

Figure 5.2.1.1-3: Transport view of SMF NRM

The Figure 5.2.1.1-4 shows the transport view of UPF NRM.

Figure 5.2.1.1-4: Transport view of UPF NRM

The Figure 5.2.1.1-5 shows the transport view of N3IWF NRM.

Figure 5.2.1.1-5: Transport view of N3IWF NRM

The Figure 5.2.1.1-6 shows the transport view of PCF NRM.

Figure 5.2.1.1-6: Transport view of PCF NRM

The Figure 5.2.1.1-7 shows the transport view of AUSF NRM.

Figure 5.2.1.1-7: Transport view of AUSF NRM

The Figure 5.2.1.1-8 shows the transport view of UDM NRM.

Figure 5.2.1.1-8: Transport view of UDM NRM

The Figure 5.2.1.1-9 shows the transport view of NRF NRM.

Figure 5.2.1.1-9: Transport view of NRF NRM

The Figure 5.2.1.1-10 shows the transport view of NSSF NRM.

Figure 5.2.1.1-10: Transport view of NSSF NRM

The Figure 5.2.1.1-11 shows the transport view of SMSF NRM.

Figure 5.2.1.1-11: Transport view of SMSF NRM

The Figure 5.2.1.1-12 shows the transport view of 5G location service related NRM.

Figure 5.2.1.1-12: Transport view of LMF NRM

The Figure 5.2.1.1-13 shows the transport view of 5G-EIR NRM.

Figure 5.2.1.1-13: Transport view of 5G-EIR NRM

The Figure 5.2.1.1-14 shows the transport view of SEPP NRM.

Figure 5.2.1.1-14: Transport view of SEPP NRM

The Figure 5.2.1.1-15 shows the NRM fragment for control of QoS monitoring per QoS flow per UE.

Figure 5.2.1.1-15: NRM fragment for control of QoS monitoring per QoS flow per UE

The Figure 5.2.1.1-16 shows the NRM fragment for control of GTP-U path QoS monitoring.

Figure 5.2.1.1-16: NRM fragment for control of GTP-U path QoS monitoring

The Figure 5.2.1.1-17 shows the NRM fragment for pre-configured 5QIs in 5GC.

Figure 5.2.1.1-17: NRM fragment for pre-configured 5QIs in 5GC

The Figure 5.2.1.1-18 shows the NRM fragment for 5QI and DSCP mapping.

Figure 5.2.1.1-18: NRM fragment for 5QI and DSCP mapping.

The Figure 5.2.1.1-19 shows the NRM fragment for predefined PCC rule.

Figure 5.2.1.1-19: NRM fragment for predefined PCC rule

The Figure 5.2.1.1-20 shows the NRM fragment for dynamically assigned 5QIs in 5GC.

Figure 5.2.1.1-20: NRM fragment for dynamically assigned 5QIs in 5GC

The Figure 5.2.1.1-21 shows the transport view of  NSACF NRM.

Figure 5.2.1.1-21: Transport view of NSACF NRM

Figure 5.2.1.1-22: Transport view of NEF NRM

The Figure 5.2.1.1-23 shows the transport view of 5G DDNMF NRM.

Figure 5.2.1.1-23: Transport view of 5G DDNMF NRM

The Figure 5.2.1.1-24 shows the transport view of 5G EASDF NRM.

Figure 5.2.1.1-24: Transport view of 5G EASDF NRM

Figure 5.2.1.1-25: EcmConnectionInfo NRM

Figure 5.2.1.1-26: AFFunction NRM

Figure 5.2.1.1-27: NSSAAFFunction NRM

The Figure 5.2.1.1-28 shows the transport view of CHFFunction NRM:

Figure 5.2.1.1-28: Transport view of CHFFunction NRM

The Figure 5.2.1.1-29 shows the transport view of AANFFunction NRM:

Figure 5.2.1.1-29: Transport view of AANFFunction NRM fragment

The Figure 5.2.1.1-30 shows the transport view of GMLCFunction NRM:

Figure 5.2.1.1-30: Transport view of GMLCFunction NRM fragment

The Figure 5.2.1.1-31 shows the transport view of NWDAFFunction NRM:

Figure 5.2.1.1-31: Transport view of NWDAFFunction NRM fragment

The Figure 5.2.1.1-32 shows the transport view of TSCTSFFunction NRM:

Figure 5.2.1.1-32: Transport view of TSCTSFFunction NRM fragment

The Figure 5.2.1.1-a shows the transport view of TSCTSFFunction NRM:

Figure 5.2.1.1-a: Transport view of MB-SMF Function NRM fragment

The Figure 5.2.1.1-b shows the transport view of TSCTSFFunction NRM:

Figure 5.2.1.1-b: Transport view of MB-UPF Function NRM fragment

#### 5.2.1.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 5.2.1.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM.

Figure 5.2.1.2-1: Inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM

Figure 5.2.1.2-2 shows the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM.

Figure 5.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to the 5GC NF NRM

Figure 5.2.1.2-3: Inheritance hierarchy for IOC QFQoSMonitoringControl

Figure 5.2.1.2-4: Inheritance hierarchy for IOC GtpUPathQoSMonitoringControl

Figure 5.2.1.2-5: Inheritance hierarchy for IOC Configurable5QISet

Figure 5.2.1.2-6: Inheritance hierarchy for IOC FiveQiDscpMapping

Figure 5.2.1.2-7: Inheritance hierarchy for predefined PCC rule modeling

Figure 5.2.1.2-8: Inheritance hierarchy for IOC Dynamic5QISet

Figure 5.2.1.2-9: Inheritance hierarchy for EcmConnectionInfo

Figure 5.2.1.2-10: FiveQICharacteristics Inheritance

2nd Modified Section

Annex Q (informative):
PlantUML source code

# Q.1	5GC NRM relationship diagram

## Q.1.1	the 5GC NF NRM containment/naming relationship

@startuml Figure 5.2.1.1-1: 5GC NF NRM containment/naming relationship

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

left to right direction

skinparam ranksep 100

skinparam nodesep 4

class ManagedElement <<InformationObjectClass>>

class LMFFunction <<InformationObjectClass>>

class NGEIRFunction <<InformationObjectClass>>

class NSSFFunction <<InformationObjectClass>>

class SEPPFFunction <<InformationObjectClass>>

class SCPFFunction <<InformationObjectClass>>

class SMSFFunction <<InformationObjectClass>>

class UDRFunction <<InformationObjectClass>>

class N3IWFFunction <<InformationObjectClass>>

class "5G DDNMFFunction" <<InformationObjectClass>>

class NSSAAFFunction  <<InformationObjectClass>>

class MFAFFunction <<InformationObjectClass>>

class NSACFFunction <<InformationObjectClass>>

class GMLCFunction <<InformationObjectClass>>

class TSCTSFFunction <<InformationObjectClass>>

class MBUPFFunction <<InformationObjectClass>>

class AMFFunction <<InformationObjectClass>>

class SMFFunction <<InformationObjectClass>>

class UPFFunction <<InformationObjectClass>>

class PCFFunction <<InformationObjectClass>>

class NEFFunction <<InformationObjectClass>>

class NRFFunction <<InformationObjectClass>>

class UDMFunction <<InformationObjectClass>>

class AUSFFunction <<InformationObjectClass>>

class AFFunction <<InformationObjectClass>>

class EASDFFunction <<InformationObjectClass>>

class CHFFFunction <<InformationObjectClass>>

class DCCFFunction <<InformationObjectClass>>

class NWDAFFunction <<InformationObjectClass>>

class AANFFunction <<InformationObjectClass>>

class BSFFunction <<InformationObjectClass>>

class MBSMFFunction <<InformationObjectClass>>

LMFFunction        "*" --* "1" ManagedElement: <<names>>

NGEIRFunction      "*" --* "1" ManagedElement: <<names>>

NSSFFunction       "*" --* "1" ManagedElement: <<names>>

SEPPFFunction      "*" --* "1" ManagedElement: <<names>>

SCPFFunction       "*" --* "1" ManagedElement: <<names>>

SMSFFunction       "*" --* "1" ManagedElement: <<names>>

UDRFunction        "*" --* "1" ManagedElement: <<names>>

N3IWFFunction      "*" --* "1" ManagedElement: <<names>>

"5G DDNMFFunction" "*" --* "1" ManagedElement: <<names>>

NSSAAFFunction     "*" --* "1" ManagedElement: <<names>>

MFAFFunction       "*" --* "1" ManagedElement: <<names>>

NSACFFunction      "*" --* "1" ManagedElement: <<names>>

GMLCFunction       "*" --* "1" ManagedElement: <<names>>

TSCTSFFunction     "*" --* "1" ManagedElement: <<names>>

MBUPFFunction      "*" --* "1" ManagedElement: <<names>>

ManagedElement "1" *-- "*" AMFFunction  : <<names>>

ManagedElement "1" *-- "*" SMFFunction  : <<names>>

ManagedElement "1" *-- "*" UPFFunction  : <<names>>

ManagedElement "1" *-- "*" PCFFunction  : <<names>>

ManagedElement "1" *-- "*" NEFFunction  : <<names>>

ManagedElement "1" *-- "*" NRFFunction  : <<names>>

ManagedElement "1" *-- "*" UDMFunction  : <<names>>

ManagedElement "1" *-- "*" AUSFFunction : <<names>>

ManagedElement "1" *-- "*" AFFunction   : <<names>>

ManagedElement "1" *-- "*" EASDFFunction: <<names>>

ManagedElement "1" *-- "*" CHFFFunction : <<names>>

ManagedElement "1" *-- "*" DCCFFunction : <<names>>

ManagedElement "1" *-- "*" NWDAFFunction : <<names>>

ManagedElement "1" *-- "*" AANFFunction : <<names>>

ManagedElement "1" *-- "*" BSFFunction  : <<names>>

ManagedElement "1" *-- "*" MBSMFFunction: <<names>>

class SubNetwork <<InformationObjectClass>>

class ExternalNRFFunction <<InformationObjectClass>>

class ExternalNSSFFunction <<InformationObjectClass>>

class ExternalAMFFunction <<InformationObjectClass>>

ExternalNRFFunction      "*" -u-* "1" SubNetwork: <<names>>

ExternalNSSFFunction     "*" -u-* "1" SubNetwork: <<names>>

SubNetwork "1" *-d- "*" ExternalAMFFunction  : <<names>>

SubNetwork "1" *-- "*" ManagedElement  : <<names>>

@enduml

## Q.1.a the Figure 5.2.1.1-2 for the transport view of AMF NRM

'Figure 5.2.1.1-2: Transport view of AMF NRM

@startuml Figure 5.2.1.1-2: Transport view of AMF NRM

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

skinparam linetype ortho

skinparam nodesep 2

skinparam ranksep 100

left to right direction

class AMFFunction <<InformationObjectClass>>

class EP_N2 <<InformationObjectClass>>

class EP_N8 <<InformationObjectClass>>

class EP_N11 <<InformationObjectClass>>

class EP_N12 <<InformationObjectClass>>

class EP_N13 <<InformationObjectClass>>

class EP_N15 <<InformationObjectClass>>

class EP_N17 <<InformationObjectClass>>

class EP_N22 <<InformationObjectClass>>

class EP_N26 <<InformationObjectClass>>

class EP_N20 <<InformationObjectClass>>

class EP_NLS <<InformationObjectClass>>

class EP_NL2 <<InformationObjectClass>>

class EP_N60 <<InformationObjectClass>>

class EP_N41 <<InformationObjectClass>>

class EP_N42 <<InformationObjectClass>>

class EP_N89 <<InformationObjectClass>>

class EP_N11mb <<InformationObjectClass>>

class FarEnd_N2 <<ProxyClass>>

class UDMFunction <<InformationObjectClass>>

class SMFFunction <<InformationObjectClass>>

class AUSFFunction <<InformationObjectClass>>

class "AMFFunction " <<InformationObjectClass>>

class PCFFunction <<InformationObjectClass>>

class NGEIRFunction <<InformationObjectClass>>

class NSSFFunction <<InformationObjectClass>>

class MMEFunction <<InformationObjectClass>>

class SMSFFunction <<InformationObjectClass>>

class LMFFunction <<InformationObjectClass>>

class GMLCFunction <<InformationObjectClass>>

class NSACFFunction <<InformationObjectClass>>

class "CHFFunction (HPLMN)" <<InformationObjectClass>>

class "CHFFunction (VPLMN)" <<InformationObjectClass>>

class TSCTSFFunction <<InformationObjectClass>>

class MBSMFFunction <<InformationObjectClass>>

AMFFunction "1" *-- "*" EP_N2: <<names>>

AMFFunction "1" *-- "*" EP_N8: <<names>>

AMFFunction "1" *-- "*" EP_N11: <<names>>

AMFFunction "1" *-- "*" EP_N12: <<names>>

AMFFunction "1" *-- "*" EP_N13: <<names>>

AMFFunction "1" *-- "*" EP_N15: <<names>>

AMFFunction "1" *-- "*" EP_N17: <<names>>

AMFFunction "1" *-- "*" EP_N22: <<names>>

AMFFunction "1" *-- "*" EP_N26: <<names>>

AMFFunction "1" *-- "*" EP_N20: <<names>>

AMFFunction "1" *-- "*" EP_NLS: <<names>>

AMFFunction "1" *-- "*" EP_NL2: <<names>>

AMFFunction "1" *-- "*" EP_N60: <<names>>

AMFFunction "1" *-- "*" EP_N41: <<names>>

AMFFunction "1" *-- "*" EP_N42: <<names>>

AMFFunction "1" *-- "*" EP_N89: <<names>>

AMFFunction "1" *-- "*" EP_N11mb: <<names>>

EP_N2  "1" --> "1" FarEnd_N2

EP_N8  "1" --> "1" UDMFunction

EP_N11 "1" --> "1" SMFFunction

EP_N12 "1" --> "1" AUSFFunction

EP_N13 "1" --> "1" "AMFFunction "

EP_N15 "1" --> "1" PCFFunction

EP_N17 "1" --> "1" NGEIRFunction

EP_N22 "1" --> "1" NSSFFunction

EP_N26 "1" --> "1" MMEFunction

EP_N20 "1" --> "1" SMSFFunction

EP_NLS "1" --> "1" LMFFunction

EP_NL2 "1" --> "1" GMLCFunction

EP_N60 "1" --> "1" NSACFFunction

EP_N41 "1" --> "1" "CHFFunction (HPLMN)"

EP_N42 "1" --> "1" "CHFFunction (VPLMN)"

EP_N89 "1" --> "1" TSCTSFFunction

EP_N11mb "1" --> "1" MBSMFFunction

note top of FarEnd_N2: "This proxy class represents\n GNBFunction or \n GNBCUCPFunction (TS\n 28.541) or ENBFunction (TS\n 28.658) or N3IWFFunction"

@enduml

## Q.1.b the Figure 5.2.1.1-3 for the transport view of SMF NRM

@startuml Figure 5.2.1.1-3: Transport view of SMF NRM

hide empty members

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam ClassStereotypeFontStyle normal

skinparam nodesep 3

skinparam ranksep 100

skinparam linetype ortho

left to right direction

class SMFFunction <<InformationObjectClass>>

class EP_NS5C <<InformationObjectClass>>

class EP_N4 <<InformationObjectClass>>

class EP_N7 <<InformationObjectClass>>

class EP_N10 <<InformationObjectClass>>

class EP_N11 <<InformationObjectClass>>

class EP_N16 <<InformationObjectClass>>

class EP_N40 <<InformationObjectClass>>

class EP_N88 <<InformationObjectClass>>

class EP_N16mb <<InformationObjectClass>>

class FarEnd_S5C <<ProxyClass>>

class UPFFunction <<InformationObjectClass>>

class UDMFunction <<InformationObjectClass>>

class AMFFunction <<InformationObjectClass>>

class PCFFunction <<InformationObjectClass>>

class "SMFFunction " <<InformationObjectClass>>

class CHFFunction <<InformationObjectClass>>

class EASDFFunction <<InformationObjectClass>>

class MBSMFFunction <<InformationObjectClass>>

SMFFunction "1" *-- "*" EP_NS5C : <<names>>

SMFFunction "1" *-- "*" EP_N4   : <<names>>

SMFFunction "1" *-- "*" EP_N7   : <<names>>

SMFFunction "1" *-- "*" EP_N10  : <<names>>

SMFFunction "1" *-- "*" EP_N11  : <<names>>

SMFFunction "1" *-- "*" EP_N16  : <<names>>

SMFFunction "1" *-- "*" EP_N40  : <<names>>

SMFFunction "1" *-- "*" EP_N88  : <<names>>

SMFFunction "1" *-- "*" EP_N16mb  : <<names>>

EP_NS5C  "1" --> "1" FarEnd_S5C

EP_N4    "1" --> "1" UPFFunction

EP_N7    "1" --> "1" PCFFunction

EP_N10   "1" --> "1" UDMFunction

EP_N11   "1" --> "1" AMFFunction

EP_N16   "1" --> "1" "SMFFunction "

EP_N40   "1" --> "1" CHFFunction

EP_N88   "1" --> "1" EASDFFunction

EP_N16mb   "1" --> "1" MBSMFFunction

note top of FarEnd_S5C: "It represents ServingGWFunction\(from TS 28.708)"

@enduml

# Q.2	5GC NRM inheritance diagram

## Q.2.1	the inheritance hierarchy from IOC ManagedFunction related to the 5GC NF NRM

@startuml Figure 5.2.1.2-1: Inheritance hierarchy from IOC ManagedFunction

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

left to right direction

skinparam linetype ortho

skinparam nodesep 2

class ManagedFunction <<InformationObjectClass>>

class ExternalNRFFunction <<InformationObjectClass>>

class ExternalNSSFFunction <<InformationObjectClass>>

class LMFFunction <<InformationObjectClass>>

class NGEIRFunction <<InformationObjectClass>>

class NSSFFunction <<InformationObjectClass>>

class SCPFFunction <<InformationObjectClass>>

class SEPPFFunction <<InformationObjectClass>>

class SMSFFunction <<InformationObjectClass>>

class UDRFunction <<InformationObjectClass>>

class N3IWFFunction <<InformationObjectClass>>

class NSACFFunction <<InformationObjectClass>>

class EASDFFunction <<InformationObjectClass>>

class ExternalAMFFunction <<InformationObjectClass>>

class AMFFunction <<InformationObjectClass>>

class SMFFunction <<InformationObjectClass>>

class UPFFunction <<InformationObjectClass>>

class PCFFunction <<InformationObjectClass>>

class NEFFunction <<InformationObjectClass>>

class NRFFunction <<InformationObjectClass>>

class UDMFunction <<InformationObjectClass>>

class AUSFFunction <<InformationObjectClass>>

class "5G DDNMFFunction" <<InformationObjectClass>>

class AFFunction <<InformationObjectClass>>

class NWDAFFunction <<InformationObjectClass>>

class NSSAAFFunction  <<InformationObjectClass>>

class CHFFFunction <<InformationObjectClass>>

class DCCFFunction <<InformationObjectClass>>

class MFAFFunction <<InformationObjectClass>>

class AANFFunction <<InformationObjectClass>>

class BSFFunction <<InformationObjectClass>>

class GMLCFunction <<InformationObjectClass>>

class TSCTSFFunction <<InformationObjectClass>>

class MBUPFFunction <<InformationObjectClass>>

class MBSMFFunction <<InformationObjectClass>>

ExternalNSSFFunction --|> ManagedFunction

LMFFunction          --|> ManagedFunction

ExternalNRFFunction  --|> ManagedFunction

NGEIRFunction        --|> ManagedFunction

NSSFFunction         --|> ManagedFunction

SCPFFunction         --|> ManagedFunction

SEPPFFunction        --|> ManagedFunction

SMSFFunction         --|> ManagedFunction

UDRFunction          --|> ManagedFunction

N3IWFFunction        --|> ManagedFunction

NSACFFunction        --|> ManagedFunction

EASDFFunction        --|> ManagedFunction

NSSAAFFunction       --|> ManagedFunction

MFAFFunction         --|> ManagedFunction

AANFFunction         --|> ManagedFunction

BSFFunction          --|> ManagedFunction

MBUPFFunction        --|> ManagedFunction

ManagedFunction <|-- AMFFunction

ManagedFunction <|-- SMFFunction

ManagedFunction <|-- ExternalAMFFunction

ManagedFunction <|-- UPFFunction

ManagedFunction <|-- PCFFunction

ManagedFunction <|-- NEFFunction

ManagedFunction <|-- NRFFunction

ManagedFunction <|-- UDMFunction

ManagedFunction <|-- AUSFFunction

ManagedFunction <|-- "5G DDNMFFunction"

ManagedFunction <|-- AFFunction

ManagedFunction <|-- NWDAFFunction

ManagedFunction <|-- CHFFFunction

ManagedFunction <|-- DCCFFunction

ManagedFunction <|-- GMLCFunction

ManagedFunction <|-- TSCTSFFunction

ManagedFunction <|-- MBSMFFunction

@enduml

## Q.2.a	Figure 5.2.1.2-2 the inheritance hierarchy from IOC EP_RP related to 5GC NF NRM

@startuml igure 5.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to the 5GC NF NRM

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

left to right direction

skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class EP_RP <<InformationObjectClass>>

class EP_N2 <<InformationObjectClass>>

class EP_N3 <<InformationObjectClass>>

class EP_N4 <<InformationObjectClass>>

class EP_N5 <<InformationObjectClass>>

class EP_N6 <<InformationObjectClass>>

class EP_N7 <<InformationObjectClass>>

class EP_N8 <<InformationObjectClass>>

class EP_N9 <<InformationObjectClass>>

class EP_N10 <<InformationObjectClass>>

class EP_N11 <<InformationObjectClass>>

class EP_N12 <<InformationObjectClass>>

class EP_N13 <<InformationObjectClass>>

class EP_N14 <<InformationObjectClass>>

class EP_N15 <<InformationObjectClass>>

class EP_N16 <<InformationObjectClass>>

class EP_N17 <<InformationObjectClass>>

'class EP_N18 <<InformationObjectClass>>

class EP_N22 <<InformationObjectClass>>

class EP_N26 <<InformationObjectClass>>

class EP_N27 <<InformationObjectClass>>

class EP_N28 <<InformationObjectClass>>

class EP_N31 <<InformationObjectClass>>

class EP_N32 <<InformationObjectClass>>

class EP_N33 <<InformationObjectClass>>

class EP_N40 <<InformationObjectClass>>

class EP_N41 <<InformationObjectClass>>

class EP_N42 <<InformationObjectClass>>

class EP_N58 <<InformationObjectClass>>

class EP_N60 <<InformationObjectClass>>

class EP_Npc4 <<InformationObjectClass>>

class EP_Npc6 <<InformationObjectClass>>

class EP_Npc7 <<InformationObjectClass>>

class EP_Npc8 <<InformationObjectClass>>

class EP_N59 <<InformationObjectClass>>

class EP_N88 <<InformationObjectClass>>

class EP_NLS <<InformationObjectClass>>

class EP_NL2 <<InformationObjectClass>>

class EP_N20 <<InformationObjectClass>>

class EP_N22 <<InformationObjectClass>>

' AANFFunction

class EP_N61 <<InformationObjectClass>>

class EP_N62 <<InformationObjectClass>>

class EP_N63 <<InformationObjectClass>>

' GMLC

class EP_NL3 <<InformationObjectClass>>

class EP_NL6 <<InformationObjectClass>>

class EP_NL5 <<InformationObjectClass>>

class EP_NL9 <<InformationObjectClass>>

' TSCTSFFunction

class EP_N86 <<InformationObjectClass>>

class EP_N84 <<InformationObjectClass>>

class EP_N85 <<InformationObjectClass>>

class EP_N87 <<InformationObjectClass>>

class EP_N89 <<InformationObjectClass>>

class EP_N96 <<InformationObjectClass>>

' MB-SMF & MB-UPF

class EP_N11mb <<InformationObjectClass>>

class EP_N16mb <<InformationObjectClass>>

class EP_N19mb <<InformationObjectClass>>

class EP_Nmb1  <<InformationObjectClass>>

class EP_Nmb9  <<InformationObjectClass>>

class EP_N3mb  <<InformationObjectClass>>

class EP_N4mb  <<InformationObjectClass>>

EP_N2  --|> EP_RP

EP_N3  --|> EP_RP

EP_N4  --|> EP_RP

EP_N5  --|> EP_RP

EP_N6  --|> EP_RP

EP_N7  --|> EP_RP

EP_N8  --|> EP_RP

EP_N9  --|> EP_RP

EP_N10 --|> EP_RP

EP_N11 --|> EP_RP

EP_N12 --|> EP_RP

EP_N13 --|> EP_RP

EP_N14 --|> EP_RP

EP_N15 --|> EP_RP

EP_N16 --|> EP_RP

EP_N17 --|> EP_RP

EP_N22 --|> EP_RP

EP_N26 --|> EP_RP

EP_N27 --|> EP_RP

EP_N28 --|> EP_RP

EP_N31 --|> EP_RP

EP_N32 --|> EP_RP

EP_N33 --|> EP_RP

EP_N40 --|> EP_RP

EP_N41 --|> EP_RP

EP_N42 --|> EP_RP

'since SA5#151

EP_Nmb1 --|> EP_RP

EP_Nmb9 --|> EP_RP

EP_N3mb --|> EP_RP

EP_N4mb --|> EP_RP

EP_RP <|-- EP_N58

EP_RP <|-- EP_N60

EP_RP <|-- EP_Npc4

EP_RP <|-- EP_Npc6

EP_RP <|-- EP_Npc7

EP_RP <|-- EP_Npc8

EP_RP <|-- EP_N59

EP_RP <|-- EP_N88

EP_RP <|-- EP_NLS

EP_RP <|-- EP_NL2

EP_RP <|-- EP_N20

EP_RP <|-- EP_N22

EP_RP <|-- EP_N61

EP_RP <|-- EP_N62

EP_RP <|-- EP_N63

EP_RP <|-- EP_NL3

EP_RP <|-- EP_NL6

EP_RP <|-- EP_NL5

EP_RP <|-- EP_NL9

EP_RP <|-- EP_N86

EP_RP <|-- EP_N84

EP_RP <|-- EP_N85

EP_RP <|-- EP_N87

EP_RP <|-- EP_N89

EP_RP <|-- EP_N96

'since SA5#151

EP_RP <|-- EP_N11mb

EP_RP <|-- EP_N16mb

EP_RP <|-- EP_N19mb

@enduml

End of Modified Section