3GPP TSG-SA5 Meeting #151 	S5-236602

Xiamen, China, 09 - 13 October 2023									Revision of SP-22XXXX

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0018 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on triggers for AMF charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CHF enable/disable the trigger in the AMF event charging will takes effect at the next AMF functionality, which is the same with the AMF charging profile mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the CHF enable/disable the trigger and charging profile for AMF charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMF charging is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.2.1, 5.2.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. SUPI).

Each trigger condition (i.e. chargeable event) defined for the 5G connection and mobility converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.2.1.2.1.1 summarizes the set of default trigger conditions and their category which shall be supported by the AMF when charging is active for the corresponding AMF functionality. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from AMF towards the CHF.

Table 5.2.1.2.1.1: Default Trigger conditions in AMF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Registration |  |  |  |  |  |
| Registration checked | - | Immediate | Not Applicable | Yes (NOTE1,2) | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Registration completed | - | Immediate | Not Applicable | Yes (NOTE1,2) | PEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |
| Deregistration | - | Immediate | Not Applicable | Yes (NOTE1,2) | PEC: Charging Data Request [Event] |
| N2 connection |  |  |  |  |  |
| Start of N2 connection | - | Immediate | Not Applicable | Yes (NOTE1,2) | PEC: Charging Data Request [Event] |
| End of N2 connection | - | Immediate | Not Applicable | Yes (NOTE1,2) | PEC: Charging Data Request [Event] |
| Location reporting |  |  |  |  |  |
| User Location change | - | Immediate | Not Applicable | Yes (NOTE1,2) | PEC: Charging Data Request [Event] |
| Change of UE presence in Presence Reporting Area(s) | - | Immediate | Not Applicable | Yes (NOTE1,2) | PEC: Charging Data Request [Event] |
| NOTE 1: CHF enable/disable the trigger takes effect at the next AMF functionality. NOTE 2: The set of triggers related to the AMF functionality enable/disabled together. |  |  |  |  |  |

AMF functionality charging activation/deactivation is performed by CHF via AMF charging profile update as defined in clause 5.2.1.2.3.

The AMF charging profile is defined in clause 5.2.1.2.2 and shall be used by the AMF to determine whether converged charging is active or not, and which scenario is applicable when active.

During UE initial registration in the AMF, if registration charging is active in the AMF charging profile/Charging Characteristics, a Charging Data Request [Event, Registration] or Charging Data Request [Initial, Registration] shall be sent to CHF in the corresponding charging scenario (i.e. IEC, PEC or ECUR) when applicable triggers are met. At N2 connection, if N2 connection charging is active in the AMF charging profile/charging characteristics, a Charging Data Request [Event, N2 connection] shall be sent to CHF in PEC scenario when applicable triggers are met.

At N2 connection, if location reporting charging is active in the AMF charging profile/charging characteristics, the AMF shall start the location reporting control procedure towards NG-RAN specified in clause 4.10 of TS 23.502 [202] for a continuous reporting whenever the UE location changes. A Charging Data Request [Event, location reporting] shall be sent by AMF upon receiving the location report from NG-RAN with current UE location.

If the AMF receives in a Charging Data Response [Event/Initial] from CHF, an updated AMF charging profile deactivating location reporting charging, and the AMF has a N2 connection, the AMF shall cancel the location reporting control procedure towards NG-RAN.

Next change

5.2.1.2.3	Independent triggers activation/deactivation

During the UE registration lifetime in the AMF, the CHF may activate/deactivate charging for any AMF functionality (i.e. Registration, N2 connection, Location Reporting) each time the CHF is invoked for a given AMF functionality. The set of triggers enabled/disabled via charging activation/deactivation by CHF are independent from the invoked AMF functionality. The triggers activation/deactivation take effect at the next AMF functionality invoking the CHF.

Table 5.2.1.2.3.1 describes the applicable messages for CHF to activate/deactivate triggers via update of AMF Charging profile:

Table 5.2.1.2.3.1: AMF charging profile update by CHF

| Functionality | AMF Charging Profile field which can be updated | Message from CHF to AMF |
| --- | --- | --- |
| Registration | - charging "active or inactive" | Charging Data Response [Event/Initial/Termination] |
| N2 connection | - charging "active or inactive" | Charging Data Response [Event/Initial/Termination] |
| Location Reporting | - charging "active or inactive" - level for location - list of Presence Reporting Area(s) | Charging Data Response [Event/Initial/Termination] |

In ECUR scenario, any update of AMF Charging Profile received in Charging Data Response [Initial/Termination] will take effect after the corresponding Charging Data Response [Termination] until next update.

End of change