# 3GPP TSG-SA5 Meeting #152	S5-237485

# 13 - 17 November 2023, Chicago, IL, USA

# Source:	Nokia, Nokia Shanghai Bell

# Title:		Input to DraftCR 28.105 Configuration of load balancing functions

# Document for:	Approval

# Agenda Item:	6.4.3.1 - AIML_MGMT_WoP#1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	S5-235988 DraftCR : “Draft CR AIML_MGMT - TS 28.105; Enhancements for AI-ML management”.

# 3	Rationale

The inference function for AI/ML-based distributed Load balancing has been introduced in TS28.105 but without any description of its configurable features. This pCR adds the description of configurable features and requirements for AI/ML-based Distributed Load balancing Functions.

# 4	Detailed proposal

Start of First change

### 6.5.4	AI/ML inference function configuration management

#### 6.5.4.1	Description

TBD

#### 6.5.4.2	Use cases

##### 6.5.4.2.2	Managing AI/ML-based distributed Load balancing

An AI/ML-based Distributed Load balancing (DLB) function may use one or more ML entities to derive load balancing recommendations.

This function needs to be managed.

To optimize load distribution among cells, the DLB function needs to compute the degree of overlap between a serving cell and given candidate target cell. For a given proposed redistribution of load, an MnS consumer may wish to know the corresponding cell neighborliness. The DLB function should inform the MnS consumer of the computed degree of neighborliness, called the Cell Proximity Coupling (CPC).

DLB may the distribution of load in consideration of the services or network slices that applicable for the different users, i.e., DLB would optimize the cell reselection, HO or transmission settings in considering the load, the radio characteristics such as RLF and ping pong HOs, as well as service dependent QoE, such as latency or throughput. The DLB function should provide inference indicating the optimal load redistribution settings corresponding to different service or slice use distributions among the cells.

#### 6.5.4.3	Requirements for AI/ML inference management

Table 6.5.4.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF-01 | The MnS producer of AI/ML-based Distributed Network Energy Saving should enable an authorized MnS consumer to request to manage the Energy saving inference function | Managing AI/ML-enabled for Distributed Network Energy Saving (clause 6.5.4.2.1) |
| REQ- AI/ML_INF-02 | The MnS producer of AI/ML-based Distributed Mobility Optimization should enable an authorized MnS consumer to request to manage the Mobility Optimization inference function | Managing AI/ML-enabled for distributed Mobility Optimization (clause 6.5.4.2.2) |
| REQ- AI/ML_INF-03 | The MnS producer of AI/ML-based Distributed Load balancing should enable an authorized MnS consumer to request to manage the Load balancing inference function | Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |
| REQ- INF_DLB-01 | The MnS producer of AI/ML-based Distributed Load balancing should enable an authorized MnS consumer to receive information on the computed Cell Proximity Coupling (CPC), which is the degree of neighborliness among any serving cell and its candidate target cell. | Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |
| REQ- INF_DLB-02 | The MnS producer of AI/ML-based Distributed Load balancing should enable an authorized MnS consumer to receive inference indicating the optimal load redistribution settings corresponding to different service or slice use distributions among the cells. | Managing AI/ML-enabled for distributed Load balancing (clause 6.5.4.2.3) |

End of Changes