3GPP TSG-SA5 Meeting #152 	S5-237667

Chicago, US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | draftCR | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 InputToDraftCR TS 28.533 Correct the description of attribute identityType and identityName |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSAC |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The definitions of attributes identityType and identityName are wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Switch the definitions of attributes identityType and identityName. - Turn the enum value into uppercase |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definitions of attributes identityType and identityName are wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Input to draftCR S5-235998 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

X.4	Attribute definitions

X.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| identityType | This indicates a type of identifier AllowedValues: USERNAME, EMAIL_ADDRESS, PHONE_NUMBER, IP_ADDRESS, MACHINEUSER | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| identityName | This defines a readable string to uniquely represent an identity AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| credential | The credential of an MnS consumer or producer used for authentication with authentication service producer. It could be password, certificate, key, pass phrase, etc., based on authentication protocol and factor. AllowedValues: NA | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No value isNullable: False |
| roleRefList | This defines the list of roles associated with an identity AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| roleName | This string defines the name of a role AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| mnsType | This attribute defines the type of management service AllowedValues: ProvMnS, FaultSupervisionMnS, StreamingDataReportMnS, FileDataReportingMnS | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| permissionsForMnSsList | This defines the list of permissions for MnSs which constitue the resources and corresponding actions AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| permissionList | This defines the list permissions for resources and corresponding actions for static objects AllowedValues: NA | type: Permission multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentA | This defines the operation of the MnS as defined in 28.532 AllowedValues: NA | type: string multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| componentBObjectClass | This attribute defines a MnS component type B represented by information models of managed entities. A MnS component type B is also called Network Resource Model (NRM). MnS component type B examples are: 1) Network resource models as defined in TS 28.622 2) Network resource models as defined in TS 28.541 AllowedValues: eg: ManagedNFService, HeartbeatControl, NtfSubscriptionControl, AlarmList, TraceJob, PerfMetricJob, | type: ENUM multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentBAttributePermissionList | This defines the list of componentBAttributePermissions which contains the IOC attribute and corresponding actions AllowedValues: NA | type: componentBAttributePermissions multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentC | This attribute defines a MnS component type C which is the performance management data, KPIs of the managed entity and fault information of the managed entity. The following are examples of Management service component type C: 1. Alarm information as defined in TS 28.532 and TS 28.545 2. Performance data as defined in TS 28.552 , TS 28.554 and TS 32.425. AllowedValues: NA | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentBAttributeName | This attribute defines a attribute of a MnS component type B represented by information models of managed entities. A MnS component type B is also called Network Resource Model (NRM). Attributes of MnS component type B examples are: 1) Network resource models as defined in TS 28.622 2) Network resource models as defined in TS 28.541 AllowedValues: NA | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentBAttributeOperation | This indicates which operation is specified on the particular component B attribute AllowedValues: Read-only, Writable | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |

End of changes