3GPP TSG-SA5 Meeting #152 	S5-237769

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0040 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.254 Addition of Trusted AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-10-26 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The charging when a Trusted AF is used without NEF is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allowing the Trusted AF to interface with the CHF using the same mechanism as the NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How charging can be performed when a Trusted AF is used without a NEF is unclear which may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Annex x (normative):
Trusted AF charging

# x.1	General

This Annex describes charging support in the Trusted AF.

The Trusted AF charging is an alternative to the NEF based and may be used instead of the NEF based for Trusted AFs.

# x.2	Charging Architecture for Trusted AF charging

## B.2.1	General

The Trusted AF converged charging architecture is specified in the following clause x.2.1.1.

### x.2.1.1	Logical Charging architecture

Converged charging may be performed by the Trusted AF interacting with CHF using Nchf specified in TS 32.290 [57] and TS 32.291 [58]. In order to provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.), the Trusted AF may be able to perform converged charging for the southbound API access.

If there is a trusted AF then this can take the place of the NEF and use the same southbound interfaces, without providing any northbound interface, in this case an architectural options for converged charging are depicted in figure x.1.1 in service-based representation:

Figure x.1.1: Trusted AF converged charging architecture

Details on the interfaces and functions can be found in TS 32.240 [1] for the general architecture components, Ga is described in clause 5.2.4 and Bea in clause 5.2.5 of this document, and Nchf is described in TS 32.290 [57].

Figure x.1.2 depicts the Trusted AF converged charging architecture for non-roaming in reference point representation:

Figure x.1.2: Northbound API converged charging architecture non-roaming reference point representation

## x.2.3	Charging principles and scenarios

### x.2.3.1	Basic principles

The Trusted AF acts like the NEF in the specification with exception that the Trusted AF charging is triggered based on southbound API access. This means that for all flows in clause 5.4.2 the NEF should be replaced by the Trusted AF and all the requests/responses/notifications to/from the Trusted AF would be in the opposite direction i.e., requests would be going from the Trusted AF, responses would be going to the Trusted AF, notifications would be going to the Trusted AF, and notification acknowledgements would be going from the Trusted AF

End of changes