3GPP TSG-SA5 Meeting #152 	S5-237772

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0958 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.298 Correction of duration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 3GPP TS 32.298 table 5.1.5.0.1.”, specifies Duration as “This field holds the duration of this record.”, and clause 5.1.5.1.3 specifies it as “This field contains the relevant duration in seconds for the session. It is the duration from Record Opening Time to record closure. For partial records this is the duration of the individual partial record and not the cumulative duration.”. The 3GPP TS 32.298 table 5.1.5.0.1 also states that the Duration is a mandatory parameter which means that it also applies to events. In case of "Individual partial record" a new CDR shall be opened for each Charging Data Request [Initial, Update, Termination], charging information shall be added and then closed. This means that in case of events and individual partial record it will be the time from opening i.e., reception of Charging Data Request [Initial, Update, Termination], until closed when the charging information have been added, while for default partial records it would be from the opening until a trigger for partial closing is met. The Duration is in seconds, and any rounding/truncation is not specified in 3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add consideration around rounding/truncation, following 3GPP TS 32.298 clause 5.1.2.2.12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use of rounding/truncation may be misinterpreted which may cause interoperability issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.5.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-236703 |  |  |  |  |  |  |  |  |

First change

##### 5.1.5.1.3	Duration

This field contains the relevant duration in seconds from Record Opening Time to record closure. For partial records this is the duration of the individual partial record and not the cumulative duration.

It should be noted that the internal time measurements may be expressed in terms of tenths of seconds or even milliseconds and, as a result, the calculation of the duration may result in the rounding or truncation of the measured duration to a whole number of seconds.

Whether or not rounding/truncation is to be used is considered outside the scope of the present document, however it is subject to the following restrictions:

-	A duration of zero seconds shall be accepted providing that the transferred data volume is greater than zero.

-	The same method of truncation/rounding shall be applied to both single and partial records.

End of changes