3GPP TSG-SA5 Meeting #152 	S5-237601

# Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0493 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update connection data volumes meansurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the virtualized resource utilization analysis for MDAS (see 8.4.x.1 in S5-237274 DraftCR for eMDAS_ph2), Connection Point data volumes of NF is used as the enabling data, however the current measurement of Connection Point data volumes is only applicable for EAS, not other virtualised NFs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Suggest to extend the usage of measurement for Connection Point data volumes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NF resource utilization analysis is not applicable for most of NFs as defined by 3GPP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2.1, 5.7.2.2 and 5.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.7.2	Connection data volumes of NF

5.7.2.1	Data volume of incoming bytes

a)	This measurement provides the number of incoming bytes received for a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the ByteIncomingVnfExtCp measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of received measurement(s) from VNFC instance to MOI(s) of NF(s). The measurement is generated by aggregating the values of the ByteIncomingVnfExtCp measurement(s).

d)	A single integer value.

e)	DataVolum.InBytes

f)	AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

EASFunction

EESFunction

ECSFunction

g)	Valid for packet switched traffic.

h)	5GS.

5.7.2.2	Data volume of outgoing bytes

a)	This measurement provides the number of outgoing bytes transmitted from a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the ByteOutgoingVnfExtCp measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s)  from VNFM, and maps the measured object of received measurement(s) from VNFC instance to the MOI(s) of NF(s). The measurement is generated by aggregating the values of the ByteOutgoingVnfExtCp measurement(s).

d)	A single integer value.

e)	DataVolum.OutBytes

f)	AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

EASFunction

EESFunction

ECSFunction

g)	Valid for packet switched traffic.

h)	5GS.

5.7.2.3	Data volume of incoming packets

a)	This measurement provides the number of incoming packets received by a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the PacketIncomingVnfExtCp measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of received measurement(s) from VNFC instance to MOI(s) of NF(s). The measurement is generated by aggregating the values of the PacketIncomingVnfExtCp measurement(s).

d)	A single integer value.

e)	DataVolum.InPackets

f)	AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

EASFunction

EESFunction

ECSFunction

g)	Valid for packet switched traffic.

h)	5GS.

5.7.2.4	Data volume of Outgoing packets

a)	This measurement provides the number of outgoing packets received by a virtualized 3GPP NF. This measurement is not applicable to the scenario that one VNFC instance supports more than 1 NFs.

b)	OM.

c)	The measurement job control service producer for NF(s) receives the PacketOutgoingVnfExtCp measurement(s) (see ETSI GS IFA 027 [17]) for the VNFC instances(s) from VNFM, and maps the measured object of received measurement(s) from VNFC instance to MOI(s) of NF(s). The measurement is generated by aggregating the values of the PacketOutgoingVnfExtCp measurement(s).

d)	A single integer value.

e)	DataVolum.OutPackets

f)	AMFFunction
SMFFunction
UPFFunction
N3IWFFunction
PCFFunction
AUSFFunction
UDMFunction
UDRFunction
UDSFFunction
NRFFunction
NSSFFunction
SMSFFunction
LMFFunction
NWDAFFunction
NGEIRFunction
SEPPFunction

EASFunction

EESFunction

ECSFunction

g)	Valid for packet switched traffic.

h)	5GS.

End of Change