# 3GPP TSG SA WG5 Meeting #152                 	           S5-237863

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Update CDR generation, record transfer and file transfer in charging scenarios

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the updated CDR generation, record transfer and file transfer in charging scenarios of TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

This contribution proposes updated CDR generation, record transfer and file transfer in charging scenarios of TS 32.282.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

5.2.3	CDR generation

The CHF CDRs for TSN charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The CHF CDRs can be per individual subscriber or per TSN subscriber.

The following clauses describe in detail the conditions for generating the CHF CDR, which shall be supported by the CHF.

5.2.3.2.1	General

A TSN charging CHF CDR is used to capture charging information related to TSN chargeable events for PEC, IEC and ECUR scenarios.

5.2.3.2.2	Triggers for CHF CDR generation

A TSN charging CHF CDR is generated by the CHF for each received Charging Data Request [Event].

##### 5.2.3.2.3	Triggers for CHF CDR opening

A CHF CDR shall be opened when the CHF receives Charging Data Request [Initial] with charging information added.

##### 5.2.3.2.4	Triggers for CHF CDR closure

When the CHF receives Charging Data Request [Termination], the charging information shall be added in the CHF CDR and the CDR shall be closed.

5.2.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [55].

5.2.5	Btsn CDR file transfer

Details of the Btsn protocol application are specified in TS 32.297 [56].

End of change