3GPP TSG-SA5 Meeting #152	S5-237735

# Chicago, US, 13th – 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0230 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.531 Update allocate for network slice isolation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSRULE |  |  |  |  |  | Date: |  |  | 2023-10-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An operator has a set of predefined IsolationProfiles. The consumer need to indicate which Isolationprofile should be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the use case for network slice and network slice subnet allocation - Update AllocateNsi and AllocateNssi operation to include additional parameter where the consumer indicates which isolationProfile is needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network slice isolation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2, 5.2.1, 5.2.2, 6.5, 6.5.1.2, 6.5.1.3, 6.5.2.2, 6.5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.541 CR 1119 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.1.1	Network slice instance allocation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To satisfy request for allocation of a network slice instance with certain characteristics, by creating a new or using an existing network slice instance. |  |
| Actors and Roles | A network slice provisioning management service consumer. |  |
| Telecom resources | Network slice instance Network slice subnet instance Transport network A network slice provisioning management service provider. A network slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice provisioning management service provider receives the request for allocation of the network slice instance with network slice related requirements. The network slice provisioning management service provider has the capability to process the network slice related requirements (e.g., SLA information from GSMA GST) which are represented by service profile parameters [6]. The service profile can be translated to the corresponding requirements for dedicated domains and NSSIs. |  |
| Step 1 (M) | If the requested NSI can be shared and if an existing NSI can be used, the network slice provisioning management service provider decides to use the existing NSI. Modification of the existing NSI may be needed to satisfy the network slice instance related requirements. Use case is completed go to "Step 8". Otherwise, the network slice provisioning management service provider triggers to create a new NSI, for which the following steps 2 – 8 are needed. |  |
| Step 2 (M) | The network slice provisioning management service provider decides on the constituent NSSIs and the topology of the NSI to be created using the information from service profile [6] and if applicable the isolation profile. For the constituent NSSIs, the network slice provisioning management service provider derives network slice subnet related requirements from the network slice related requirements. If reconfiguration of the transport network is needed, the network slice provisioning management service provider derives transport network related requirements (e.g., latency, bandwidth) from the network slice related requirements. |  |
| Step 3 (M) | For the required NSSI(s), the network slice provisioning management service provider sends network slice subnet related requirements to the network slice subnet provisioning management service provider to request allocation of the required NSSI(s). | Network slice subnet instance allocation use case |
| Step 4 (M) | The network slice provisioning management service provider receives the information of the allocated NSSI(s) (e.g., the management identifier of NSSI, service access point information of NSSI, external connection point information of NSSI) from NSSMF. |  |
| Step 5 (M) | The network slice provisioning management service provider, via the network slice subnet provisioning management service provider, sends the transport network related requirements (e.g., external connection point, latency and bandwidth) to the TN Manager. The TN manager reconfigures the TN accordingly and responds to the network slice provisioning management service provider via the network slice subnet provisioning management service provider. |  |
| Step 6 (M) | The network slice provisioning management service provider receives the response from TN Manager via the network slice subnet provisioning management service provider. |  |
| Step 7 (M) | The network slice provisioning management service provider associates the NSSI(s) with the corresponding NSI (e.g., allocation of the management identifier of NSI and mapping the management identifier of NSI with the received management Identifier of NSSI(s)), if applicable with the corresponding isolation profile, and triggers to establish the links between the service access points of the NSSI(s). |  |
| Step 8 (M) | The network slice provisioning management service provider notifies the network slice instance information of NSI (e.g., the management identifier of NSI). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | An NSI is ready to satisfy the network slice related requirements. |  |
| Traceability | REQ-PRO_NSSI-FUN-1, REQ-PRO_NSI-FUN-3, REQ-PRO_NSI-FUN-X. |  |

### 5.1.2	Network slice subnet instance allocation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Create a new network slice subnet instance or use an existing network slice subnet instance to satisfy the network slice subnet related requirements; provide the provisioning service consumer with identity of the NFVO which the consumer can use for further access to the information of the involved VNFs, PNFs and NSs. |  |
| Actors and Roles | A network slice subnet provisioning management service consumer. |  |
| Telecom resources | Network Slice Subnet instance Network Service instance A network slice subnet provisioning management service provider. The operator deployed NFVO to manage the lifecycle of VNFs and interconnection between the VNFs and PNFs in terms of the NS instances. |  |
| Assumptions | Network slice subnet instance may include network functions which are virtualized. |  |
| Pre-conditions | VNF Packages for virtualized network functions to be included in the network slice subnet instance have been already on-boarded. |  |
| Begins when | The network slice subnet provisioning management service consumer sends to the network slice subnet provisioning management service provider a request for an NSSI to be associated with the NSI; the request contains network slice subnet related in TS 28.541 [6]. The network slice subnet provisioning management service provider receives request for a network slice subnet instance. The request contains network slice subnet related requirements. The request may also include query of the identity of the NFVO to be used. |  |
| Step 1 (M) | Based on the network slice subnet related requirements received and if applicable the isolation profile, the network slice subnet provisioning management service provider decides to create a new NSSI or use an existing NSSI. |  |
| Step 2 (M) | If an existing network slice subnet instance is decided to be used, the network slice subnet provisioning management service provider may trigger to modify the existing network slice subnet instance to satisfy the network slice subnet related requirements. Go to “Step 8”. Otherwise, the network slice subnet provisioning management service provider triggers to create a new NSSI, the following steps are needed. |  |
| Step 3 (O) | If the required NSSI contains constituent NSSI(s) managed by other network slice subnet provisioning management service provider (s), the first network slice subnet provisioning management service provider derives the requirements for the constituent NSSI(s) and sends those requirements to the corresponding network slice subnet provisioning management service provider (s) which manages the constituent NSSI(s). The first network slice subnet provisioning management service provider receives the constituent NSSI information from the other network slice subnet provisioning management service provider (s) and associates the constituent NSSI(s) with the required NSSI. |  |
| Step 4 (M) | Based on the network slice subnet related requirements received and SliceProfile [6], the network slice subnet provisioning management service provider decides that to satisfy the NSSI requirements, the part of the network controlled by certain NFVO should be involved. The network slice subnet provisioning management service provider determines the NS related requirements (i.e., information about the target NSD and additional parameterization for the specific NS to instantiate, see clause 7.3.3 in ETSI GS NFV-IFA013 [3]), and if applicable the isolation rules. |  |
| Step 5 (M) | Based on the NS related requirements, the network slice subnet provisioning management service provider triggers corresponding NS instantiation request to NFVO via Os-Ma-nfvo interface as described in clause 6.4.3 in TS 28.525 [2], and the NFVO performs NS instantiation. (see note) | TS 28.525 [2] Clause 6.4.3 NS instance use cases |
| Step 6 (M) | The network slice subnet provisioning management service provider associates the NS instance with corresponding network slice subnet instance (e.g., allocation of the management identifier of NSSI and mapping with the corresponding identifiers). |  |
| Step 7 (M) | The network slice subnet provisioning management service provider is using the NF provisioning service to configure the NSSI constituents. In case of RAN NSSI, the configuration contains RRM policy information for individual Radio cells. In the cells shared by multiple NSSIs such policy includes guidance for split of Radio resources between the NSSIs. | NF provisioning service |
| Step 8 (M) | The network slice subnet provisioning management service provider notifies the provisioning service consumer with the NSSI information (e.g., the management identifier of NSSI). The network slice provisioning management service provider associates the NSSI with the NSI and if applicable the isolation profile. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | A NSSI is ready to satisfy the network slice subnet related requirements. |  |
| Traceability | REQ-PRO_NSSI-FUN-2, REQ-PRO_NSSI-FUN-3, REQ-PRO_NSSI-FUN-4, REQ-PRO_NSSI-FUN-5, REQ-PRO_NSSI-FUN-6, REQ-PRO_NSSI-FUN-14, REQ-PRO_NSSI-FUN-X. |  |
| NOTE: According to the TS 28.525 [2], for the PNFs, NS instantiation includes only establishment of interconnection with other NFs. |  |  |

2nd Change

### 5.2.1	Requirements for network slice provisioning service

REQ-PRO_NSI-FUN-1	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request a network slice instance.

REQ-PRO_NSI-FUN-2	The network slice provisioning service provider shall have the capability allowing its authorized consumer to send the network slice related requirements.

NOTE 1: The network slice related requirements include requirements such as area traffic capacity, coverage area, isolation/sharing, end-to-end latency, mobility, overall user density, priority, service availability, service reliability, UE speed; see TS 22.261 [5] where these parameters are defined for end user services.

NOTE 2: The network slice related requirements also include requirements derived from the Generic network Slice Template (GST) defined by GSMA in [9].

NOTE 3: The SLA requirements can be translated to service profile which can be used to decides on the constituent NSSIs and the topology of the NSI.

NOTE 4: Void

REQ-PRO_NSI-FUN-3	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request the deallocation of a network slice instance.

REQ-PRO_NSI–FUN-4	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request activation of a network slice instance.

REQ-PRO_NSI–FUN-5	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request deactivation of a network slice instance.

REQ-PRO_NSI-FUN-6	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request the modification of a network slice instance.

REQ-PRO_NSI-FUN-7	The network slice provisioning service provider shall have the capability allowing its consumer to obtain the network slice management data.

REQ-PRO_NSI-FUN-8	The network slice provisioning service provider shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning the requested network slice instance at a particular point of time.

REQ-PRO_NSI-FUN-X 	The network slice provisioning service provider shall have the capability to satisfy the requirements using shared or dedicated managed resources for the network slice.

REQ-PRO_NSI-FUN-X 	The network slice provisioning service provider shall have the capability to allow an authorized consumer to select an isolation profile.

### 5.2.2	Requirements for network slice subnet provisioning service

REQ-PRO_NSSI-FUN-1	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request a network slice subnet instance.

REQ-PRO_NSSI-FUN-2	The network slice subnet provisioning service provider shall have the capability of interaction with NFVO via the NS lifecycle management interface.

REQ-PRO_NSSI-FUN-3	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to send network slice subnet related requirements.

REQ-PRO_NSSI-FUN-4	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request to create a new NSSI or use an existing NSSI based on the network slice subnet related requirements.

REQ-PRO_NSSI-FUN-5	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request to associate the NS instance with corresponding NSSI.

REQ-PRO_NSSI-FUN-6	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request the configuration of the RAN NSSI constituents with the RRM policy information for simultaneous support of multiple NSIs.

REQ-PRO_NSSI-FUN-7	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain network slice subnet instance information.

REQ-PRO_NSSI-FUN-8	The network slice subnet provisioning service provider shall have the capability of allowing its authorized consumer to request the deallocation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-9	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request activation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-10	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request deactivation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-11	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request modification of a network slice subnet instance.

REQ-PRO_NSSI-FUN-12	The network slice subnet provisioning service provider shall have the capability allowing its consumer to obtain information regarding available network slice subnet resources.

REQ-PRO_NSSI-FUN-13	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning a network slice subnet instance at a particular point of time.

REQ-PRO_NSSI-FUN-14	The network slice subnet provisioning service provider shall have the capability to satisfy the request to consume the NF provisioning service.

REQ-PRO_NSSI-FUN-16	The network slice subnet provisioning service provider shall have the capability allowing its consumer to provide slice specific operation information for the (re)configuration to a NSSI.

REQ-PRO_NSI-FUN-17	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to assign priority of a network slice subnet.

REQ-PRO_NSSI-FUN-18	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain network slice subnet provider capability information.

REQ-PRO_NSSI-FUN-X 	The network slice subnet provisioning service provider shall have the capability to satisfy the requirements using shared or dedicated managed resources for the network slice subnet.

REQ-PRO_NSSI-FUN-X 	The network slice subnet provisioning service provider shall have the capability to allow an authorized consumer to select an isolation profile.

## 6.5.	Operations of provisioning

### 6.5.1	AllocateNsi operation

#### 6.5.1.1	Description

This operation is invoked by network slice provisioning MnS consumer to request the provider to allocate a network slice instance to satisfy network slice related requirements. The provider may create a new NSI or using existing NSI to satisfy the request. The requirements in the request are compared/matched against the actual capabilitites of all candidate NSIs by the provider. If an existing NSI can be found it is eligible for allocation, or else a new NSI is created, provided that required NSSIs can be created.

#### 6.5.1.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies the network slice related requirements defined in ServiceProfile in Clause 6.3.3 and may contain the IsolationProfile in Clause 6.3.b in TS 28.541 [6]. |

#### 6.5.1.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the ServiceProfile [6] which has been allocated and the actual value assigned to each. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |
| networkSliceDN | M | The DN of NetworkSlice MOI uniquely identifying the network slice instance. |  |
| isolationProfile | O | The DN of IsolationProfile MOI uniquely identifying the isolation profile instance. |  |

### 6.5.2	AllocateNssi operation

#### 6.5.2.1	Description

This operation is invoked by network slice subnet provisioning MnS consumer to request the provider to allocate a network slice subnet instance to satisfy the network slice subnet related requirements. The provider may create a new NSSI or using existing NSSI to satisfy the request. The requirements in the request are compared/matched against the actual capabilitites of all candidate NSSIs by the provider. If an existing NSSI can be found  then it is eligible for allocation, or else a new NSSI is created, provided that resoures are available.

#### 6.5.2.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies the network slice subnet related requirements defined in SliceProfile in Clause 6.3.4 and may contain the IsolationProfile in Clause 6.3.b in TS 28.541 [6]. |

#### 6.5.2.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the SliceProfile [6] which has been allocated and the actual value assigned to each. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |
| networkSliceSubnetDN | M | The DN of NetworkSliceSubnet MOI uniquely identifying the network slice subnet instance. |  |
| isolationProfile | O | The DN of IsolationProfile MOI uniquely identifying the isolation profile instance. |  |

End of change