3GPP TSG-SA5 Meeting #153 	S5-240350

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | 0012 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.557 Add solution for shared and dedicated resources in NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The solution for use case "shared/dedicated resource demand for NPN service customers" is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide a solution for shared/dedicated resource demand in NPN by reusing the solution of IOC IsolationProfile which shall be applicable to realize isolation for managed resources. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution is missing in NPN specification TS 28.557 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new clause 6.1.X, new Annex B.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 6.1	Generic solutions for management of NPN

### 6.1.X	Solution for shared/dedicated resources in NPN

To satisfy the shared/dedicated resources demand for NPN service customers, the provisioning MnS producer may provision isolation profiles which contain sets of isolation requirements when configuring managed resources.

Figure 6.1.X-1 shows the process on network slice allocation considering resource isolation requirements.

Figure 6.1.X-1: Process on network slice allocation considering shared/dedicated resources requirements

1)	NPN service customer (e.g. tenant) identifies the SLA requirements which includes the shared/dedicated resources demand based on its target communication applications, such as smart grid applications. The shared/dedicated resource demand may mainly be derived from the security aspects of the communication applications.

2)	NPN-SC sends the sharing/dedicated resources requirements to NPN-SP.

3)	NPN-SP maps the shared/dedicated resources requirements into isolation profiles.

4)	NPN-SP requests the allocation of instance of IOC IsolationProfile defined in TS 28.541 [7].

5)	To fulfil the shared/dedicated resources demand, NSMS_P and/or NSSMS_P could create a new instance of IOC IsolationProfile or reuse an existing one with proper modification if necessary. The MOI IsolationProfile contains resource isolation rules, which specify whether the resources (e.g. management resource, network service resource) is dedicated or could be shared with others.

6)	NSMS_P returns the isolation profile allocation results, which contains the information of IsolationProfile instance if the allocation succeeds.

7)-9)	Same as steps 1)-3) in clause 6.3.1.

10)	When allocating NSI (NSSI) for NPN-SC who has the shared/dedicated resource demand, the NSMS_P (NSSMS_P) associates IsolationProfile instance(s) that allocated in Step 5 to NSI (NSSI).

11)	Same as step 5) in clause 6.3.1 with additional association relations with IsolationProfile instance(s).

Since the network slice instance is allocated for NPN service customer satisfying its dedicated or shared resource demand derived from the targeted running communication applications, there are several interpretations with different association scenarios between NSI(s) and IsolationProfile MOI (s).

For one IsolationProfile MOI associated to two or more network slice instances, see Figure 6.2.1-6: IsolationProfile NRM fragment relationship related to NetworkSlice in TS 28.541 [7], these NSIs are configured with same resource isolation rule:

-	If the resource isolation rule requires the resources to be shared, which means the attribute isolationRule is SHARED, only NSIs associated to this IsolationProfile MOI could share the resources with each other. NSIs associated to this IsolationProfile MOI could not share the resources with other NSIs which associate with different IsolationProfile instance(s).

-	If the resource isolation rule requires the resources to be dedicated, which means the attribute isolationRule is DEDICATED, NSIs associated to this IsolationProfile MOI are supported by dedicated resources exclusively.

For one IsolationProfile MOI associated to one network slice instance, see Figure 6.2.1-6: IsolationProfile NRM fragment relationship related to NetworkSlice in TS 28.541 [7], if the isolationRule is SHARED, it means the network slice resources can be shared with other NSI(s), which helps the Provisioning MnS producer make choices when allocating other NSIs.

Figure 6.1.X-2: Example of solutions for shared/dedicated resource demand for NPN-SC

Figure 6.1.X-2 illustrates an example of solutions for shared/dedicated resource demand for NPN-SC.

The dedicated-shared resource requirements come from the use case described in clause 5.1.0.X. Different NSI is allocated for each L-Group to fulfil the dedicated logical resources requirement. For NSIs allocated for same P-Group, they are associated with the same isolation profile instance with isolationRule equals to SHARED. For NSIs allocated for different P-Group, the associated isolation profile instance is also different.

Next Change

## B.Y	Process on network slice allocation considering shared/dedicated resources requirements

The following PlantUML source code is used to describe the Process on network slice allocation considering shared/dedicated resources requirements, as depicted by Figure 6.1.X-1:

@startuml

skinparam NoteBackgroundColor White

participant "NPN-SC" as cmer

participant "NPN-SP" as prov

participant "NSMS_P" as prov2

participant "NSSMS_P" as prov3

rnote over cmer

1.Identify SLA requirements

and shared/dedicated resource demand

endrnote

cmer -> prov: 2.send shared/dedicated resource requirements

prov -> prov: 3.map the shared/dedicated \nresource requirements \ninto isolationProfile

prov -> prov2: 4.request the allocation of isolationProfile

rnote over prov2,prov3

5.IsolationProfile instance allocation

(Decide to create a new IsolationProfile

instance or reuse an existing one)

endrnote

prov2 -> prov: 6.return isolationProfile allocation results

cmer -> prov: 7. SLA requirements\n(coverage, DL/UL throughout, latency, NPN lifetime)

prov -> prov: 8. Map SLS requirements \n into ServiceProfile

prov -> prov2: 9. AllocateNsi request

rnote over prov2,prov3

10. NSI Allocation;

Associate NSI and NSSI with

proper isolationProfile instance(s)

endrnote

prov2 -> prov:11. AllocateNsi response

@enduml

End of changes