3GPP TSG-SA5 Meeting #153	S5-240667

# Sevilla, Spain, 29th Jan 2024 - 2nd Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0178 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.554 KPI for End to End Reliability in DL and UL for URLLC NS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reliability performance is also an integral part of URLLC slice by nature. A CSP/NOP may want to assess “reliability” of a URLLC Network Slice (NS) that is being used for communication services requiring very high reliability, such as Cloud/Edge/Split Rendering, Gaming or Interactive Data Exchanging, wireless road-side infrastructure backhaul etc. Moreover, Reliability by definition is a unidirectional characterisitic of a network slice hence needs to be calculated separately in DL and UL Hence, the solution is proposed to calculate reliability performance of a URLLC slice for comprehensive E2E assessment in DL and UL separately. |  |  |  |  |  |  |  |  |
| Summary of change: |  | new KPIs for calculating End to end reliability of a URLLC network slice separately in DL and UL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | End to End reliability of URLLC slice cannot be determined in DL and UL |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause no. 6.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 6.8.1.Y	End to End Downlink Reliability KPI of an URLLC Network Slice

a)  E2ERelURLLC,PSR,DL

b) This KPI describes the end to end downlink Reliability based on Packet Success Rate (PSR) Percentage of an URLLC network slice. It is the multiplication of Packet transmission reliability KPI in DL on Uu interface and Packet transmission reliability KPI in DL on N3 interface. By definition, Reliability is a unidirectional performance metric in a network. i.e. The percentage value of the packets successfully delivered out of all the packets transmitted. It is a measure of the DL packet delivery success i.e. PSR% over a combined end to end interfaces of Uu and N3. It is a percentage value (%). This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

c)  Thus, KPI E2ERelURLLC,PSR,DL is given by:

E2ERelURLLC,PSR,DL  =  DLRelPSR_Uu.SNSSAI × DLRelPSR_N3.SNSSAI

Where,

DLRelPSR_Uu.SNSSAI is PSR% in DL for Uu interface per SNSSAI as defined in TS 28.554 [2] clause 6.8.1.1

and DLRelPSR_N3.SNSSAI is PSR% in DL for N3 interface per SNSSAI as defined in TS          28.554 [2] clause 6.8.1.3

d) NetworkSlice

##### 6.8.1.Z	End to End Uplink Reliability KPI of an URLLC Network Slice

a)  E2ERelURLLC,PSR,UL

b) This KPI describes the end to end uplink Reliability based on Packet Success Rate (PSR) Percentage of an URLLC network slice. It is the multiplication of Packet transmission reliability KPI in UL on Uu interface and Packet transmission reliability KPI in UL on N3 interface. By definition, Reliability is a unidirectional performance metric in a network. i.e. The percentage value of the packets successfully delivered out of all the packets transmitted. It is a measure of the UL packet delivery success i.e. PSR% over a combined end to end interfaces of Uu and N3. It is a percentage value (%). This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

c)  Thus, KPI E2ERelURLLC,PSR,UL is given by:

E2ERelURLLC,PSR,UL  =  ULRelPSR_Uu.SNSSAI × ULRelPSR_N3.SNSSAI

Where,

ULRelPSR_Uu.SNSSAI is PSR% in UL for Uu interface per SNSSAI as defined in TS 28.554 [2] clause 6.8.1.2

and ULRelPSR_N3.SNSSAI is PSR% in UL for N3 interface per SNSSAI as defined in TS          28.554 [2] clause 6.8.1.4

d) NetworkSlice

End of changes