3GPP TSG-SA5 Meeting #153                    	S5-240807

# Sevilla, Spain, 29 Jan. - 02 Feb. 2024

Title:	LS on energy states in TS 22.261

Response to:

Release:	3GPP Rel-19

Work Item:	FS_Energy_OAM_Ph3

Source:	3GPP SA5

To:	3GPP SA1

Cc:	3GPP SA, 3GPP SA2

Contact Person:

#### Name:	Jean-Michel Cornily

E-mail Address:	jean <dot> michel <dot> cornily <at> huawei <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

In TS 22.261, the term ‘energy state’ is defined as ‘state of a cell, a network element and/or a network function with respect to energy, e.g. (not) energy saving states, which are defined in TS 28.310 [47]‘.

Furthermore, TS 22.261 clause 6.15a.3 (Support of energy states) captures requirements for the 5G system to:

- support different energy states of network elements and network functions

- support dynamic changes of energy states of network elements and network functions

SA5 has been working on energy saving related states for several releases (see energy saving definitions and concepts in TS 28.310 clauses 3.1 and 4.3) and has defined one energy saving related state attribute named energySavingState (see TS 28.541 clause 4.4.1), with two allowed values:

- isEnergySaving (state in which some functions of a cell or network function are powered-down),

- isNotEnergySaving (state when no energy saving in progress).

Therefore, SA5 would like to know whether the SA5-defined energySavingState attribute satisfies TS 22.261 clause 6.15a.3 requirements.

2. Reference documentation:

- 3GPP TS 28.310: Management and orchestration; Energy efficiency of 5G

- 3GPP TS 28.541: Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

3. Actions:

To SA1: SA5 kindly requests SA1 to reply to the above question and, if deemed necessary, to provide SA5 with any further clarification about TS 22.261 energy states.

5. Date of Next TSG-SA WG5 Meetings:

SA5#154	15 – 19 Apr., 2024	Changsha, China

SA5#155	May 27 – 31, 2024	Jeju, South Korea