# 3GPP TSG-SA5 Meeting #153	S5-240556

# Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to Draft CR 28.105 Add descriptions on data resources related to ML training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The inference results feedback is a general practice for ML entity training. This contribution is to add some description words about such sort of feedback, i.e., backflow data for optimizing performance through the collection of specific inferenece input and output data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add data backflow description in ML model training requested by consume. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No discriptions on additional backflow data from inference for ML training. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2a.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

## 6.2	Void

6.2a			ML training phase

6.2a.1		ML model training

6.2a.1.1		Description

In operational environment before the ML entity is deployed to conduct inference, the ML model associated with the ML entity needs to be trained (e.g. by ML training function which may be a separate or an external entity to the AI/ML inference function). The ML model training can be initial training or re-training of an already trained ML entity.

The ML model is trained by the ML training (MLT) MnS producer, and the training can be triggered by request(s) from one or more MLT MnS consumer(s), or initiated by the MLT MnS producer (e.g., as a result of model performance evaluation).

#### 6.2a.1.2	Use cases

##### 6.2a.1.2.1	ML model training requested by consumer

The ML training capabilities are provided by an MLT MnS producer to one or more consumer(s).

Figure 6.2a.1.2.1-1: ML training requested by MLT MnS consumer

The ML training may be triggered by the request(s) from one or more MLT MnS consumer(s). The consumer may be for example a network function, a management function, an operator, or another functional differentiation.

To trigger an initial ML training, the MnS consumer needs to specify in the ML training request the inference type which indicates the function or purpose of the ML entity, e.g. CoverageProblemAnalysis. The MLT MnS producer can perform the initial training according to the designated inference type. To trigger an ML re-training, the MnS consumer needs to specify in the ML training request the identifier of the ML entity to be re-trained.

The consumer may provide the data source(s) that contain(s) the training data which are considered as inputs candidates for training. To obtain the valid training outcomes, consumers may also designate their requirements for model performance (e.g. accuracy, etc) in the training request.

The performance of the ML entity depends on the degree of commonality between the distribution of the data used for training and the distribution of the data used for inference. As time progresses, the distribution of the input data used for inference might change as compared to the distribution of the data used for training. In such a scenario, the performance of the ML entity degrades over time. The MLT MnS producer may re-train the ML model associated to the entity if the inference performance of the ML entity falls below a certain threshold, which needs to be configurable by the MnS consumer. In addition, to optimize performance of ML re-training, the training data may also contain the backflow data sets (i.e., the labelled, unlabelled or pre-defined data sets derived from the specific inference input and output) produced by AI/ML inference function.

Following the ML training request by the MLT MnS consumer, the MLT MnS producer provides a response to the consumer indicating whether the request was accepted.

If the request is accepted, the MLT MnS producer decides when to start the ML training with consideration of the request(s) from the consumer(s). Once the training is decided, the producer performs the followings:

-	selects the training data, with consideration of the consumer provided candidate training data. Since the training data directly influences the algorithm and performance of the trained ML Entity, the MLT MnS producer may examine the consumer's provided training data and decide to select none, some or all of them. In addition, the MLT MnS producer may select some other training data that are available;

-	trains the ML model using the selected training data;

-	provides the training results (including the identifier of the ML entity generated from the initially trained ML model or the version number of the ML entity associated with the re-trained model, training performance results, etc.) to the MLT MnS consumer(s).

End of modifications