3GPP TSG-SA5 Meeting #153 	S5-240584

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0031 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.256 Addition of CHF information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHRACHF |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The terms used in the architecture is not in line with the inter-CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting the terms used, reference, and removing editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inter-CHF is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, and 6.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 4.2.2	Roaming

Figure 4.2.2.1 shows the 5G connection and mobility converged charging architecture in roaming service based representation:

Figure 4.2.2.1: 5G connection and mobility converged charging architecture in roaming

The architectural options of figure 4.2.1.1 apply with AMF located in VPLMN, V-CHF and corresponding Billing domain/CGF located in VPLMN.

The architectural options of figure 4.2.1.1 apply with AMF located in VPLMN, H-CHF and corresponding Billing domain/CGF located HPLMN. The Security Edge Protection Proxy (SEPP) specified in TS 23.501 [200] is used for inter-PLMN Nchf.

Figure 4.2.2.2 shows the 5G connection and mobility converged charging architecture in roaming with AMF to H-CHF, reference point representation:

Figure 4.2.2.2: 5G connection and mobility converged charging architecture in roaming with AMF to H-CHF - reference point representation

The N41 reference point is defined for the interactions between AMF and H-CHF.

The N42 reference point is defined for the interactions between AMF and V-CHF.

Figure 4.2.2.3, an alternative to Figure 4.2.2.2, shows the 5G connection and mobility converged charging architecture in roaming with inter-CHF (V-CHF to H-CHF) in reference point representation:

Figure 4.2.2.3: 5G connection and mobility converged charging architecture in roaming with inter-CHF - reference point representation

The N42 reference point is defined for the interactions between AMF and V-CHF.

The N107 reference point is defined for the interactions between V-CHF and H-CHF.

One or both of the architectures in Figure 4.2.2.2 and Figure 4.2.2.3 may be supported for local breakout roaming.

In case both architectures in Figure 4.2.2.2 and Figure 4.2.2.3 are supported for local breakout roaming, SMF and V-CHF determines which architecture should be selected for a roaming UE based on operator agreement.

Second change

#### 6.2.1.5	Definition of inter-CHF information

Specific charging information used for information when the V-CHF have a connection to the H-CHF.

The detailed structure of the inter-CHF Information can be found in table 6.2.1.5.1.

Table 6.2.1.5.1: Structure of Inter-CHF Information

| Information Element | Category | Description |
| --- | --- | --- |
| Remote CHF resource | OC | This field holds the reference to the Charging Data resource in the CHF not directly connected to the NF (i.e., H-CHF) e.g., the resource URI |
| Original NF Consumer Id | M | This field holds information on the NF triggering the request i.e., AMF |
| NF received CHF address | OC | This field holds the V-CHF address received from the NF i.e., AMF |

End of changes