3GPP TSG-SA5 Meeting #153	S5-240653

# Seville, Spain, 29th Jan 2023 – 2nd Feb 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0522 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Octet Measurements at gNB end |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The KPIs defined in 3GPP TS 28.554, clauses 6.3.2 and 6.3.3 for Upstream and Downstream throughput on N3 interface per network slice instance respectively, are only available for CN subnet slice consumer, since these KPI are using measurements which are calculated at UPFs (the end point of N3 interface contained in UPF). Hence from the RAN consumer perspective, there is no way for RAN to measure and monitor the throughput on N3 interface per RAN slice/slice subnet across RAN’s aggregation points, e.g., gNBs/CU-UPs. Therefore, RAN NSSMF is not able to determine how much RAN slice/slice subnet throughput on N3 is being served by constituent gNBs/CU-UPs for that slice. In addition, a consumer NSSMF in core network slice subnet may want to know throughput on N3 at RAN end i.e. gNB/CU-UP, but as currently this measurement is not standardized at RAN (gNB/CU-UP) end of N3 hence there is no standardized mechanism to provide the same to the core network slice subnet consumer. Further for better gNB/CU-UP capacity planning the performance metric is required to be calculated at gNB/CU-UP end of N3. Hence it is important to define these metrics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New measurements for octets of incoming/outgoing GTP data packets at gNB end of N3 interface are defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN consumer cannot know the throughput on N3 interface per RAN slice/slice subnet across RAN’s aggregation points, e.g., gNBs/CU-UPs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.1.1.X	Number of octets of incoming GTP data packets on the N3 interface,                          	           from UPF to RAN

a)	This measurement provides the number of octets of incoming GTP data packets on the N3 interface at gNB/CU-UP end, which have been generated by the GTP-U protocol entity on the N3 interface. The measurement can optionally be split into sub-counters per S-NSSAI.

b)	CC

c)	Reception of a GTP-U data PDU by the gNB/CU-UP on the N3 interface from UPF.

d)	Each measurement is a single integer value, the number of measurements is equal to one. If the optional S-NSSAI sub-counter measurements are performed, the number of measurements is equal to the number of supported S-NSSAIs.

e)	GTP.InDataOctN3gNB and optionally GTP.InDataOctN3gNB.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_NgU, EP_N3

g)	Valid for packet switching

h)	5GS

#### 5.1.1.Y              Number of octets of outgoing GTP data packets on the N3 interface,

from RAN to UPF

a)	This measurement provides the number of octets of outgoing GTP data packets on the N3 interface at gNB/CU-UP end, which have been generated by the GTP-U protocol entity on the N3 interface. The measurement can optionally be split into sub-counters per S-NSSAI.

b)	CC

c)  Transmission of a GTP-U data PDU by the gNB/CU-UP on the N3 interface to the UPF.

d)	Each measurement is a single integer value, the number of measurements is equal to one. If the optional S-NSSAI sub-counter measurements are performed, the number of measurements is equal to the number of supported S-NSSAIs.

e)	GTP.OutDataOctN3gNB and optionally GTP.OutDataOctN3gNB.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_NgU, EP_N3

g)	Valid for packet switching

h)	5GS

End of changes