3GPP TSG-SA5 Meeting #153 	S5-240670

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0514 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Network slice charging into generic business charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MATRIXX Software |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE_CH_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Network slice charging is one case of businss charging.The flows for Network slice charging need to be more generic to also cover other business converged charging based on 5G data connectivity |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update flows for Network Slice charging to generic business converged charging based on 5G data connectivity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Business converged charging not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.2.21	Business charging based on 5G data connectivity

##### 5.2.2.21.1	General

The clause below describes business charging based on PDU session charging, using the C-CHF and B-CHF architecture defined in clause 4.3.

##### 5.2.2.21.2	Business charging based on 5G data connectivity

The clause describes the high level charging procedure for business converged charging based on 5G data connectivity with PDU session SCUR scenario.

The following figure 5.2.2.21.2-1 describes a UE PDU session charging establishment for business converged charging based on 5G data connectivity

Figure 5.2.2.21.2-1: business converged charging based on 5G data connectivity – UE PDU session

- Business dependent charging informationis configured in - C-CHF and B-CHF.

- According to clause 5.2.2.2.2 PDU Session charging from SMF about PDU session establishment for individual UE PDU session charging.

2ch-a.	The SMF creates a Charging Identifier for the PDU session, and sends Charging Data Request [Initial], triggered by "Start of PDU session" to C-CHF.

2ch-b.	The CHF opens CDR for this PDU session per UE.

2ch-c.	Business charging based on 5G data connectivity is enabled.

2ch-d.  C-CHF sends the Charging Data Request [Initial] to B-CHF with UE PDU session charging information received from SMF which includes business specific identifier, and may include Tenant ID. Account, Rating and reservation control for the Tenant.

2ch-e.	 The B-CHF opens or update B-CDR for this business specific identifier with the PDU session.

2ch-f.	 B-CHF provides appropriate Charging Data Response [Initial] for Tenant.

2ch-g.	C-CHF provides appropriate Charging Data Response [Initial] for the individual UE.

- During the PDU session, when enable triggers is armed, the Charging Data Request/Response [Update] is send between SMF and C-CHF for individual UE, between C-CHF and  B-CHF for the tenant.

- According to clause 5.2.2.2.2 PDU Session charging about PDU session modification from SMF for individual UE charging.

4ch-a.	The SMF sends Charging Data Request [Update], upon triggers for PDU session modification  to C-CHF.

4ch-b.	The CHF updates CDR for this PDU session per UE.

4ch-c.	Business charging based on 5G data connectivity is enabled.

4ch-d. C-CHF sends the Charging Data Request [Update] to B-CHF with PDU session charging information received from SMF which includes business specific identifier, and may include Tenant ID.

4ch-e.	 The B-CHF updates B-CDR for this business specific identifier with the PDU modified session.

4ch-f.	 B-CHF provides appropriate Charging Data Response [Update].

5. PDU session modification continuation

6. According to clause 5.2.2.2.2 PDU Session charging about PDU session release from SMF for individual UE charging.

6ch-a.	The SMF sends Charging Data Request [Termination], triggered by "End of PDU session" to C-CHF.

6ch-b.	The C-CHF closes CDR for this PDU session per UE.

6ch-c.	Business charging based on 5G data connectivity is enabled.

6ch-d. C-CHF sends the Charging Data Request [Termination] to  B-CHF withPDU session charging information received from SMF which includes business specific identifier, and may include Tenant ID.

6ch-e.	 The B-CHF updates or closes B-CDR for this business specific identifier .

6ch-f.	  B-CHF provides appropriate Charging Data Response [Termination] for Tenant.

6ch-g.	C-CHF (UE CCS) provides appropriate Charging Data Response [Termination] for the individual UE.

Editor’s note: flow description is ffs.

End of changes