3GPP TSG-SA5 Meeting #153	S5-240428

# Sevilla, Spain, 29th January- 2nd February 2024

# Source:	Nokia, Nokia Shanghai Bell, Ericsson

# Title:	Rel-18 pCR TS28.319 OpenAPI for REST Solution for Access control for management service

# Document for:	Approval

Agenda Item:	6.6.1.3

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

None

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.319

Introduction of Stage 3 with respect to REST and OpenAPI solution set which is mapped to the Stage 1 and stage 2 submitted in meeting #152

Begin of modifications

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[xx]	IETF RFC 6749: " The OAuth 2.0 Authorization Framework".

[yy]	IETF RFC 8341: " Network Configuration Access Control Model".

[zz]	Management and Orchestration APIs Stage 3 Repository https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA102/

# 4 Concepts and overview

## 4.1	Concepts

### 4.1.1	Identity and Access Control

Network Management systems are becoming a challenge to manage in terms of the users of the system as well as the access control that needs to be applied continuously on a need-to-know basis.

Flexibility and agility to adapt to these growing needs is the key for a sustainable identity and access control implementation.

The service based architecture needs to factor seamless integration to any system for authentication and authorization of Management Service (MnS) consumers.

The need to make continuous access control related changes should be supported with ease and simplicity.

Today we have various human users, machine users and various resources that are continuously growing with complexities below:

- Human users seem to require various levels of access control.

- Machine type of communication in the direction towards automated systems require another type of access and may not necessarily be user name password based.

- Resources and modules themselves require various level of clearance depending on the sensitivity of the data being accessed.

The identity and access control system should be designed for role based access control. Also the authorization rules may support the fine grained permissions i.e. based on MnS component A, B and C.

### 4.1.2  	Role based access control

This is based upon the concept of assigning the appropriate permissions and privileges to authorized users. The combinations of the permissions and privileges make up a role. The users who belong to a role should be assigned to resources based on a least privilege principle and access rules associated to the resource. The principle of least privileges states that the users should be granted access only to the data and the operations that are required to perform the job. This minimizes the possibility of a security breach.

The management system should be setup for access control by a system administrator who will have the know-how of creating the required users and roles. Roles will use the various access rules. Additionally, the system administrator will also need to setup the access rules for a MnS producer.

Roles could be in various categories like full access and restricted access.

Figure 4.9.x.1.1-y - role based access

Figure 4.9.x.1.1-y - role based access

The Figure 4.9.x.1.1-y - role based access shows how a user is assigned to a role. The roles in turn are able to act upon a resource with the required operation on a MnS Producer. The operations that can be performed are defined by the access rules.

The Figure 4.9.x.1.1-z shows an example of how role based access control is adapted to service based management architecture. In the service based management architecture, the user is regarded as MnS consumer, and the combination of resource and operation is represented by management services. Different MnS consumers can be assigned to one role or different roles. The access rules for roles will be configured according to the MnS consumer access right of management service instances which are composed by different MnS component A and component B or component C.

Figure 4.9.x.1.1-z Example of role based access control of management services

MnS consumers belonging to different domains (e.g. NOP domain and Vertical domain) are assigned to different roles respectively. For each role, there is an associated access rule list which shows the allowed accessing scope of MnSs. MnS 1, MnS 2 and MnS 3 are different with each other in at least one of the component A, component B and component C.

### 4.1.3	Entities in access control

In a distributed system the responsibilities of authentication and authorization is split between several entities (Figure 4.1.2.-1):

Figure 4.1.2.-1: Entities in access control

The above figure shows the various entities that need to be involved in setting up and using access control. Irrespective of any implementation and functional split, the concept of access control is based on the following steps:

- The resource server needs to relate to the request coming from the user to a known identity

- The resource server needs to relate to the request coming from the user to resource, which in case of SBMA means a combination of MnS components as of TS 28. 533, i.e. MnS operations (component A), object instances or classes from the NRMs (component B), and alarm information or performance data (component C).

- The resource server needs to check whether the requesting identity is allowed to perform the requested operation on the requested resources. These checks can be assigned to a functional block for authentication and a block for authorization, no matter whether these functional blocks are internal parts of a management function (e.g. as part of a Netconf server) or whether these blocks are visible as standalone services (e.g. like a dedicated OAuth authorization server).

- As precondition for any access control and irrespective of any implementation or deployment, the owner of the resources needs to configure these functional blocks in order to define which identity is allowed to access which resources.

Access control is highly specific to the solution set, the protocol and the CRUD operations defined by the solution underlying the solution set. For example, oAUTH offers the possibility of the authentication and authorization function integrated and Netconf has the possibility of authentication taking place separately on the transport layer and the authorization separately on the Netconf server. Netconf will use standardized RFC 8341 Network Configuration Access Control Model (NACM).  REST solutions with OpenAPI will use OAUTH2.0.

The different entities need to have a common notion of the security-related parameters. E.g. Authentication and authorization function and potential user need to use the same notion of identity, as well as authentication and authorization function and potential resource server need to use the same notion of resource. Therefore the involved entities need to be based on an overarching security-related information model(explained in next section related to Information Model), which is the basis for the concrete data models of the security-related interfaces that are needed to fulfil the security-related use cases of access control

## 4.2    Usecases

### 4.2.1	 Authentication and authorization

- Authorization function and resource server need to have a common understanding of resources and refer to the same resources and corresponding actions. The access control information needs to be provisioned in the authentication and authorization function.

- To carry out authentication and authorization based on role-based access the following tasks needs to be in place.

- Pre-deployment task – This relates to the identification of the resources represented by the MnS component B and C and it associated operations represented by MnS component A. This is typically done by a Network Equipment Provider (NEP) during the design phase.

- Post deployment task – this relates to the set of administrative tasks which are requires to enable role-based access control typically caried out by a network operator (NOP). This is done once the system is up and running and access control needs to be administered.

NOTE:     The NOP may have a system administration department in charge of defining and configuring role-based access control.

Post the above tasks we have the possibility to have role-based access in operation with every service call being authenticated and authorized.

When an MnS is invoked, the MnS consumer authenticates towards an authentication service producer. This is then followed by checking the access rights with respect to the role-based access. This takes place in the authorization service producer which is invoked from the MnS producer to check if its resources can be accessed with respect to the related operation.

### 4.2.2	 Identity to roles association

The identity or user of the system needs to be assigned the permissions to carry out the management operations. A set of permissions is defined as a role. An efficient and flexible means is to define various roles by a network operator. In a second step, association of the roles to an identity (entity or human user) is carried out.

### 4.2.3	 Roles  associated to resources (consisting of IOCs(),MOIs() and corresponding operations

Role-based access control defines roles. Roles are associated to access rules which are combinations of resources and operations (e.g., CRUD operations). The resources can be represented by IOCs(static) and/or MOIs (dynamic). Hence the resources in context are typically Component B and the operations are Component A.

Component C which represents that management data is also a possible resource to be access control protected.

Note: Whether and how component C will be access controlled is FFS.

# 5   Requirements

- UC-MSAC-0x - 3GPPP management system shall support role-based access control for the resources represented by the MnS component A, B and C.

- UC-MSAC-0x.01- 3GPP management system shall support authentication and authorisation for management services.

- UC-MSAC-0x.02 - 3GPP management system shall support identity to role assignment.

- UC-MSAC-0x.03 - 3GPP management system shall support roles which are associated to resources (IOCs() and/or MOIs()) and the corresponding operations.

# 6 Solution description

## 6.1 Information model

Editor’s note: the management of the classes defined in the information model is for FFS

## 6.2    Information architecture

This clause elaborates the information architecture of the classes necessary to enable the role based access control explained in the section above. This section elaborates the concept into further logical classes that need to interact to enable the design.

The classes, attributes specified here shall be seen as concepts that might not have a direct mapping in some solution sets.

@startuml #152 information model for access control

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide empty members

hide circle

class Identity

class Role

Identity "*" -d- "*" Role

Role "*" -d- "*" "Access Rule"

class  "Access Rule" {

resources

operation

action

componentCdata

}

@enduml

Figure 5.1-1 Information model for role based access control relationship

The information model above depicts the following to realise a role based access control relationship.

- One Identity could be associated to one or more Roles.

- One Role might be associated to one or more Access Rules. One Role might be associated to one or more Identities.

- The Access rule class allows protecting resources, by specifying which permissions are eligible for each resource. A resource is identified by the class name or class instance. The permission specifies which operation (identified by a CRUD) is applicable and possibly which action (allow or deny) is applicable".

The classes represent the information architecture that is necessary to implement an access control system. They are and not meant to be provisioned and managed like the NRM classes.

This clause provides an overview of the relationships between relevant classes in UML format.

As mentioned in the usecases the classes are provisioned during the integration time by a network operator. The data is sent to an authentication and authorization service producer.

Post this during integration time when a MnS consumer invokes an operation on the MnS producer, the authentication and authorization service producer validates the action on the resource to enable the decision for the MnS producer. The decision could be that the MnS producer allows or disallows the action on the resource.

The information architecture translates to the design which considers the below

- The authentication function mainly contains the ‘who’ of the MnS consumers. The authorization function contains the information of the resource and the action of the MnS producer associated to the ‘who’ of the MnS consumers. These functions could be collocated or distributed.

- The MnS consumer is associated to ‘who’ is carrying out the operation and has to be known to the resource owner who will provide this information. The MnS consumer interacts with an authentication function to identify itself.

- The MnS consumer interacts with the authorization function whether it can carry out the action on the resource. The resource owner also provisions the information associating various resources and corresponding actions to valid MnS consumers as a pre-step.

- The MnS consumer interacts with the MnS producer after getting responses from the authentication and authorization functions. The MnS producer further does a validation if the action on its resource can be allowed or not.

The above interactions can be realized in any implementation.

# 7	Model

## 7.1	Class diagrams

Below are the classes that need to interact in order to store the data required for the authentication and authorization operations with respect to role based access control.

@startuml #152 classes and fields

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

'hide members

hide empty members

hide circle

'skinparam maxMessageSize 250

class Identity

{

String identityName

String identityType

String credential

}

class Role

{

String roleName

List accessRules

}

class "Access Rule" {

String ruleName

String resources

List operations

List actions

String componentCdata

}

Identity "*" -down- "*" Role

Role "*" -down- " *"  "Access Rule"

@enduml

Figure 5.2-1 Classes for role based access control name containment

The classes related to Identity, Role,  Access rules are contained within the authentication and authorization service producer.

## 7.2	Class definitions

### 7.2.1	Identity

#### 7.2.1.1	Definition

This class represents an identity of a MnS consumer.

It is used for authentication and authorization.

The MnS consumer can be a human or a machine user. This class enables the creation and storage of an identity of a MnS consumer. The information in this class is the starting point for a MnS consumer to identity who it is. This is validated against an authentication service producer.

For the authentication operation to take place the identity related information has to be provisioned into the system by a network operator who could be an administrator. The administrator adds the identityType attribute and identityName attribute which characterizes a machine user or human user respectively. For example, an operator might have an identity like a tenant mapped to the relevant list of roles.

Attribute credential is used to provide information for the credential used together with identity when requesting authentication. The examples of credential are password, certificate, biometric, etc.

The roleList attribute defines the role names associated to a particular Identity.

The class stores the details of the expected tasks to be performed by an identity. The tasks are what is to be done on the network management system. To ease the administration on the system, the tasks are organised as roles. The user can be associated to one or more roles.

#### 7.2.1.2	Attribute

The Identity class includes the following attributes:

| Attribute Name | S |
| --- | --- |
| identityType | M |
| identityName | M |
| credential | O |
| Attributes related to role |  |
| roleList | M |

#### 7.2.1.3	Attribute constraints

None

### 7.2.2	Role

#### 7.2.2.1	Definition

The Role class represents a task or collection of tasks in a network management system.

The Role class enables the storage of information as to what resources and actions an identity can work upon. This class maintains the resources that are known to the management system. This contains all the granular level resources and the corresponding actions.

The roleName attribute defines the name of a role.

The accessRulesList attribute contains a list of access rules that contain the list of granular permission sets.This could be the possible order in which the access rules are considered by the MnS producer.

#### 7.2.2.2	Attribute

| Attribute Name | S |
| --- | --- |
| roleName | M |
| Attribute related to role |  |
| accessRulesList | M |

#### 7.2.2.3	Attribute constraints

None

### 7.2.3	AccessRule

#### 7.2.3.1	Definition

The AccessRule class represents the granular resource and actions in a network management system on which an action has to be performed.

This class enables the storage of the resource types in the system and the possible actions that are allowed on it. The permutations and combinations of these permissions are assigned to a role.

The ruleName attribute binds the instances in the network as well as the permissions and the operations allowed upon it.

The resources attribute defines the resources. The resources are classes(IOC) or instances of classes(MOI) in the network that need to be access controlled. The resources define the root instances or the leaf instances. For example, the Managed Element could be the root object and the attributes could be referred to as the leaf objects. The resources here could be whole classes or specific instances of classes with a known DN value or could be an expression(eg: XPATH or JEX) that could be resolved by the producer to get the nodes at runtime.

Examples of the resources attribute value could be as below. Please note this is not an exhaustive set of examples and shown for depiction purpose.

  - Values related to IOC

    - Description: this means that

      - all attributes of an IOC are eligible for the access rule

      - at operation time, all instances of this IOC are eligible for the access rule.

    - Examples:

      - IOC name		: "ManagedElement"

      - Expression resolving to IOCs under a subnetwork SN1 :

"/SubNetwork[id="SN1"]/ManagedElement"

  - Values related to one or more instances of an IOC

    - Description: this means that

      - all attributes of the IOC are eligible for the access rule

      - at operation time, only the specified instances of this IOC are eligible for the access rule.

    - Examples:

      - Specific instance of IOC name	: "SN1/ME1"

  - Values related to one or more IOC attributes

    - Description: this means that

      - only the specified attributes of the IOC are eligible for the access rule

      - at operation time, all attributes of the instances of this IOC are eligible for the access rule.

    - Examples:

      - Attribute name: "SubNetwork/ManagedElement/vendorName"

      - Expression resolving to specific instance of attribute name "/SubNetwork[id="SN1"]/ManagedElement[id="ME1"]/attributes[vendorName="Company XY"]"

      - Specific attribute instance: "SN1/ME1/vendorName=’Company XY’"

  - Any combination between a-c.

The operations attribute define the list of operations that are permitted on the  resources value encompassed under this ruleName.

The actions is an optional attribute which specifies whether the operation allows to permit all or deny all and maybe used depending on the solution set.

The componentCData is an optional attribute which specifies the management data to be allowed or not.

#### 7.2.3.2	Attribute

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| resources | M |
| operations | M |
| actions | O |
| componentCData | O |

#### 7.2.3.3	Attribute constraints

None

Editor’s note: Additional example to elaborate Identity, Role, Access rules is for FFS

## 7.3	Attribute definitions

### 7.3.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| identityType | This indicates a type of identifier AllowedValues: username, email address, phone number, IP address, machineuser | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| identityName | This defines a readable string to uniquely represent an identity AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| credential | The credential of an MnS consumer or producer used for authentication with authentication service producer. It could be password, certificate, key, pass phrase, etc., based on authentication protocol and factor. AllowedValues: NA | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No value isNullable: False |
| roleList | This defines the list of roles associated with an identity AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| roleName | This string defines a unique representation of the name of a role AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| accessRuleList | This defines the list of access rules associated with a role AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ruleName | This string defines a unique representation of the name of an access rule. The name of the access rule could also contain the name of the management service AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| resources | This attribute defines a MnS component type B represented by information models of managed entities. A MnS component type B is also called Network Resource Model (NRM). MnS component type B examples are: 1) Network resource models as defined in TS 28.622 2) Network resource models as defined in TS 28.541 A resource might be: * AnIOC or MOIs * A specific instance of an IOC, * An attribute or attributes * Specific instance of attribute or attributes AllowedValues: NA | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| operations | This defines the Component A related operations for resources. The operations related to attributes are also contained in this set. The operations are of the MnS as defined in 28.532 AllowedValues: NA | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| actions | This defines whether the operation is allowed or denied on the operation AllowedValues: allow, deny | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| componentCData | This attribute defines a MnS component type C which is the performance management data, KPIs of the managed entity and fault information of the managed entity. The following are examples of Management service component type C: 1. Alarm information as defined in TS 28.532 and TS 28.545 2. Performance data as defined in TS 28.552 , TS 28.554 and TS 32.425. AllowedValues: NA | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

# 8	Operations

Once the data is provisioned in an authentication and authorization function it has to be utilised by an MnS consumer and MnS producer. Hence we need to define the below functions which provide information to the MnS producer as to who is accessing which resources.

## 8.1	authentication operation

### 8.1.1	Description

This operation is invoked by MnS consumer to request the authentication service producer to authenticate MnS consumer. MnS consumer and authentication service producer shall use one of the following methods for authentication:

-	Mutual authentication between MnS consumer and authentication service producer provided by the transport layer solution. Mutual TLS is the concept where the client and the server authenticate each other in this case we have the MnS consumer authenticating itself.

-	Client credentials assertion based authentication.

When client credentials assertion based authentication is used, the operation is invoked by a MnS consumer to request authentication by authentication service producer. The authentication service producer gets the identifier and credential of the MnS consumer for validation. Based on identifier in the request, authentication service producer gets identity information, e.g. credential of the identity, associated role(s) of the identity, etc., from local data store (e.g., a directory). The authentication service producer authenticates the MnS consumer by validating the identity information. associated to the role(s) the MnS consumer belongs to.

If authentication is successful, the authentication service producer sends successful response with assertion to the MnS consumer. MnS producer validates the assertion to authenticate a MnS consumer.

If fail for authentication, the authentication service producer sends failure response to the MnS consumer.

### 8.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| identityName | M | String | This parameter specifies the identity of the MnS consumer that is to be authenticate. |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the identity to be authenticated. |

### 8.1.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the authenticated MnS consumer and the actual value assigned to each. |
| Status | M | ENUM (OperationSucceeded, OperationFailed) |  |

### 8.1.4	Exceptions and constraints

| Exception Name | Definition |
| --- | --- |
| operation_failed | Condition: Operation is failed Returned Information: The output parameter status Exit state: Entry State |

## 8.2	authorization operation

### 8.2.1	Description

The operation is used to authorize a MnS consumer by the authorization service producer.

MnS consumer, MnS producer and authorization service producer shall use one of the following methods for authorization:

-    Token based authorization framework including various grant modes (e.g., as specified in RFC 6749)

-	Static authorization as specified (e.g. in RFC 8341 for NACM)

When token based authorization framework is used, authorization service producer receives authorization request, gets the assertion of the MnS consumer related to the authentication and probably other context information (e.g. resource , action) from the request. If access token is supported by the MnS producer and consumer:

-  The MnS consumer gets access token from authorization service producer by providing the assertion from authentication session.

-  The MnS consumer starts normal operation with MnS producer with the access token. The MnS producer validates the token. If the token is valid, the MnS producer performs the action(operation) on the resource authorized and returns result to the MnS consumer.

If access token is not supported by the MnS producer and consumer:

-  When normal operation starts, the MnS producer validate the authentication assertion and check permission of the MnS consumer with authorization service producer.

-  The MnS producer performs the operation on the targeted resource which is part of the authorized scope and returns result to the consumer if the MnS request is allowed according to permissions.

If fail for authorization, the authorization service producer sends failure response to the consumer.

### 8.2.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the identity to be authorized. |

### 8.2.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the authorization for the MnS consumer and the actual value assigned to each. |
| Status | M | ENUM (OperationSucceeded, OperationFailed) |  |

### 8.2.4	Exceptions and constraints

| Exception Name | Definition |
| --- | --- |
| operation_failed | Condition: Operation is failed Returned Information: The output parameter status Exit state: Entry State |

# Annex A (normative): Solution sets

## A.1	RESTful HTTP-based solution set

### A.1.1	Mapping of the operations

Start of modification

### A.1.2	OpenAPI definitions

OpenAPI definitions for the NRM are specified in Forge [zz].

Directory: OpenAPI

Files:

OpenAPI/TS28319_MsacNrm.yaml

The below OpenAPI  is the schema which will be used for provisioning inputs to the oAuth server.

openapi: 3.0.1

info:

title: MSAC NRM

version: 18.0.0

description: >-

OAS 3.0.1 definition of the MSAC NRM

Â© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.319; MSAC NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.319/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

Identity-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

identityType:

type: string

enum:

- USERNAME

- EMAIL_ADDRESS

- PHONE_NUMBER

- IP_ADDRESS

- MACHINEUSER

identityName:

type: string

credential:

type: string

roleRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

Role-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

roleName:

type: string

accessRulesList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

AccessRule-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

type: object

properties:

ruleName:

type: string

resources:

type: string

operations:

type: array

items:

type: string

actions:

type: string

enum:

- ALLOW

- DENY

componentCData:

type: array

items:

type: string

resources-msacNrm:

oneOf:

- $ref: '#/components/schemas/Identity-Single'

- $ref: '#/components/schemas/Role-Single'

- $ref: '#/components/schemas/AccessRule-Single'

End of changes

# Annex Z (Informative):

## Z.1	Informative sequence diagram: Commissioning flow to be carried out by an administrator:

The sequence diagram below explains the IOCs to be used as a pre-requisite for enabling the role based access control. The notes above in the diagram also depict an example of each of the tasks.

The sequence diagram below explains the operations and the interactions with the authentication and authorization service which typically should suit any implementation be in oAUTH or a NACM implementation.

The notes along the flows show the examples related to the IOCs provisioning operations.

Editors note: the sequence diagram update will be FFS

Figure Z.1 - Sequence of commissioning for role based access control

### Z.1.1 Commissioning usecases to be carried out by an administrator

- To carry out authentication and authorization based on role based access a set of tasks needs to be in place at the commissioning phase.

The network equipment vendor will typically take care of the below:

    - The granularity level at which the management objects need to be authorized

    - The actions that need to be permitted on these granular level objects

This is typically carried out by an administrator who has to setup the system for operational purposes which included authentication and authorization for every operation. Hence the administrator will need to have a good view of the permissions of the devices being integrated and managed. The administrator will need to know the possible scope of action for various users in the system on the instances that are created. The administrator will need to know the roles that need to be created in the system as well as the mapping of the same to the users. Below is the detail of every task.

The below elaborates the sequence flow to depict the IOCs involved for provisioning.

#### Z.1.1.1 Permissions with resources and actions

- Permissions are combinations of static resources and actions. The network equipment vendor can introduce the resources and actions at various levels of granularity as well as in the language that can be suitable for the vendor. When the commissioning phase happens it is important to translate these permissions to the authentication and authorization system so that it is aware of the static resources and corresponding actions on them for access control.

Hence the IOC for PermissionForMnSs is required to create the necessary data.

#### Z.1.1.2 Role creation with association to permissions and conditions

The meaningful task is the role which defines the responsibility realm for the user. Hence the role contains the overall distribution of resources and actions it can work on by resolving the condition and permissions. The static resources identified need to be combined with the dynamically created instances of the resource in order to be assigned to a meaningful task in the system.

The roles are typically created according to the convenience and hierarchy of the organisation.  Coarse grained or fine grained permissions can be created with the help of roles.

The management system should be setup for access control by a system administrator who will have the know how of creating the required users and roles. Roles will use the various possibilities to map resources and actions.

Hence the IOC for Role is required to create the necessary data related to various roles in the system.

#### Z.1.1.3 Identity to role assignment

The identity or user of the system needs to be assigned the responsibilities to carry out the OAM operations. Hence the set of responsibilities are defined in various roles. Assignment of the roles to an identity is required to make the administrative task more efficient and flexible. This also shields the administrator from getting into the details of the know how of granular resources and actions relevant to it.

Hence the IOC for Identity is required to create the necessary data.

## Z.2 Informative sequence diagram: Authentication and Authorization with role based access control:

Post the above tasks we have the possibility to have role-based access in operation with every service call being authenticated and authorized.

The below sequence diagram shows how the operations are invoked from a consumer towards an authentication and authorization service producer.

When an MnS is invoked, the MnS consumer authenticates towards an authentication service producer. This is then followed by checking the access rights with respect to the role-based access. This takes place in the authorization service producer which is invoked from the MnS producer to check if its resources can be accessed with respect to the related operation.

Hence then need of a authentication and authorization service producer which will validate a user who want to access the resources as well as validates what the user can access.

Figure Z.2 - Sequence diagram for operations of authentication and authorization