3GPP TSG-SA5 Meeting #153 	S5-240620

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0514 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 TS 28.552 Add new measurements for UL CI Time Domain Proportion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Asiainfo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | URLLC_Mgt |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At present, the network resource load is mainly evaluated through resource usage-related measurements. Refering to TS 28.552, the evaluation measurements are mainly PRB usage rate-related measurements, which measures usage (in percentage) of physical resource blocks (PRBs). Although these measurements can evaluate the overall resource load of the cell, they cannot effectively evaluate the resource load of the URLLC service under the eMBB and URLLC multiplexing scenarios. For example, in a statistical time period, the PRB usage rate of the network is low. Because the URLLC service has high requirements for delay sensitivity, it needs to be transmitted immediately. At this time, on the small number of scheduled resources of the overall network resources, the URLLC service has data transmission requirements. But on these few scheduled resources, the resource requirements of URLLC services cannot be meet, so the EMBB service resources are preempted. In this case, since the PRB usage rate only reflects the overall resource load of the cell, it cannot reflect the situation that the resources of the URLLC service are insufficient at this time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New measurements UL CI Time Domain Proportion should be added to better evaluate the UL resource load of URLLC services under multiplexing scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing PRB usage rate related measurements for evaluating network resource load cannot effectively evaluate the resource load of URLLC services under eMBB and URLLC multiplexing scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 5.1.1.2.X UL CI Time Domain Proportion

a)	This measurement provides the proportion of time domain resources that invoke the cancellation indication (CI) feature in the statistical period. Taking a fixed time duration as one sampling occasion, the numerator of this measurement is the number of sampling occasions that invoke the CI feature (when the number of cancelled PRBs is greater than 0) and the denominator is the number of sampling occasions with UL data scheduled (eMBB, URLLC, etc.).

b)	SI

c)	This measurement is obtained as: PCIT=CITNUTT*100,

where

PCI(T) denotes the proportion of time domain resources that invoke the CI feature during the time period T, with value range: 0-100%;

CI(T) is the number of sampling occasions that invoke the CI feature during the time period T,  CI(T)=iCIi(T);

CIiT is the invoking CI feature result of sampling occasion i,  when the number of physical resource blocks (PRBs) that invoke the CI feature at sampling occasion i is greater than 0, CIi(T) = 1, and when the number of PRBs that invoke the CI feature at sampling occasion i is equal to 0, CIi(T) = 0;

NUT(T) is the number of sampling occasions with UL data scheduled (eMBB, URLLC, etc.) during the time period T, NUT(T)=iNUT,i(T)；

NUT,i(T) is the UL data scheduled result of sampling occasion i, when there is UL data scheduled at sampling occasion i, NUT,i(T)=1, and when there is no UL data scheduled at sampling occasion i, NUT,i(T)=0;

NOTE: UL data scheduled is scheduled data of user plane, such as eMBB data, URLLC data, etc.

T denotes the time period during which measurement is performed;

i denotes sampling occasion (e.g. 1 slot) during time period T.

d)	A single integer value from 0 to 100.

e)	RRU.CiUtUl, which indicates the proportion of time domain resources that invoke the CI feature

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for evaluating the resource load of URLLC services under eMBB and URLLC 		multiplexing scenarios.

End of Change