# 3GPP TSG-SA5 Meeting #153	S5-240813

# Sevilla, Spain, 29 January - 02 February 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:		Input to DraftCR 28.105  Information on Learned capabilities

# Document for:	Approval

# Agenda Item:	6.4.3.1 - AIML_MGMT_WoP#1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	S5-237244 DraftCR : “Draft CR AIML_MGMT - TS 28.105; Enhancements for AI-ML management”.

# 3	Rationale

After the training has converged, the MnS consumer may be interested in observing what has been learned by ML training function, e.g., to know when and where to use the ML entity. This pCR adds a requirement for the MnS consumer to be informed about the learned capabilities.

# 4	Detailed proposal

Start of First change

## 6.2	ML training phase

### 6.2.1	ML model training

##### 6.2.1.2.4	Managing ML training processes

This machine learning capability relates to means for managing and controlling ML model training processes.

To achieve the desired outcomes of any machine learning relevant use-case, the ML model applied for such analytics, needs to be trained with the appropriate data. The training may be undertaken in a managed function or in a management function.

In either case, the network (or the OAM system thereof) not only needs to have the required training capabilities but needs to also have the means to manage the training of the ML models. The consumers need to be able to interact with the training process, e.g., to suspend or restart the process; and also need to manage and control the requests related to any such training process.

The ML training MnS producer should support means to write into the ML Entity or provide to the MnS consumer a information indicating what has been learned by the ML Entity through the training process.

Next change

#### 6.2.1.3	Requirements for ML training

Table 6.2.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to request ML training. | ML training requested by consumer (clause 6.2.1.2.1) |
| REQ- ML_TRAIN-FUN-02 | The MLT MnS producer shall have a capability allowing the authorized MLT MnS consumer to specify the data sources containing the candidate training data for ML training. | ML training requested by consumer (clause 6.2.1.2.1) |
| REQ- ML_TRAIN-FUN-03 | The MLT MnS producer shall have a capability allowing the authorized MLT MnS consumer to specify the inference type of the ML entity to be trained. | ML training requested by consumer (clause 6.2.1.2.1) |
| REQ- ML_TRAIN-FUN-04 | The MLT MnS producer shall have a capability to provide the training result to the MLT MnS consumer. | ML training requested by consumer (clause 6.2.1.2.1), ML training initiated by producer (clause 6.2.1.2.2) |
| REQ- ML_TRAIN-FUN-05 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML entity. (See Note) | ML training initiated by producer (clause 6.2.1.2.2) |
| REQ- ML_TRAIN-FUN-06 | The MLT MnS producer shall have a capability to provide the version number of the ML entity and the time when it is generated by ML re-training to the authorized MLT MnS consumer. | ML training requested by consumer (clause 6.2.1.2.1), /ML training initiated by producer (clause 6.2.1.2.2) |
| REQ- ML_TRAIN-FUN-07 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML training. | ML training requested by consumer (clause 6.2.1.2.1), ML training initiated by producer (clause 6.2.1.2.2), ML entity joint training (clause 6.2.1.2.6) |
| REQ- ML_TRAIN-FUN-08 | The MLT MnS producer should have a capability to provide the grouping of ML entities to an authorized MLT MnS consumer to enable coordinated inference. | ML entity joint training (clause 6.2.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The MLT MnS producer should have a capability to allow an authorized MLT MnS consumer to request joint training of a group of ML entities. | ML entity joint training (clause 6.2.1.2.6) |
| REQ- ML_TRAIN-FUN-10 | The MLT MnS producer should have a capability to jointly train a group of ML entities and provide the training results to an authorized consumer. | ML entity joint training (clause 6.2.1.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to discover the characteristics of available ML entitiesincluding the contexts under which each of the models associated with the entities were trained. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to select an ML entity. | ML models and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-03 | The MLT MnS producer shall have a capability to enable an authorized MLT MnS consumer to request for a model to be trained to satisfy the consumer's expectations. | ML training requested by consumer (clause 6.2.2.1), ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-04 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to request for information and be informed about the available alternative ML entities of differing complexity and performance. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-05 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to request one of the known or available alternative models of differing complexity and performance to be used for inference. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-06 | The 3GPP management system shall have a capability to provide a selected ML entity to the authorized MLT MnS consumer. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_TRAIN- MGT-01 | The MLT MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML training, e.g. to modify the characteristics of the request or to delete a request. | ML training requested by consumer (clause 6.2.2.1),Managing ML Training Processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-02 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training; or to adjust the training conditions and/or characteristics. | ML training requested by consumer (clause 6.2.2.1), Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer (e.g. the function/entity different from the function that generated a request for ML training) to request for a report on the outcomes of a specific training instance. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the MLT function to report to any authorized MLT MnS consumer about specific ML training process and/or report about the outcomes of any such ML training process. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an MLT function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML-inference function. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall enable a producer of data services (e.g. a gNB) to provide to an authorized consumer (e.g. an MLT function) a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the inference function. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_VLD-01 | The MLT MnS producer should have a capability to validate the ML entities during the ML training process and report the performance of the ML entities on both the training data and validation data to the authorized consumer. | ML entity validation performance reporting (clause 6.2.1.2.7) |
| REQ-ML_VLD-02 | The MLT MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML training and validation process. | ML entity validation performance reporting (clause 6.2.1.2.7) |
| REQ- ML_TRAIN- MGT-06 | the MLT MnS producer should have a capability enabling an authorized MnS consumer to receive information on the capabilities that have been learned by the ML Entity. | Managing ML training processes (clause 6.2.1.2.4) |
| NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML entity supports) of ML entity. |  |  |

Next change

##### 7.3a.1.2.3	MLTrainingReport

###### 7.3a.1.2.3.1	Definition

The IOC MLTrainingReport represents the ML model training report that is provided by the training MnS producer.

The MLTrainingReport MOI is contained under one MLTrainingFunction MOI.

###### 7.3.1.2.3.2	Attributes

Table 7.3a.1.2.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable |  | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- | --- |
| mLEntityId | M | T | F |  | F | T |
| areConsumerTrainingDataUsed | M | T | F |  | F | T |
| usedConsumerTrainingData | CM | T | F |  | F | T |
| confidenceIndication | O | T | F |  | F | T |
| modelPerformanceTraining | M | T | F |  | F | T |
| modelPerformanceValidation | O | T | F | F |  | T |
| dataRatioTrainingAndValidation | O | T | F | F |  | T |
| areNewTrainingDataUsed | M | T | F |  | F | T |
| LearnedMLCapabilities | O | T | F |  | F | T |
| Attribute related to role |  |  |  |  |  |  |
| trainingRequestRef | CM | T | F |  | F | T |
| trainingProcessRef | M | T | F |  | F | T |
| lastTrainingRef | CM | T | F |  | F | T |
| mLEnityGeneratedRef | M | T | F |  | F | T |
| mLEnityCoordinationGroupGeneratedRef | CM | T | F | F |  | T |

###### 7.3a.1.2.3.3	Attribute constraints

Table 7.3a.1.2.3.3-1

| Name | Definition |
| --- | --- |
| usedConsumerTrainingData Support Qualifier | Condition: The value of areConsumerTrainingDataUsed attribute is ALL or PARTIALLY. |
| trainingRequestRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for the ML model training that was requested by the MnS consumer (via MLTrainingRequest MOI). |
| lastTrainingRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for the ML model training that was not initial training (i.e. the model has been trained before). |
| mLEnityCoordinationGroupGeneratedRef Support Qualifier | Condition: The MLTrainingReport MOI represents the report for a joint training of a group of ML entities. |

###### 7.3a.1.2.3.4	Notifications

The common notifications defined in clause 7.e are valid for this IOC, without exceptions or additions.

Next change

## 7.4	Data type definitions

##### 7.4.X.	LearnedMLCapability << dataType >>

#### 7.4.X.1.	Definition

This dataType represents a supported ML capability. The ML capability is the ability of the ML entity to derive a specific analysis or recommendation.

#### 7.4.X.2	Attributes

The LearnedMLCapability << dataType >> includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceOutputName | M | T | F | F | F |
| learnedMLpredictorArray | O | T | T | F | T |
| learnedMLresponseArray | M | T | T | F | T |

#### 7.4.X.3	Attribute constraints

None.

#### 7.4.X.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

Next change

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLEntityId | It identifies the ML entity. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTrainingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the ML model supports. allowedValues: the values of the MDA type (see 3GPP TS 28.104 [2]), Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related MLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingProcessRef | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| trainingReportRef | It is the DN of the MLTrainingReport MOI that represents the reports of the ML training. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication | It indicates the confidence (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityList | It describes the list of MLEntity. | type: MLEntity multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestSource | It describes the entity that requested to instantiate the MLTrainingRequest MOI. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.requestStatus | It describes the status of a particular ML training request. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | It indicates the priority of the training process. The priority may be used by the ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the MLtraining MnS producer to terminate a specific training process. allowedValues: MODEL UPDATED_IN_INFERENCE_FUNCTION, INFERENCE FUNCTION_TERMINATED, INFERENCE FUNCTION_UPGRADED, INFERENCE_CONTEXT_CHANGED. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityVersion | It indicates the version number of the ML entity. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTraining | It indicates the performance score of the ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLTrainingProcess". When the ML training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - “COLLECTING_DATA” - “PREPARING_TRAINING_DATA” - “TRAINING ” + DN of the MLEntity being trained The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an ML entity. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor's specific extensions. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an ML entity, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | Type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.cancelRequest | It indicates whether the ML training MnS consumer cancels the ML training request. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingRequest.suspendRequest | It indicates whether the ML training MnS consumer suspends the /ML training request. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.cancelProcess | It indicates whether the ML training MnS consumer cancels the ML training process. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.suspendProcess | It indicates whether the ML training MnS consumer suspends the ML training process. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef | It describes the target entities that will use the ML entity for inference. | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the ML entity e.g. for training or inference | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext | This describes the context where an MLEntity is expected to be applied or/and the RunTimeContext which is the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext | This specify the context under which the MLEntity has been trained. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext | This specifies the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTrainRef | It identifies the DN of the MLEntity requested to be trained. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEnityGeneratedRef | It identifies the DN of the MLEntity generated by the ML training. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEntityRepositoryRef | It idenfifies the DN of the MLEntityRepository. | Type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLRepositoryId | It indicates the unique ID of the ML repository. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceValidation | It indicates the performance score of the ML entity when performing on the validation data. allowedValues: N/A | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dataRatioTrainingAndValidation | It indicates the ratio (in terms of quantity of data samples) of the training data and validation data used during the training and validation process. It is represented by the percentage of the validation data samples in the total training data set (including both training data samples and validation data samples). The value is an integer reflecting the rounded number of percent * 100. allowedValues: { 0 .. 100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityIdList | It identifies a list of ML entities. allowedValues: N/A. | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| MLTestingRequest.requestStatus | It describes the status of a particular ML testing request. allowedValues: NOT_STARTED, TESTING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTestRef | It identifies the DN of the MLEntity requested to be tested. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTesting | It indicates the performance score of the ML entity when performing on the testing data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTestingResult | It provides the address where the testing result (including the inference result for each testing data example) is provided. The detailed testing result format is vendor specific. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| testingRequestRef | It identifies the DN of the MLTestingRequest MOI. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| supportedPerformanceIndicators | It identifies a specific PerformanceIndicator of an ML entity. allowedValues: N/A. | type: SupportedPerfIndicator multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| performanceIndicatorName | It indicates the identifier of the specific performance indicator. allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isSupportedForTraining | It indicates whether the specific performance indicator is supported a performance metric of ML training for the ML entity Default value is set to "FALSE". allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: False isUnique: True defaultValue: FALSE isNullable: False |
| isSupportedForTesting | It indicates whether the specific performance indicator is supported a performance metric of ML testing for the ML entity. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: False isUnique: True defaultValue: FALSE isNullable: False |
| mLUpdateProcessRef | It is the DN of the mLUpdateProcess MOI that represents the process of updating an ML entity. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateRequestRef | It is the DN of the MLUpdateRequest MOI that represents an ML update request. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportRef | It is the DN of the MLUpdateReport MOI that represents an ML update report. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportingPeriod | It specifies the time duration upon which the MnS consumer expects the ML update is reported. | Type: TimeWindow (see TS 28.622 [12]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availMLCapabilityReport | It represents the available ML capabilities. allowedValues: N/A. | Type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| UpdatedMLCapability | It represents the updated ML capabilities. allowedValues: N/A. | Type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| newCapabilityVersionId | It indicates the specific version of AI/ML capabilities to be applied for the update. It is typically the one indicated by the MLCapabilityVersionID in a newCapabilityVersion | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mlCapabilityVersionId | It indicates the version of ML capabilities that is available for the update. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceGainThreshold | It defines the minimum performance gain as a percentage that shall be achieved with the capability update, i.e., the difference in the performances between the existing capabilities and the new capabilities should be at least performanceGainThreshold otherwise the new capabilities should not be applied. Allowed value: float between 0.0 and 100.0 | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availableMLCapability | It indicates an available version of alternative ML capabilities that can be used for update. Note: MLCapability is FFS. | type: MLCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectedPerformanceGains | It indicates the expected performance gain if/when the AI/ML capabilities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities. | Type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| achievedPerformanceGain | It indicates the performance gain that is achieved by updating the AI/ML entity or network function. | Type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| updateTimeDeadline | It indicates the maximum as stated in the MLUpdate request that should be taken to complete the update | Type: TimeWindow (see TS 28.622 [12]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEntityRef | It indicates the list of references to MLEntity instances that can be updated. | Type: DN multiplicity: 1 .. * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLUpdateRequest.requestStatus | It describes the status of a particular ML update request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | Type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| memberMLEntityRefList | It identifies the list of member ML entities within a level of an ML entity coordination group. allowedValues: DN list | Type: DN (see TS 32.156 [13]) multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| mLEntityCoordinationGroupToTrainRef | It identifies the DN of the MlEntityCoordinationGroup requested to be trained. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEnityCoordinationGroupGeneratedRef | It identifies the DN of the MlEntityCoordinationGroup generated by the ML training. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEntityCoordinationGroupToTestRef | It identifies the DN of the MlEntityCoordinationGroup requested to be tested. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| retrainingEventsMonitorRef | It indicates the DN of the ThresholdMonitor MOI that indicates the performance measurements and its corresponding thresholds to be used by MnS producer to initiate the re-training of the MLEntity. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sourceTrainedMLEntityRef | It identifies the DN of the source trained MLEntity whose copy has been loaded from the ML entity repository to the inference function. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingRequest.requestStatus | It describes the status of a particular ML entity loading request. allowedValues: NOT_STARTED, LOADING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLEntityLoadingRequest.cancelRequest | It indicates whether the MnS consumer cancels the ML entity loading request. Setting this attribute to "TRUE" cancels the ML entity training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " LOADING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLEntityToLoadRef | It identifies the DN of a trained MLEntity requested to be loaded to the target inference function(s). | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| policyForLoading | It provides the policy for controlling ML entity loading triggered by the MnS producer. This policy contains two thresholds in the thresholdList attribute. The first threshold is related to the ML entity to be loaded, and the second threshold is related to the existing ML entity being used for inference. | Type: AiMlManagementPolicy multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thresholdList | It provides the list of threshold. | Type: ThresholdInfo (see TS 28.622 [12]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingProcess.cancelProcess | It indicates whether the MnS consumer cancels the ML entity loading process. Setting this attribute to "TRUE" cancels the process. Cancellation is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingProcess.suspendProcess | It indicates whether the MnS consumer suspends the ML entity loading process. Setting this attribute to "TRUE" suspends the process. Suspension is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingProcess.resumeProcess | It indicates whether the MnS consumer resumes the ML entity loading process. Setting this attribute to "TRUE" resumes the process. Suspension is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "SUSPENDED". Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingRequestRef | It identifies the DN of the associated MLEntityLoadingRequest. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingPolicyRef | It identifies the DN of the associated MLEntityLoadingPolicyRef. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| performanceMetricName | It identifies the name of the performance metric or measurement. AllowedValues: the name of the performance metrics specified for the ML training phase and AI/ML inference phase. | Type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thresholdDirection | It indicates the direction of a threshold indicating the direction for which a threshold crossing triggers a threshold. When the threshold direction is configured to "UP", the associated threshold is triggered only when the subject performance metric value is equal to or greater than the threshold value. The threshold is not triggered, when the performance metric value is smaller than the threshold value. Vice versa, When the threshold direction is configured to "DOWN", the associated threshold is triggered only when the subject performance metric value is equal or smaller than the threshold value. The threshold is not triggered, when the performance metric value s greater than the threshold value. allowedValues: - UP - DOWN | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdValue | It specifies the value against which the monitored performance metric is compared. allowedValues: float or integer | Type: Union multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| LoadedMLEntityRef | It identifies the DN of the MLEntity that has been loaded to the inference function. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| activationStatus | It describes the activation status. allowedValues: ACTIVATED, DEACTIVATED. | Type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | Type: ManagedActivationScope multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.dNList | It indicates the list of DN, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: DN multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.timeWindow | It indicates the list of time window; the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: TimeWindow (see TS 28.622 [12]) multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.geoPolygon | It indicates the list of GeoArea, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: GeoArea(see TS 28.622 [12]) multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| LearnedMLCapabilities | It indicates the list of capabilities that have been learned by an ML entity. allowedValues: N/A. | type: LearnedMLCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the specific name of the capability of the ML Entity, e.g., the person correlation. allowedValues: N/A | Type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| learnedMLpredictorArray | It indicates a specific array of input values for which corresponding responses are generated. For a distribution for example, it is the set of CDF/PDF bins for which corresponding values are generated. allowedValues: array of Real number. | Type: Real multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| learnedMLresponseArray | It indicates a specific array representing the mL output values corresponding to input values in the learnedMpredictorArray. allowedValues: array of Real number. | Type: Real multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE: When the performanceScore is to indicate the performance score for ML training, the data set is the training data set. When the performanceScore is to indicate the performance score for ML validation, the data set is the validation data set. When the performanceScore is to indicate the performance score for ML testing, the data set is the testing data set. |  |  |

End of Changes