3GPP TSG-SA5 Meeting #155 	S5-242647

# Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.156 | CR | 0096 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 32.156 Correction of interrupted annex F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The content of Annex F is broken due to the incorrect implementation of the corresponding change of agreed S5-240975. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the Annex F and Annex G according to the corresponding change of agreed S5-240975. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect Annex F and Annex G. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F, Annex G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

Annex F (normative):
Application of <<InformationObjectClass>> and <SupportIOC>>

The <<InformationObjectClass>> and <<SupportIOC>> are stereotypes.  These two stereotypes serve similar purpose in that each is a named set of network resource properties.  However, their applications, in the context of supporting network management over Itf-N or through the use of management services, can be different.  This Annex highlights their similarities and differences of such application.

|  | <<InformationObjectClass>> | <<SupportIOC>> |
| --- | --- | --- |
| Can it be an abstract class? | Yes | Yes |
| Can it be a concrete class? | Yes | Yes |
| Can it inherit from <<InformationObjectClass>>? | Yes | No, except for <<InformationObjectClass>> Top. |
| Can it inherit from <<SupportIOC>>? | No | Yes |
| Can it be name-contained by <<InformationObjectClass>>? | Yes | Yes |
| Can it be name-contained by <<SupportIOC>>? | No | Yes |
| Can an instance have a DN? | <<InformationObjectClass>> must be a class of a naming-tree meaning all its instances must have a DN. | <<SupportIOC>> may be used by specification author for a class within a naming-tree. If so, it means that all its instances will have a DN. |
| Can either 1) IRPManager use operations of Basic CM IRP [9] and Bulk CM IRP [10] or 2) MnS consumer use the Provisioning operations [17] and [16] to access the information in an instance? | Either 1) IRPManager can use the Basic CM IRP and Bulk CM IRP operations or 2) MnS consumer can use the provisioning operations to access information of all <<InformationObjectClass>> defined in all NRM [15], in accordance to the qualifier values of the <<InformationObjectClass>>. | Either 1) IRPManager can use the Basic CM IRP and Bulk CM IRP operations to access information of instances of <<SupportIOC>> defined in their respective Interface IRP (i.e. Basic CM IRP or Bulk CM IRP), in accordance to the qualifier values of the <<SupportIOC>> or 2) MnS consumer can use the provisioning operations to access information of instances of <<SupportIOC>> specified in [16] and [17] in accordance to the qualifier values of the <<SupportIOC>>. Neither 1) IRPManager can use the Basic CM IRP and Bulk CM IRP operations to access information of instances of <<SupportIOC>> not defined in their respective Interface IRP (i.e. Basic CM IRP or Bulk CM IRP) nor 2) MnS consumer can use the Provisioning operations to access information of instances of <<SupportIOC>> not defined in [16] and [17] |
| Can either 1) IRPManager use operations of Interface IRP, except Basic CM IRP [9] and Bulk CM IRP [10] (e.g. Alarm IRP [11]), or 2) MnS consumer use non Provisioning operations (e.g. fault supervision operations [14] and [16] to access the information? | No | Either 1) IRPManager can use the Interface IRP operations to access information of <<SupportIOC>> defined in their respective Interface IRP, in accordance to qualifier values of the <<SupportIOC>> or 2) . MnS consumer can use the Provisioning operations to access information of instances of <<SupportIOC>> specified in [16] and [17] in accordance to the qualifier values of the <<SupportIOC>>. Neither 1) IRPManager can not use the Interface IRP operations to access information of <<SupportIOC>> not defined in their respective Interface IRP, nor 2) MnS consumer can not use the Provisioning operations to access information of instances of <<SupportIOC>> not defined in [16] and [17]. |
| Can either IRPManager or MnS consumer receive information via Notification [12] whose objectClass and objectInstance parameters carry the instance DN? | Yes. The types of notification emitted are shown by the Notification Table associated with the class definition. | Yes if <<SupportIOC>> is a class of a naming-tree. The types of notification emitted are shown by the Notification Table associated with the class definition. No if <<SupportIOC>> is not a class of a naming-tree. |
| Measurement [13] | Measurements can be associated with <<InformationObjectClass>> instances. | Measurements can be associated with <<SupportIOC>> instances if <<SupportIOC>> class is used in a naming-tree. |

Annex G (informative): Naming rules of modeling and programming languages

# G.1	OpenAPI naming rules – OpenAPI solution set

While OpenAPI allows any string as an identifier, a number of organizations and vendors limit the allowed characters and format of identifiers to make implementation easier. Widely used guidelines include the principles:

- Use only ASCII characters mostly limited to letters, digits, underscore, hypen

- The first character shall be a letter or underscore

- Use camelcase

In 3GPP TS 29.501 [23] clause 5.1 the UCC and LCC conventions (used for IOC and attribute names) indicate only the use of upper and lower case letters and digits.

# G.2	Yang Naming rules – Netconf-YANG solution set

YANG identifier naming rules are specified in RFC 7950 at https://www.rfc-editor.org/rfc/rfc7950#section-6.2 [22].

- Each identifier starts with an uppercase or lowercase ASCII letter or an underscore character, followed by zero or more ASCII letters, digits, underscore characters, hyphens, and dots.

- Implementations SHALL support identifiers up to 64 characters in length and MAY support longer identifiers.  Identifiers are case sensitive.

# G.3	Java™ naming rules

- Names can contain letters, digits, underscores, and dollar signs

- Names shall begin with a letter, underscore or dollar sign, but should start with a letter

- Names are case sensitive ("myVar" and "myvar" are different variables)

- Reserved words (like Java keywords, such as int or boolean) cannot be used as names

# G.4	C++ naming rules

- An identifier can consist of letters (A-Z or a-z), digits (0-9), and underscores (_). Special characters and spaces are not allowed.

- An identifier can only begin with a letter or an underscore only.

- C++ has reserved keywords that cannot be used as identifiers

Modern C++ implementation may support other Unicode character with the Unicode property XID_Start and XID_Continue, but this are not widely known.

# G.5	Python naming rules

- An identifier can consist of letters (A-Z or a-z), digits (0-9), and underscores (_). Special characters and spaces are not allowed.

- An identifier can only begin with a letter or an underscore only.

- Reserved keywords that cannot be used as identifiers

Python 3 (but not Python2) includes additional characters from outside the ASCII range, but these are not widely known.

End of changes