3GPP TSG-SA5 Meeting #155 	S5-243293

# Jeju, South Korea, 27 - 31 May 2024

# Source:	CATT, China Unicom

# Title:	pCR TR 28.874 Add terms and abbreviations

# Document for:	Approval

Agenda Item:	6.19.15

1	Decision/action requested

Approval

2	References

[1]	3GPP TR 28.874-010: " Study on management aspects of NTN – Phase 2"

[2]		SP-231733: "New SID: Study on Management Aspects of NTN Phase 2"

3	Rationale

It is proposed to add definitions and terms.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st change

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 22.261 [a] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Feeder link: Wireless link between NTN Gateway and satellite.

Service link: Radio link between satellite and UE.

Regenerative payload: payload that transforms and amplifies an uplink RF signal before transmitting it on the downlink. The transformation of the signal refers to digital processing that may include demodulation, decoding, re-encoding, re-modulation and/or filtering.

Serving satellite: a satellite providing the satellite access to a UE (e.g. providing the serving cell(s)). Depending on the orbit, the serving satellite may cover a given geographic area for a limited period of time.

S&F Satellite operation: operation mode providing communication service (in storing and forwarding information) to a UE in periods of time and/or geographical areas in which the serving satellite is not simultaneously connected to the ground network via feeder link or ISL. For the case of UL, "store" refers to on-board storage of UL information from UE and "forward" refers to forwarding of stored UL information to the ground network. For the case of DL, "store" refers to on-board storage of DL information from the ground network and "forward" refers to forwarding of stored DL information to the UE.

UE-Satellite-UE Communication: refers to a communication between UEs under the coverage of one or more serving satellites, using satellite access without the user traffic transiting through the ground segment.

NTN Gateway: an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link. An NTN Gateway is a TNL node.

On board NTN gNB: gNB implemented in the regenerative payload on board a satellite.

On ground NTN gNB: gNB of a transparent satellite payload implemented on ground.

Next change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.423: "Technical Specification Group Radio Access Network; NG-RAN; Xn application protocol (XnAP) "

[3]	3GPP TS 38.300: "Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description; Stage 2"

[4]	3GPP TR 38.821: "Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) "

[5]	3GPP TR 22.865: "Study on satellite access Phase 3"

[x]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[y]	3GPP TR 23.700-29: " Study on integration of satellite components in the 5G architecture; Stage 3".

Next change

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

NTN	Non-Terrestrial Network

S&F	Store and Forward

End of change