3GPP TSG SA WG5 Meeting #155	S5-242728

Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0527 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.255 Change Inter-CHF information to be generic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, B2B_CH |  |  |  |  |  | Date: |  |  | 2024-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Release 18, there are two charging service reference points introduced for the interaction between two CHFs. - N107 is used between V-CHF and H-CHF for LBO roaming charging (CHRACHF). - N108 is used between C-CHF and B-CHF for business charging (B2B_CH). e.g. Network Slice (NETSLICE_CH_Ph2) and TSN (TSN_CH). In the 5G data connectivity charging, both LBO roaming charging and business charging (per network slice or per TSN bridge) are supported. The inter-CHF information can be generic to also support the business charging scenarios. In addition, when inter CHF interaction is supported, the source of Charging Data Request may be either SMF or CHF (i.e. C-CHF or V-CHF), and destination of Charging Data Response may also be a CHF (i.e. C-CHF or V-CHF). This should be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add CHF to the source of Charging Data Request, and destination of Charging Data Response, in Table 6.1.1.1.1. 2. Clarify the applicability of the Inter-CHF Information IE in Charging Data Request and Response, Table 6.1.1.2.1 and Table 6.2.2.1. 3. Expand the definition of Inter-CHF Information, covering the case when C-CHF has connection to B-CHF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inter-CHF information for N108 is undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1, 6.1.1.2, 6.1.1.3, 6.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.291 CR 0563 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [57] and include charging information. The Charging Data Request can be of type [Initial, Update, Termination].

Table 6.1.1.1.1 describes the use of these messages for converged charging.

Table 6.1.1.1.1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | SMF/CHF | CHF |
| Charging Data Response | CHF | SMF/CHF |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

Next change

6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the SMF as used for 5G data connectivity converged charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element |  | Category for converged charging |  | Category for offline only charging |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Session Identifier |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Subscriber Identifier |  | OM |  | M |  | Described in TS 32.290 [57] In case SUPI is not present (for emergency service), the User Equipment Info in table 6.2.1.2.1. shall be present for identifying the user. |  |
| NF Consumer Identification |  | M |  | M |  | Described in TS 32.290 [57] |  |
| NF Functionality |  | M |  | OC |  | Described in TS 32.290 [57] |  |
| NF Name |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| NF Address |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| NF PLMN ID |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Invocation Timestamp |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Invocation Sequence Number |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Retransmission Indicator |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Notify URI |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Service Specification Information |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Supported Features |  | OC |  | - |  | This field indicates the features supported by the NF consumer. |  |
| Triggers |  | OC |  | OC |  | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |  |
| Multiple Unit Usage |  | OC |  | OC |  | Described in TS 32.290 [57] This field is not applicable to QBC. |  |
| Rating Group |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Requested Unit |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Used Unit Container |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Service Identifier |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Quota management Indicator |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Triggers |  | OC |  | OC |  | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |  |
| Trigger Timestamp |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Time |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Total Volume |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Uplink Volume |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Downlink Volume |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Local Sequence Number |  | OM |  | OM |  | Described in TS 32.290 [57] |  |
| PDU Container Information |  | OC |  | OC |  | This field holds the 5G data connectivity PDU session container specific information described in clause 6.2. |  |
| UPF ID |  | OC |  | OC |  | This field holds the UPF identifier used to identify the UPF. These fields shall only be included when either quota is requested per UPF, or used units are reported per UPF |  |
| multi-homed PDU address |  | OC |  | OC |  | This field holds the IPv6 prefix used by UPF. It may only be used for IPv6 multi-homed PDU sessions and then only for reporting used units. |  |
| PDU Session Charging Information |  | OM |  | OM |  | This field holds the 5G data connectivity specific information described in clause 6.2. This field is applicable to FBC and QBC. |  |
| Roaming QBC information |  | OM |  | OM |  | This field holds the roaming QBC specific information defined in clause 6.2.1.4 This field is only applicable to QBC. |  |
| Inter-CHF Information |  | OC |  | - |  | This field holds inter CHF specific information described in clause 6.2.1.6. This field only applies to the request between two CHFs, i.e. from V-CHF to H-CHF, or from C-CHF to B-CHF. |  |

Next change

6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G data connectivity converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element |  | Category for converged charging |  | Category for offline only charging |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Session Identifier |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Invocation Timestamp |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Invocation Result |  | OC |  | M |  | Described in TS 32.290 [57] |  |
| Invoation Result Code |  | OC |  | M |  | Described in TS 32.290 [57] |  |
| Failed Parameter |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Failure Handling |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Invocation Sequence Number |  | M |  | M |  | Described in TS 32.290 [57] |  |
| Session Failover |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Supported Features |  | OC |  | - |  | This field indicates the features supported by the NF consumer. |  |
| Multiple Unit Information |  | OC |  | OC |  | Described in TS 32.290 [57] This field is not applicable to QBC. |  |
| Result Code |  | OC |  | OC |  | Described in TS 32.290 [57] |  |
| Rating Group |  | M |  | M |  | Described in TS 32.290 [57] |  |
| UPF ID |  | OC |  | OC |  | This field holds the UPF identifier used for quota granted per UPF by CHF |  |
| Granted Unit |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Validity Time |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Final Unit Indication |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Time Quota Threshold |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Volume Quota Threshold |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Unit Quota Threshold |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Quota Holding Time |  | OC |  | - |  | Described in TS 32.290 [57] |  |
| Triggers |  | OC |  | OC |  | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |  |
| Triggers |  | OC |  | OC |  | This field is described in TS 32.290 [57] and holds the 5G data connectivity specific triggers described in clause 5.2.1. |  |
| PDU Session Charging Information |  | OM |  | OM |  | This field holds the 5G data connectivity specific information described in clause 6.2. |  |
| Roaming QBC Information |  | OM |  | OM |  | This field holds the roaming QBC specific information defined in clause 6.2.1.4 This field is not applicable to FBC. |  |
| Inter-CHF Information |  | OC |  | - |  | This field holds inter-CHF specific information described in clause 6.2.1.6. This field only applies to the response from CHF (i.e. V-CHF or C-CHF) to SMF. |  |

Next change

6.2.1.6	Definition of Inter-CHF information

Specific charging information used for information when the V-CHF have a connection to the H-CHF, or C-CHF have a connection to the B-CHF.

The detailed structure of the Inter-CHF Information can be found in table 6.2.1.6.1.

Table 6.2.1.4.1: Structure of Inter-CHF Information

| Information Element | Category | Description |
| --- | --- | --- |
| Remote CHF resource | OC | This field holds the reference the Charging Data resource in the CHF not directly connected to the NF e.g., the resource URI |
| Original NF Consumer Id | OM | This field holds information on the NF triggering the request i.e., SMF |

End of change