3GPP TSG-SA5 Meeting #155 	S5-242752

Jeju, Korea, 27 - 31 May 2024

# Source:	Huawei

# Title:	Rel-19 Discussion paper on background for FS_NG_RTC_Ph2_CH

# Document for:	Discussion

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to discuss.

2	References

[1]	S5-241835: "New SID on Charging aspects of next generation real time communication services phase 2".

3	Rationale

This contribution proposes to add background for charging aspects of next generation real time communication services phase 2 which is initiated by S5-241835 [1].

4	Detailed proposal

1st Change

4	Background

4.1	General

3GPP SA4 introduced the data channel for IMS network in Rel-16. In Rel-18, SA1 studied the evolution of IMS multimedia telephony service, and documented the new use cases for IMS Multimedia Telephony service and the potential service requirements in 3GPP TR 22.873 [X1]. The data channel functionality is necessary for implementing some of the use cases. Based on the SA1 output, Rel-18 SA2 specified the architecture, interfaces and procedures of IMS data channel and AR communication documented in 3GPP TS 23.228 [X2].

Rel-19 SA2 initiated SID FS_NG_RTC_ph2 to study 8 Key Issues with multiple functionalities and services enhancement to IMS data channel documented in 3GPP TR 23.700-77 [X3], which include:

-	The architecture, interfaces and procedures of IMS network capability exposure;

-	IMS event subscription and notification mechanism;

-	DCMTSI UE interworking with MTSI UE;

-	3rd Party Identity in IMS session;

-	3GPP PS data off exemption for services over IMS data channel;

-	Standalone IMS data channel session;

-	multiplexing multiple DC applications over single SCTP connection;

-	IMS Avatar Communication.

4.2	Networks functionality and architecture of IMS data channel

Figure 4.2-1 and Figure 4.2-1 show the architecture of IMS data channel depicted in clause AC.2 of 3GPP TS 23.228 [X2]. In Figure 4.2-1, the service-based Media Function is introduced. In Figure 4.2-2, the existing MRF is enhanced.

Figure 4.2-1: Architecture option of IMS supporting DC usage with MF

Figure 4.2-2: Architecture option of IMS supporting DC usage with MRF

Three new network functions are introduced for IMS data channel:

- Data Channel Application Repository (DCAR): It stores the verified data channel applications which are retrieved by the DCSF when required.

- Data Channel Signaling Function (DCSF): It’s the signalling control function that provides data channel control logic.

- Media Function (MF): It provides the media resource management and forwarding of data channel media traffic.

The existing MRF can be enhanced to provide the same functionalitites as the MF.

4.3	Data channel application download

Figure 4.3-1 shows the data channel workflow depicted in clause 6.2.10.1 of 3GPP TS 26.114 [X4]. The local UE A and the remote UE B can download the data channel application required download through the interaction with local DCSF over the bootstrap data channels.

Figure 4.3-1: Data Channel Workflow

The bootstrap data channel setup signalling procedure depicted in clause AC.7.1 of 3GPP TS 23.228 [X2] also described how the data channel application is downloaded to the UEs in an IMS session.

4.4	Charging modes

According to 3GPP TS 32.260 [X5] Rel-18 and 3GPP TS 32.255 [X6] Rel-18, the following charging modes are used for IMS data channel charging:

- Duration-based charging

- Volume-based charging

Considering the functionalities and features studied in Rel-19 SA2 FS_NG_RTC_Ph2, such as IMS network capabilities exposure, event-based charging is also needed for IMS data channel charging.

End of changes