3GPP TSG-SA5 Meeting #155 	S5-243160

Jeju, South Korea, 27 - 31 May 2024

# Source:	ZTE Corporation, Huawei

# Title:	pCR TR 28.914 add alternative solutions for implicit intent report subscription

# Document for:	Approval, Information, Discussion

Agenda Item:	6.19.3

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	Study on intent driven management service for mobile network phase 3 v0.0.0

[2]	SP-231737 "Study on intent driven management services for mobile network phase 3"

3	Rationale

(With bullet points, describe the reasons for the proposed action. 
The objectives of the proposal should be clearly stated. 
Rejected alternative solutions should be mentioned if this aids understanding).

(For pseudo CR, the reason for change(s) and summary of change(s) must be clearly explained.)

4	Detailed proposal

Start of First change

5.3 Use case #3: Implicit intent report subscription

5.3.1 Description

In TS 28.312 [2], NtfSubscriptionControl IOC is used for MnS consumer to subscribe the intent report information explicitly. MnS consumer needs to request to create a NtfSubscriptionControl instance to subscribe the intent report information for a specified intent instance after such intent instance is created.

Figure 5.X.1-1 Explicit intent report subscription

However, in some scenarios, MnS consumer who expresses the intent may want to obtain the intent report by default, instead of triggering separate subscription action (i.e. request to create a NtfSubscriptionControl instance) to subscribe intent report information (especially intent fulfilment information).

So, the capability to support for implicit intent report subscription is important for the MnS consumer who express the intent.

5.3.2 Potential requirements

REQ-Intent_Driven_MnS-CON-1: The intent driven MnS producer should have the capability enabling MnS consumer who expresses the intent to specify intent report control information in the intent when creating or modifying the intent.

The existing REQ-Intent_Driven_MnS_Report-1,2,3,4,5,6,7 defined in TS 28.312 [2] also applied for implicit intent report subscription

5.3.3 Potential solutions

5.3.3.1 Potential solution #1

It proposes to add an attribute "reportRecipientAddress" in Intent <<IOC>> to represent the address of notification recipient for MnS consumer to receive the notification (i.e. notifyMOIAttributeValueChanges notification) with intent report information. If present, this attribute instructs the MnS producer to create, on behalf of the MnS consumer, a subscription for attribute value change notifications for IntentReport instance of the corresponding intent instance. Then MnS consumer does not need to request to create a NtfSubscriptionControl instance to receive the notification with intent report information.

Figure 5.X.3-1 Implicit intent report subscription

In addition, the existing attribute “observationPeriod” in Intent <<IOC>> can be used to allow the MnS consumer who expresses the intent to configure the frequency of the intent reporting.

In case MnS consumer who expresses the intent does not want to subscribe to the intent report content, a new attribute “expectedReportTypes” can be added as an optional attribute of Intent <<IOC>> to allow the MnS consumer who expresses the intent to specify the content of the intent report. The allowed values for attribute “expectedReportType” can be intentFulfilmentReport, intentConflictReport or intentFeasibilityCheckReport.

5.3.3.2 Potential solution #2

The existing Intent IOC can be extended to support customized intent report.

Enhancement Aspect: Add a new IntentReportControl <<datatype>> to the Intent IOC and one Intent instance can optionally have an IntentReportControl. IntentReportControl <<datatype>>  includes following attributes:

- “expectedReportTypes”, represents the type of IntentReports, which can be one/any/all of “IntentFulfilmentReport”, “IntentConflictReport”, and “IntentFeasibilityCheckReport”.

- “reportingTime”, represents the specified time condition for intent reporting. For example, it can be an interval, a specific time, or a time window.

- “reportingTargets”, represents the specified targets needed to be reported and corresponding conditions. This attribute can be a <<dataType>>, including targetName, targetCondition, and targetValueRange that are already defined in TS 28.312.

- “observationPeriod”, represents the frequency of the intent reporting.

- “reportRecipientAddress”, represent the address of notification recipient for MnS consumer.

5.3.4 Evaluation of potential solutions

TBD

End of First change