3GPP TSG-SA5 Meeting #159	S5-250062

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.567 CCL Trigger-time and action-execution-time conflicts handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add use case, requirements and solution description for Conditional control of CCLs of CCLs |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses on the use case to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 (new clause added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 6	Closed Control Loops Coordination Capabilities

## 6.A	 Closed Control Loop Conflict management

### 6.A.1	Description

#### 6.A.1.4	CCL Trigger-time and action-execution-time conflicts handling

##### 6.A.1.4.1	Introduction

Typically, a CCL will be triggered to run at a specific time and terminate when certain conditions are met, to run when a certain performance threshold is crossed. If triggered independently, there may be conflicts among the CCLs. The triggers for different CCLs to be executed need to be coordinated to avoid conflicts among the CCLs.

In a network with several CCLs, it can be that each CCL focuses on a smaller scope of the network problem space, and thus, propose changes that are suboptimal to the full problem space. To support the full problem space, several CCLs may be executed but it is important that their actions do not collide with each other in any network scope. The CCLs need to be explicitly scheduled to act by the CCL coordination entity which has a wider view of the network problems, the capabilities of the CCLs and their likely impacts.

##### 6.A.1.4.2	Avoidance of action-execution-time conflicts

Since each CCL focuses on a smaller scope of the network problem space, several CCLs may need to be executed.  To avoid collision of their actions in a given network scope, the CCLs can be explicitly scheduled by the CCL coordination entity. The CCL coordination entity obtains the capabilities of the available CCLs including their desired scopes and execution times. The CCL coordination entity evaluates network data and analytics to identify the nature of the problem and best CCLs to be scheduled at specific times to address the problem but without their execution conflicting with one another.

##### 6.A.1.4.3	Detection of potential action-execution-time conflicts

6.A.1.4.3.1	Information on action plans for alignment and selection

Each CCL deployed in the network has a set of scopes for which it takes responsibility and actions that it can execute within those scopes and from which the CCL derives the decision and action plans that should be executed. An action plan is the combination of a set of actions that can be taken and the scopes under which those actions can be applied. To minimize conflicts (e.g. where the scopes overlap), the CCLs align the action plans, for example, through a coordinator CCL that selects which action plan to execute and when. Thereby:

-	The CCLs inform the coordinator CCL about their respective action plans. The action plans contain information of target resources, scheduled time for execution, and may include other additional information such as historical results of the proposed actions.

-	The coordinator CCL assesses each plan and choose the most appropriate combination of action plan(s) based on the selection policy. The appropriateness of action plan(s) or their combinations can be evaluated by multiple means and by using, for instance, historical data and/or operational data.

-	Notify the selected action plan(s) to the related CCLs and/or the CCL coordination entity.

6.A.1.4.3.2	Information on detected action-execution-time conflict

Coordinating actions among multiple CCLs requires that there is a supervisory action-critic functionality that oversees the actions of the different CCLs. The action-critic functionality which may be part of the CCL coordination entity that takes the responsibility for the end-to-end performance of the Network.

For a given CCL, the action-critic receives the recommended changes from the CCLs, evaluates them to see:

1)	if they overlap with other proposed changes from other CCLs; and

2)	what their likely effects may be.

To determine the likely impacts, the action-critic may rely on network states analytics capabilities which discretize the state of the network into specific discrete scenarios and provide insights on performance characteristics in those scenarios. Such insights may for example characterize whether the network is in a scenario of low traffic and normal performance or scenario of normal traffic and anomalous performance.

Where there are likely conflicts and expected undesired impacts, the orchestration functionality decides the changes that should be executed on the network to minimize concurrent changes on the same network resources. The selection of actions to be accepted may be based on the priorities of the CCLs or the priorities of their goals and targets.

The CCL coordination entity then provides feedback to the CCL instance (s) regarding their recommended actions. The feedback may include information on which actions can be executed or not as well as information on the expected effects of the CCLs actions. Feedback may also include redefining the allowed control parameter spaces and ranges of the individual CCLs (i.e. which parameters the CCL should not control any further or the range in which the CCL may set the value of a control parameter).

##### 6.A.1.4.4	Resolution of CCL Trigger-time conflicts

The triggers for execution of different CCLs need to be coordinated to avoid conflicts among the CCLs. And in some instances, the conditions in the network may be such that it is not clear which CCL should be triggered, requiring to trigger multiple CCL in sequence. The triggering may be done by a coordination function that consumes the CCL-related monitoring and governance services to receive information with which to evaluate the conditions and determines which CCL to be triggered.

It may be the case that CCLs need to operate in a hierarchy with each CCL having an operational profile indicating the specific level of hierarchy. The operational profile describes characteristic sunder which the CCL operates, e.g. when or after which other CCLs, this CCL should be executed. For example, to ensure that handovers are always optimal, a CCL on handover optimization may need to be triggered every after a CCL on Energy saving has been executed to be sure that there are appropriate handover relations even when some cells may have been disabled. The MnS consumer that coordinates the execution times of the CCLs needs to configure the appropriate hierarchy for the CCLs. Using the operational profiles of the CCLs, , the MnS consumer evaluates the description of the third CCL against at least one of the profiles P1 and P2 and accordingly determines and configures the operational profile of the third CCL.

NOTE:	A CCL may be involved in more than 1 hierarchies or within a single hierarchy, the CLL may relate to multiple other CCLS, which requires the hierarchies to be coordinated.

End Change