3GPP TSG-SA5 Meeting #159	S5-250070

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.561 NDT concept and scope |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add a description of NDT concept and scope |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use cases and solutions may be hard to agree to without appropriate context |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 (new clause 5.X added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

# 4	Concepts and overview

## 4.1	Introduction and Overview

### 4.1.1	Digital Twins and Network Digital Twins

A Digital twin is a representation of an object that models or simulates the characteristics and behaviours of the physical object. A digital twin can be created for any physical object, including any objects in communication networks. The digital twin may also be created for a group of objects, e.g. for the sets of network objects that form the RAN of a city.

Accordingly, a digital twin modelling an object of a communication network is called Network Digital twin

Network Digital Twin (NDT) may be used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any changes to the actual mobile network. To provide meaningful results, the Network digital twin needs to emulate (or simulate) the behavior of the mobile network, so that the result of the operations on the virtual replica are a good approximation to similar operations on the actual network. The standardization for an NDT focuses on the implementation of independent aspects of a network.

NOTE:	The terms simulation and emulation are used interchangeably through this document but both imply the same outcome, i.e., the NDT executes a process in an environment that is not the real network and generates output similar to that of the real network.

### 4.1.2	Scope of NDTs

Existing automation capabilities include analytics services and decision-making capabilities with the assumption that the knowledge about network behavior is available within the automation functions. Accordingly, the unaddressed gap for network automation is the capabilities for modelling the behavior of the network. The Network Resource Model (NRM) to describes the attributes of a mobile network and their relationships. The concept of Network Digital Twin also adds the ability to model the behavior of a mobile network. This behavior is modelled by emulating or simulating a complete mobile network or limited aspects of a mobile network.

### 4.1.3	Relations between digital twins and network automation functions

The NDT can be requested by the consumer to model a mobile network or part of one, support evaluating the corresponding impact, and return the report of the simulated impact generated by the NDT. The NDT may be used as a replica of a mobile network, which may synchronize data from the managed network for modelling. The digital twins provide modelling capabilities that are used by the network automation functions (e.g. MDA, SON, etc.) to accomplish their automation functionality. The related automation capabilities are provided by the network automation functions regardless of whether the digital twin models are integrated within or external to the network automation functions - see figure 4.1.3-1.

Option 1	NDT internal/integrated into existing Network Automation Functions.

Option 2	NDT separate from existing Network Automation Functions

Figure 4.1.3-1: Relation of NDTs with network automation functions

NOTE:	The double headed arrows indicate candidate flow of data and controls from the network to the NDT and to the management function and related flow of control from the MnFs to the network of NDT while one headed arrows indicate only flow of control.

## 4.2	NDT Life-cycle management

An NDT is an object that can be managed, i.e., an MnS consumer can be enabled to configure and control the characteristics of an NDT. NDT LCM may include the following capabilities:

-	Creation: When the MnS consumer submits a request to create an NDT, the MnS producer who provides the NDT may create an instance to fulfil or satisfy the specific scenarios.

-	Configuration: the MnS consumer may define or modify the attributes of an NDT instance

-	Activation: the MnS consumer may start execution of an NDT instance or restart execution of an instance that has prior-to been suspended.

-	Deactivation: the MnS consumer may stop execution of an NDT instance or suspend execution of an instance that is running.

-	Re-configuration: the MnS consumer may modify the attributes of an NDT instance.

-	Deletion: the MnS consumer may delete an NDT instance that is not under execution.

End Change