3GPP TSG-SA5 Meeting #159	S5-250072

Sofia-Antipolis, US, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev |  | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS28.561 NDT support to automation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2025-02-13 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add a description of NDT use cases and requirements on support to network automation |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | solutions cannot be agreed without agreement on use case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5 (new clause 5.X added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

# 5	Use cases and requirements

## 5.Y	NDT support for network automation

### 5.Y.1	Description

NDTs can be used to support many automations use cases (e.g. MDA, SON, etc.). An NDT may be integrated into a network automation function, or it may be external to the network automation function. In the case where the NDT is external to the network automation function, it could be possible for the network automation function to configure the NDT and the scenario that could be modelled and simulated by the NDT. NDTs do not execute any actions in form of configuration of the live network but support decision-making by proposing recommendations.

An NDT, depending upon the network or service management use case and scenario to be modelled and simulated, might need data originating from various sources (network data, environment data, analytics, UEs data) and suitable hardware/software resources to function properly. MnS consumers would prefer to specify needed NDT characteristics or configurations to the NDT tailored to fulfil consumer specific needs i.e. to define the consumer preference for the specific NDT. For example, consumer preferences may be related to environment data sources e.g. weather, synthetic data etc, data characteristics (e.g. robustness, data granularity, maximum tolerable latency), required NDT output latency, characteristics of the service to be twinned, resource constraints (HW/SW), etc. Furthermore, in the case that consumer's preference on NDT characteristics or configuration may change over time and MnS consumer may update the NDT with the needed changes.

The achieved performance by NDT may depend on NDT characteristics such as e.g. load; the simulation results would change based on the network load, time; the simulation results would change based on the target time of the day, energy saving state; the simulation results would change considering whether the simulated node is running in energy saving mode.

The following are examples of automation that the NDT might support:

-	prediction of Network failures and risks, by simulating and evaluating high-risk operations, potentially dangerous configuration or policy modification on the NDT before executing them on the network.

### 5.Y.2	Requirements

REQ-NDT-Auto-1:  The NDT should support a capability enabling an MnS consumer to specify NDT characteristics or configurations.

Note: example characteristics include

-	environment data sources (e.g. weather, synthetic data etc), and the related data characteristics (e.g. robustness, data granularity, maximum tolerable latency),

-	environment characteristics (e.g. network load, time of the day and energy saving state).

REQ-NDT-Auto-2: The NDT should support a capability to inform the MnS consumer whether a specific set of NDT characteristics or configurations defined by the MnS consumer are feasible, and if not, what changes could be made to the NDT configurations to make then feasible.

End Change