3GPP TSG-SA5 Meeting #159	S5-250204

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0058 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 cell proximity-based energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce a new use case for energy-saving optimization for scenarios where there may be multiple capacity booster cells are partially or fully overlaid by one or more candidate cell(s). In such scenarios, the capacity booster cells need to be matched to the appropriate candidate cells and the capacity booster cells need to be sequenced. This CR is related to Energy_OAM_Ph3, WT-8 Cell proximity-based energy saving. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proximity-based energy saving optimization for multi-layer RAN scenarios not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.X (new), 5.2.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.1.X	Proximity-based energy saving optimization for multi-layer RAN scenarios several capacity booster cells partially or fully overlaid by candidate cell(s)

#### 5.1.X.1	Overview

5G RAN can be multi-layer, characterized by several capacity booster cells, which are cells that can be deactivated when the load is low, overlaid by several candidate cells which can replace coverage when the capacity booster cells are deactivated. As illustrated in Figure 5.1.X.1-1, the multi-layer RAN may involve:

-	A cell layer where for several candidate cells, a candidate cell may overlap several capacity booster cells of different characteristics, e.g., the capacity booster cells may be macro or multi-sector cells.

-	A capacity booster cell’s coverage is partially or completely overlaid by the candidate cells’ coverage i.e. candidate cells provide complete coverage which the capacity booster cells complement with extra capacity

Figure 5.1.X.1-1: An example multi-layer (possibly multi-RAT) network with macro cells A and B, and small cells C, D and E.

A candidate cell may have several capacity booster cells that overlap with its coverage and can be deactivated. For a capacity booster cell to be deactivated, a candidate cell should have adequate overlap with the capacity booster cell to make sure that deactivation of the booster cell transfers the load to the appropriate candidate cells.

NOTE: "Overlap" means that the area covered by the candidate cell is also fully covered by the larger cell while overlap means that the area covered by the small cell can be partly or fully covered by the larger cell.

For example, a candidate cell (e.g., cell A) has 2 or more capacity booster cells that are associated with it, say on the different sides of the candidate cell's border (e.g. cell C1 and cell D2).  For cell deactivation in this scenario, there is need to decide which of those 2 or more capacity booster cells should be deactivated and in which order.

For centralized energy saving, an energy saving function, e.g., the "domain-centralized ES function", needs to select the best among the capacity booster cells to be deactivated at a given time. The energy saving function should be enabled to support the selection for and sequencing of deactivation of the capacity booster cells. The energy saving function needs to derive or obtain information on the degree of neighbourliness, the grouping of cells to be considered together or energy saving and the sequence in which the several cells may be activated or deactivated.

#### 5.A.1.2	Degree of neighborliness

To activate or deactivate cells, an energy saving function (e.g., the Domain-centralized ES function), computes the degree to which the coverage of cells overlap among each other to ensure that the coverage of a deactivated cell (the capacity booster cell) is compensated by a candidate cell. The degree of neighborliness measures and characterizes the degree to which a capacity booster cell overlaps with the candidate cell. The energy saving function determines the degree of neighborliness among the cells. Alternatively, the degree of neighborliness can be configured onto the energy saving function, e.g., by the operator or another function. As input, the entity computing the degree of neighborliness takes 1) the cell configuration data including transmit powers, applied antennas characteristics and tilts; 2) the propagation characteristics like building information; and 3) performance information like cell handover statistics.

NOTE: The degree of neighborliness is information contained by the energy saving function, it does not need to be defined in the RAN.

#### 5.A.1.3	Cell grouping and categorization

To optimize the selection of cells to be deactivated or reactivated, the energy saving function needs to group the cells that need to be treated togther for energy saving into a group, say called the Energy Saving Cell Group. Within the Energy Saving Cell Group, the cells are categorized into two sets - the primary candidate cells which are not deactivated and provide primary coverage and capacity booster cells which only enhance capacity and can thus be deactivated when the load is low. The entity computing the cell grouping can inform  the operator or an energy saving function about the group of cells that need to be treated together, i.e., the Energy Saving Cell Group and the categorization of cells either as a primary candidate or a capacity booster cell., Where the energy saving function does not have the capability to compute the Energy Saving Cell Group, it should be possible  to configure the Energy Saving Cell Group and cell categorisations to the energy saving function.

NOTE:  The Energy Saving Cell Group is quasi-static information on the energy saving function, when configured it remains unchanged until reconfigured.

#### 5.A.1.4	Cell deactivation sequence

When selecting cells to deactivate, the energy saving function considers the load in a set of cells to then decides the most optimal cell(s) to be deactivated and the order in which they can be deactivated. Based on the degree of neighborliness among cells as well as the cell grouping and categorization as primary candidate or capacity booster cells, the the energy saving function can compute the order in which the cells can be deactivated. The energy saving function can inform the e.g., the operator of the cell deactivation order, e.g., by indicating each capacity booster cell’s rank in the deactivation process. The an Operator can be enabled to request for such a deactivation order. Where the energy saving function does not have the capability to determine the cell deactivation order, another function(e.g. an AIML inference function) can be enabled to configure the cell deactivation order onto the energy saving function.

Given the cell deactivation order, the energy saving function routinely compares the load in the set of cells (the Cell Group load) to the low and high load thresholds to decide if deactivation can be executed. The Cell Group load needs to be computed for the set of active cells. Moreover, an automaton function can be enabled to configure the activation or deactivation load thresholds.

NOTE 1: The term deactivation is used to represent a total switching off of the cell or any disabling of a part of the cell, e.g., a disabling of the cell's power amplifier.

NOTE 2: The activation sequence is expected to be the opposite order to the deactivation sequence, but whether extra details are required for the activation sequence will be addressed in the normatve work.

NOTE 3: For all the input information, the energy saving function may be configured by or based on the output of other automation functions including MDA functions and AIML inferenc efunctions. For example, MDA may derive the cell grouping which is then configured onto the Energy saving function

Next Change

### 5.2.Y	Requirements for Proximity-based energy saving Optimization for multi-layer RAN

REQ-NES-01: The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with degree of neighborliness, describing the degree to which the coverage areas of any candidate neighbour cells overlap with each other.

REQ-NES-02:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the group of cells that need to be treated together (called the Energy Saving Cell Group) when selecting cell to be activated or deactivated.

NOTE 1: Configuration is needed only if the energy saving function cannot compute the Energy Saving Cell Groups.

REQ-NES-03:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the cell categories indicating candidate cells and capacity booster cells.

NOTE 2: Primary candidate cells are those that provide primary coverage and are not deactivated while capacity booster cells are those which only enhance capacity and can thus be deactivated when the load is low.

REQ-NES-04:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with information on the cell deactivation order which is the capacity booster cell’s rank in the deactivation and reactivation process indicating the order in which the cells in a given area should be deactivated.

NOTE 3: Configuration is needed only if the energy saving function does not determine the cell deactivation order.

REQ-NES-05:	The 3GPP management system should have a capability allowing an authorized MnS consumer to configure an energy saving function with the deactivation and reactivation load thresholds.

End Change