3GPP TSG-SA5 Meeting #159	S5-250379

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.277 | CR | 0056 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Message flows for ProSe UE-to-Network Direct Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 7.4.5 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ProSe Multi-Hop UE-to-Network relay communication technology, standardized by 3GPP SA2, involves multiple relays between the remote UE and the network. The Layer 3 UE-to-Network information flows of ProSe Multi-Hop are detailed in 3GPP SA2, TS 23.304, which may also influence charging information flows |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the message flows and descriptions for ProSe Multi-Hop Layer 3 UE-to-Network relay communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information flows for ProSe Multi-Hop Layer 3 UE-to-Network relay communication are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.7.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.2.7.8	Message flows for ProSe UE-to-Network Direct Communication - PEC

Figure 5.4.2.7.8-1: Message flow for ProSe Direct Communication via Layer-3 UE-to-Network Relay

1-7. These steps are the same as message flow for ProSe UE-to-Network Direct Communication via Layer-3 procedures described in TS 23.304 [241] clause 6.5.1.1.8. When the UE decides that reporting criteria are met, according to the pre-configuration, the UE creates the corresponding usage information report. UE triggers the usage reporting procedure.

9.  The Remote UE sends the usage reporting to ProSe UE-to-Network Relay UE. Then Relay UE sends the usage information report to the CTF located in ProSe NF (e.g., 5G-DDNFM).

9ch-a. Upon reception of Direct Communication usage information report, the CTF (ADF) triggers the Charging Data Request [Event]. The CTF (ADF) sends Charging Data Request [Event] to CHF.

9ch-b.  The 5G ProSe Direct communication via UE-to-Network Relay CDR is generated by CHF for the Remote UE.

9ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CTF (ADF).

10.	5G ProSe UE-to-Network Relay UE triggers the usage reporting procedure and creates the corresponding usage information report when the reporting criteria are met.

11.	5G ProSe UE-to-Network Relay UE sends the usage information report to the ProSe NF (CTF).

NOTE 1:	Step 10 and Step 11 can occur before Step 8 and Step 9.

11ch-a. Upon reception of Direct Communication usage information report, the CTF (ADF) triggers the Charging Data Request [Event]. The CTF (ADF) sends Charging Data Request [Event] to CHF.

11ch-b.  The 5G ProSe Direct communication via UE-to-Network Relay CDR is generated by CHF for the Relay UE.

11ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the CTF (ADF).

NOTE 2:	The procedure applies to Remote UE to UE-to-Network Relay UE independently, i.e. each of the UE sends the respective usage information reports according to different reporting criteria.

Figure 5.4.2.7.8-2: Message flow for ProSe Direct Communication via Layer-2 UE-to-Network Relay

1-8. These steps are the same as message flow for ProSe UE-to-Network Direct Communication via Layer-2 procedures described in TS 23.304 [241] clause 6.5.2.2.

9-11.  These steps are the same as message flow for Layer-3 UE-to-Network Relay in figure 6.2.4.5.3.1-1.

Figure 5.4.2.7.8-3: Message flow for ProSe Communication via Multi-hop Layer-3 UE-to-Network Relay

1-7: Similar message flows to step1-7 in Figure 5.4.2.7.8-1, with the differences and clarifications specified in clause 6.5.1.4 of TS 23.304 [241].

8-12. Similar message flows to step 8-11 in Figure 5.4.2.7.8-1, with the following differences and clarifications:

1.	Usage information from the remote UE to the network is sent sequentially through the UE-to-Network intermediate Relay UE and the UE-to-Network Relay UE.

2.	The UE-to-Network intermediate Relay may report usage information to the network through the UE-to-Network Relay if the specified criteria are met.

# * * * * End of changes * * * *