3GPP TSG-SA5 Meeting #159	S5-250414

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	Discussion on Correction of Recipient Address in SMS Charging

# Document for:	Discussion

Agenda Item:	7.3

1	Decision/action requested

Analysis and Discuss the correction on the recipient Address of SMS Charging.

2	References

[1]	3GPP TS 32.274: "Charging management; Short Message Service (SMS) charging".

[2]	3GPP TS 32.291: "Charging management; 5G system, charging service; Stage 3".

[3]	3GPP TS 32.298: "Charging management; Charging Data Record (CDR) parameter description".

3	Rationale

3.1	The Problem

The TS 32.274 [1] specifies the SMS Charging mechanism for Legacy and 5G system in stage 2. The TS 32.291 [2] specifies the corresponding attributes and the Open API for 5G SMS Charging in the stage 3.

The "RecipientAddress" is present in the clause 6.1.6.2.3.6 Type RecipientAddress and the clause A.2	 Nchf_ConvergedCharging of TS 32.291 [2]. However, the "RecipientAddress" Type is not referenced or used in "RecipientInfo" of the "SMSChargingInformation" or other attributes.

Problem:  "RecipientAddress" Type in the TS 32.291 is independent and not used.

3.2	Background

3.2.1	EPC SMS Charging

According to the Clause 6.4	 Bindings for SMS charging in the TS 32.274 [1], the mapping between the Service Information fields, AVPs and CDR parameters for SMS charging are present. The "Recipient-Info" AVP is of type Grouped, including three "Recipient-Address" AVPs, and mapped to the Recipient IMSI/MSISDN/Other Address in the CDRs.

The "Recipient-Address" AVP is of type grouped, including e.g. "Address-Type" and "Addressee-Type".

The "Recipient-Received-Address" AVP is of type grouped, including e.g. "Address-Type".

3.2.2	5G SMS Charging

The SMS Charging information is reported to CHF which includes "recipientOtherAddress" and "recipientReceivedAddress". Both "recipientOtherAddress" and "recipientReceivedAddress" uses the same Data Type "SMAddressInfo".

The "SMAddressInfo" includes "sMaddressType", "sMaddressData" and "sMaddressDomain".

3.2.3	Potential Solutions

Proposal 1: Delete the data type "RecipientAddress" definition in the TS 32.291 [2].

Proposal 2: Change the Data Type of "recipientOtherAddress" from "SMAddressInfo" to "RecipientAddress" in the TS 32.291 [2] and TS 32.298 [3].

Proposal 3: Add the "recipientOtherAddressExtension" new attribute with the data type "RecipientAddress" in the TS 32.274 [1]，TS 32.291 [2] and TS 32.298 [3].

3.2.4	Comparison

| Potential Solutions | Pros | Cons |
| --- | --- | --- |
| Proposal 1: Delete "RecipientAddress" | The problem for the unused data type "RecipientAddress" is resolved. | The "AddresseeType " (i.e. enumeration value: TO/CC/BCC) is not applicable in the 5G SMS Charging. |
| Proposal 2: Change the "recipientOtherAddress" | The problem for the unused data type "RecipientAddress" is resolved. | The back compatible issues should be considered in TS 32.291 Open API. |
| Proposal 3: Add the new attributes "recipientOtherAddressExtension" | The problem for the unused data type "RecipientAddress" is resolved. No back compatible issues. | One More new attribute and impact on Stage 2 and Stage 3 charging specifications. |

3.3			Proposed way forward

Considering the potential solutions comparison and minimization of charging impaction, the proposal 2 about Change the "recipientOtherAddress" is preferred.

4	Detailed Proposal

The detailed proposals on the CRs for the TS 32.291 and TS 32.298 in Rel18/Rel19 are present for SA5 #159 meeting (S5-250415/S5-250416/S5-250417/S5-250418).