3GPP TSG-SA5 Meeting #159	S5-250460

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0240 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.105 correct the components for ML model deployment and AI/ML inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some incorrections of service components for ML model deployment and AI/ML inference. In clause 7.3a.1.2.3.1, it states “The MLTrainingReport instance is created by the training MnS producer automatically when creating an MLTrainingRequest instance”, “an instance of the MLTrainingProcess is created by the MnS Producer” that means this IOC can not create by MnS consumer, therefore, changeMOIs operation is not suitable for MLTrainingReport and MLTrainingProcess. Similarly, MLTestingReport, AIMLInferenceReport, MLUpdateProcess have similar problems. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the Management service component type B for ML model training, ML model testing, ML model deployment and AI/ML inference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect descriptions will make confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

8	Service components

8.0	General

The operations for generic provisioning management service refer to clause 11.1.1 of TS 28.532 [11]. For notifications, see clause 7.6.

8.1	Lifecycle management operations for AI/ML management MnS

The components for ML model training MnS are listed in table 8.1-1.

Table 8.1-1: Components for ML model training

| ML model training management capability | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Create an ML model training request | createMOI operation | MLTrainingRequest | N/A |
| Modify an ML model training request | modifyMOIAttributes operation |  |  |
| Query an ML model training report | getMOIAttributes operation | MLTrainingReport |  |
| Modify an ML model training process | modifyMOIAttributes operation | MLTrainingProcess |  |
| Query an ML model training process | getMOIAttributes operation |  |  |
| Create, Delete, and Update ML training | changeMOIs operation | MLTrainingRequest |  |

The components for ML model testing are listed in table 8.1-2.

Table 8.1-2: Components for ML model testing

| ML model testing management capability | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Create an ML model testing request | createMOI operation | MLTestingRequest | N/A |
| Modify an ML model testing request | modifyMOIAttributes operation |  |  |
| Query an ML model testing report | getMOIAttributes operation | MLTestingReport |  |
| Subscribe an ML model testing report | createMOI operation |  |  |
| Unsubscribe an ML model testing report | deleteMOI operation |  |  |
| Query an ML model testing report subscription | getMOIAttributes operation |  |  |
| Create, Delete, and Update ML testing | changeMOIs operation | MLTestingRequest |  |

The components for ML model deployment are listed in table 8.1-3

Table 8.1-3: Components for ML model deployment

| ML model deployment management capability | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Create an ML model loading request | createMOI operation | MLModelLoadingRequest | N/A |
| Modify an ML model loading request | modifyMOIAttributes operation |  |  |
| Create an ML model loading policy | createMOI operation | MLModelLoadingPolicy |  |
| Delete an ML model loading policy | deleteMOI operation |  |  |
| Modify an ML model loading policy | modifyMOIAttributes operation |  |  |
| Query an ML model loading policy | getMOIAttributes operation |  |  |
| Modify an ML model loading process | modifyMOIAttributes operation | MLModelLoadingProcess |  |
| Query an ML model loading process | getMOIAttributes operation |  |  |
| Create, Delete, and Update ML model Loading | changeMOIs operation | MLModelLoadingRequest MLModelLoadingPolicy |  |

The components for ML model inference are listed in table 8.1-4

Table 8.1-4: Components for AI/ML inference

| AI/ML Inference management capability | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Create an ML model update request | createMOI operation | MLUpdateRequest | N/A |
| Modify an ML model update request | modifyMOIAttributes operation |  |  |
| Modify an ML model update process | modifyMOIAttributes operation | MLUpdateProcess |  |
| Query an ML model update process | getMOIAttributes operation |  |  |
| Query an ML model update report | getMOIAttributes operation | MLUpdateReport |  |
| Query an AI/ML inference report | getMOIAttributes operation | AIMLInferenceReport |  |
| Create, Delete, and Update ML model Update and ML Inference Report | changeMOIs operation | MLUpdateRequest |  |

The components for AI/ML inference emulation are listed in table 8.1-5.

Table 8.1-5: Components for AI/ML inference emulation MnS

| ML model emulation management capability | Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- | --- |
| Query an AI/ML inference emulation report | getMOIAttributes operation | AIMLInferenceReport | N/A |

End of modified sections