3GPP TSG-SA5 Meeting #159	S5-250485

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	Rel-19 pCR TS 28.561 Relations with existing network automation functions

# Document for:	Approval

Agenda Item:	6.19.5.1

1	Decision/action requested

For approval

2	References

[1] 	3GPP TS 28.561: “Management aspects of Network Digital Twins”.

3	Rationale

This contribution proposes to explain the relationship between NDT and existing network automation functions.

4	Detailed proposal

It is proposed to make the following changes to TS 28.561 [1].

1st Change

X	Relations between digital twins and network automation functions

The NDT is used to model a mobile network or part of one, supports evaluating the corresponding impact, and returns the results of simulation generated by the NDT. The NDT provides modelling capabilities that may be used by network automation functions (e.g. MDA, etc.) to accomplish their automation functionality. The related automation capabilities are provided by the network automation functions regardless of whether the NDT are integrated within or external to the network automation functions - see figure X-1 and figure X-2.

Figure X-1: NDT are integrated within the network automation functions

Deployment scenario 1 represents that NDT internal/integrated into existing network automation functions. NDT is used to support network automation function services.

Figure X-2: NDT are external to the network automation functions

Deployment scenario 2 represents that NDT separates from existing network automation functions. NDT service can be consumed by network automation functions.

NOTE:	The double headed arrows indicate candidate flow of data and controls from the network to the NDT and to the management function and related flow of control from the MnFs to the network of NDT while one headed arrow indicates only flow of control.

End of changes