3GPP TSG-SA5 Meeting #159	S5-250489

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	DP on the relation of general NDT framework and scenario specific attributes

# Document for:	Endorsement

Agenda Item:	6.19.5.1

1	Decision/action requested

The group is asked to discuss and endorsement.

2	References

[1]	3GPP SP-241938: "New WID on management aspects of Network Digital Twin "

3	Rationale

3.1	Problem statement

Objects in NDT WID SP-241938

SA#106 approved NDT WID SP-241938 [1] which splits the solution for NDT into two parts: general NDT model framework and scenario specific modelling information per use case.

"

- WT3: Specify the solutions for NDT to support above use cases, which includes the procedure, management operations and management information (e.g., NDT modelling).

  - WT3.1 Generic operations and generic information model for NDT.

  - WT3.2 NDT application specific solution extending the generic information model with NDT application specific modeling information for the above use cases.

"

How specific solution can be defined based on the generic solution framework is not discussed in study phase and deserves to be figured out before further discussion in NDT NRMs. This discussion paper proposes a way forward which introduces the relation of general NDT modelling framework and scenario specific modelling and the mechanism of how these two are combined and work.

3.2	Way forward

3.2.1	General NDT modelling framework

Figure 1: Relationship UML diagram for NDT management

Figure 1 shows a relationship diagram for NDT management. NDTFunction IOC represents the function that can handle NDT services and is name-contained by the proxy class ManagedEntity which represents SubNetwork. IOC NDTService IOC represents the simulation task request from MnS consumer and NDTReport IOC represents the simulation results provided by NDT. Both NDTService IOC and NDTReport IOC are name-contained by NDTFunction IOC.

- Attributes for NDTService IOC are captured in Table 1. Attributes simulationScope and behaviour are highly associated with specific use cases and therefore are extended in clause 3.2.2.

NOTE: All details of attributes can be further discussed and defined. This contribution focuses only on explain how general solution and scenario solution are combined and work together.

Table 1: Attributes for NDTService IOC

| Attribute Name | Description |
| --- | --- |
| ndtServiceID | Unique ID for an NDT service request |
| simulationScope | The scope of mobile network that needs to be simulated, which is further specified in different scenarios |
| behaviour | Network behaviour that defined by MnS consumer and will be configured to NDT. It is further specified in different scenarios |
| simulationTime | A time window which indicates if the simulation is for the past, present, or future |
| performanceRequirements | Performance requirements for NDT modelling, e.g., the synchronization frequency, maximum run time for each simulation task, precision |

- Attributes for NDTReport IOC are captured in Table 2. Attributes performanceData is highly associated with specific use cases and therefore is extended in clause 3.2.2

Table 2: Attributes for NDTReport IOC

| Attribute Name | Description |
| --- | --- |
| ndtReportID | Unique ID for an NDT report |
| performanceData | Specified performance metrics and/or alarm types that needs to be collected and reported by NDT after the behaviour configured in NDT. It is further specified in different scenarios |

3.2.2	Scenario specific modelling information

Takin RAN ES policy verification as an example, scenario specific modelling information is defined as follows

3.2.2.1	RAN NDT

This clause introduces the scenario specific modelling information for RAN NDT services.

3.2.2.1.1	simulationScope

| Attribute Name | Allowed Values |
| --- | --- |
| simulationType (CM) | RAN SubNetwork |
| simulationInstance (CM) | DN of RAN SubNetwork |
| simulationScopeFilter (O) | Filters on geographic area, gNB list, frequency range |

3.2.2.1.2	behaviour

| Attribute Name | Allowed Values |
| --- | --- |
| ranEsPolicyVerification | - object: DN of DESManagementFunction defined in TS 28.541 - actionList: - key: attribute name of DESManagementFunction - value: value of the attribute above |

3.2.2.1.3	performanceData

| Performance measurements | Description |
| --- | --- |
| NG-RAN data Energy Efficiency | This measurement provides mobile network data energy efficiency in operational NG-RAN (see clause 6.7.1.1 in TS 28.554). |
| NG-RAN EC | This measurement describes the Energy Consumption (EC) of the NG-RAN (see clause 6.7.3.4.1 in TS 28.554). |
| gNB EC | This measurement describes the Energy Consumption (EC) of the gNB (see clause 6.7.3.4.2 in TS 28.554). |
| UE throughput | This set of measurements describes the UE throughput in gNB (see clause 5.1.1.3 in TS 28.552) |

3.2.3	Guideline for using the NDT modelling across different use cases

This clause summaries an example of mapping relations, as guideline for using the NDT modelling, between general framework and specific modelling information across three typical NDT use cases: RAN ES policy verification, Signalling Storm Analysis and ML training data generation.

Table 3: Guideline for using the NDT modelling to support different use cases

| Use case | NDTtype | simulatonScope | behaviour | performanceData |
| --- | --- | --- | --- | --- |
| RAN ES policy verification | RAN NDT | RAN SubNetwork with filters on geographic area, gNB list, frequency range | - ranEsPolicyVerification | - NG-RAN data Energy Efficiency - NG-RAN EC - gNB EC - UE throughput |
| Signalling Storm Analysis | CN NDT | CN SubNetwork with filters on CN NF list | - signallingStormAnalysis | - packet delay - number of PDU sessions - number of registered subscribers - number of successful registrations |
| ML training data generation | RAN NDT AND/OR CN NDT | RAN SubNetwork with filters on geographic area, gNB list, frequency range AND/OR CN SubNetwork with filters on CN NF list | - dataGeneration | / |

4	Detailed proposal

It proposes to endorse following proposals:

Proposal#1. General NDT NRMs should be defined to capture the general modelling framework for NDT

Proposal#2. Scenario specific modelling information should be defined case by case following the general modelling framework.

Proposal#3. A guideline table of using NDT modelling can be maintained to associate the scenario specific modelling information with general modelling framework per use case.